# 3GPP TSG-RAN WG3 #105bis	R3-196054

Chongqing, China, 14th – 18th October

Agenda Item:	15.3.1

Source:	Ericsson

Title:	Conditional Handover - HO preparation.

Document for:	Discussion and Decision

# Introduction

During Mobility Enhancements WIs discussion in RAN2, it was agreed that conditional handover is introduced in LTE and NR to solve robustness/reliability issue. The discussion has been ongoing in RAN3 in order to provide the needed support for the conditional handover. In this contribution we discuss further aspects of Conditional Handover (CHO) preparation, we provide our views and present our proposals.

# Discussion

In the previous meeting the issue of CHO preparation has been discussed and the following was captured as a summary:

1) Enabling sending multiple CHO requests in a single message

Decision: At this moment, the participating companies prefer to keep the solution with a single target cell per request. If clear motivation is identified in future, RAN3 may reconsider the need. However, it is agreed that parallel transactions for HO Preparation shall be allowed to handle the scenarios allowed in RAN2 and each request initiated for the same source’s UE ID is identified with the target cell ID.

Following the above decision, the following sentence was added in the BL CRs for 36.423 and 38.423:

If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified with the target cell ID.

Some clarifications should be made regarding the above statement. Indeed, the parallel requests can be identified with the target cell ID in the case that the source UE AP IDs are the same.

Proposal 1: We propose to clarify in the BL CRs for 36.423 and 38.423 that the parallel requests can be identified with the target cell ID in the case that the source UE AP IDs are the same.

Otherwise, if the source UE AP IDs are different, other ways to identify that the requests are for the same UE, should be explored. A solution would be to introduce a Transaction ID, to assist in the identification of the parallel procedures that concern the same HO, the same UE, the same target node albeit different target cells. Another idea would be the UE context, whether this can provide the necessary identification.

The introduction of new IEs to assist in the identification of the requests being for the same UE suggests an overcomplication that is unnecessary. We propose to clarify that if the identification of parallel requests are needed then the source UE AP IDs shall be the same.

Proposal 2: We propose to clarify that for the case the identification of parallel procedures for a CHO are needed, the source UE AP IDs shall be the same.

# Conclusion

### In this contribution we looked in more detail in the Conditional Handover preparation and the following proposals were made:

Proposal 1: We propose to clarify in the BL CRs for 36.423 and 38.423 that the parallel requests can be identified with the target cell ID in the case that the source UE AP IDs are the same.

Proposal 2: We propose to clarify that for the case the identification of parallel procedures for a CHO are needed, the source UE AP IDs shall be the same.

Proposal 3: RAN3 is kindly requested to agree with the changes proposed in the accompanying TPs