| 3GPP TR 38.856 V0.1.0 (2019-8) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on local NR positioning in NG-RAN (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	 Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	General	6

5 	 Local LMF in NG-RAN	7

5.1	Architecture	7

5.1.X 	 Alternative 1	7

5.1.Y 	 Alternative 2	7

5.1.Z 	 Alternative 3	8

5.1.Y 	 Function supported by LMC [FFS]	8

5.2	Impact on existing protocols and interfaces	9

5.2.1	Location Service procedures involving LMC	9

5.2.1.1	RAN Induced Location Request (RI-LR)	9

5.2.1.2	5GC Mobile Terminated Location Request (5GC-MT-LR)	10

5.3	Coordination and coexistence with LMF in the 5GC	11

6	LCS client in NG-RAN	11

6.1	Architecture	11

6.2	 Impact on existing protocols and interfaces	12

7	Conclusion	12

Annex <X> (informative): Change history	13

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

A new SI “Study on local NR positioning in NG-RAN” was approved for Release 16 at the 3GPP TSG RAN #83 meeting in [2]. The study aims to the feasibility and specification impact for supporting local LMF and/or LCS client in NG-RAN.

1		Scope

The present document captures studies and findings of specification impact for supporting local LMF and/or LCS client in NG-RAN.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP RP-191460:  "Revised SID: Study on local NR positioning in NG-RAN"

3	Definitions of terms, symbols and abbreviations

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	General

5 		Local LMF in NG-RAN

Editor note: Specification impact of Local LMF including location of the LMF, potential new interface(s) (if any), impact on existing protocols, and coordination with the LMF in the 5GC

Editor note: Evaluation section (in case multiple solutions are captured) could be added

5.1	Architecture

5.1.X 		Alternative 1

Editor’s note: An outline of the characteristics of the architecture should be provided, explaining rationale and differences vs other architectures

- Alternative 1: LMC is a functionality of NG-RAN node. In case of split gNB, the LMC is a function of the gNB-CU-CP

Figure 5.1-x  Alternative 1: UE Positioning Architecture applicable to NG-RAN, including LMC

5.1.Y 		Alternative 2

Editor’s note: An outline of the characteristics of the architecture should be provided, explaining rationale and differences vs other architectures

- Alternative 2: LMC is a logical node of gNB in split gNB architecture, the LMC is connected to gNB-CU

Figure 5.1-y Alternative 2: UE Positioning Architecture applicable to NG-RAN, including LMC

5.1.Z 		Alternative 3

Editor’s note: An outline of the characteristics of the architecture should be provided, explaining rationale and differences vs other architectures

- Alternative 3: LMC is a logical node of NG-RAN architecture. A LMC connects with a set of NG-RAN nodes through a new interface or extension of an existing interface.

Figure 5.1-z Alternative 3: UE Positioning Architecture applicable to NG-RAN, including LMC

5.1.Y 		Function supported by LMC [FFS]

Editor note: At this stage it is not clear if the LMC is able to support all the function of the LMF. The positioning method supported by the LMC is FFS.

The LMC need to support all the location request types defined for the LMF in clause 4.1a of TS 23.273 [4] and supported in Rel-15, i.e.:

-	Network Induced Location Request (NI-LR);

-	Mobile Terminated Location Request (MT-LR);

-	Mobile Originated Location Request (MO-LR);

Note:	According to TS 38.305, MO-LR location service is not supported in Rel-15. It is therefore out-of-scope of this study item.

-	Immediate Location Request; and

-	Deferred Location Request.

The LMC should be able to support location service requests from functions internal to the NG-RAN node, this type of location request can be called RAN Induced Location Request (RI-LR). The LMC is ideally suited to handle RI-LR since the necessary operations are internal to the RAN and can therefore be handled locally.

In order to manage the various location services for target UEs, the LMC can be expected to have capabilities similar to an LMF, e.g.:

-	receive location service requests for a target UE from the serving AMF

Editor note: it is FFS whether service request can come from an internal function of the serving NG-RAN node (e.g. RRM, MDT, etc.);

-	decide on the position method(s) to be used;

-	interact with other internal functions of the serving NG-RAN node (e.g. TMF) in order to e.g. obtain position measurements for the UE;

-	interact with a target UE in order to e.g. deliver assistance data or to obtain a location estimate;

-	interact with neighboring NG-RAN nodes in order to e.g. obtain position measurements for the UE or coordinate dynamic PRS configuration, handling positioning request for UE in neighboring node;

-	calculate or verify the final location of the UE and provide it to the requesting entity or requesting internal function of the serving NG-RAN node.

5.2	Impact on existing protocols and interfaces [FFS]

5.2.1	Location Service procedures involving LMC

Editor Note: flows are intended to show common operation for all architectures with LMC. Specific architectures may require further details

5.2.1.1	RAN Induced Location Request (RI-LR)

Figure 5.2.1.1-1 illustrates the steps applicable to the UE and NG-RAN node in case of RI-LR involving LMC.

In this scenario, the UE is assumed to be in connected mode prior to step 1. It is also assumed that the NG-RAN node is always authorized to use the LCS service to obtain the target UE location estimation, thus no privacy verification and LCS service authorization are required for requests initiated by the NG-RAN (see subclause 6.11.1 of TR 23.731 [1]).

Figure 5.2.1.1-1: RI-LR involving LMC

The steps of Figure 5.2.1.1-1 are as follows:

1.	Some function in the NG-RAN node (e.g. RRM, MDT) requests some location service (e.g. positioning) for a target UE to the LMC using signaling internal to the NG-RAN node.

EN:	Enhancements to F1AP, if needed (e.g. in case the requesting function is in the DU), are FFS.

2.	The NG-RAN node (LMC) sends an LCS Indication message to the AMF, indicating that UE positioning is being performed locally (LCS session start). The LCS Indication message includes information about the “local” LCS Request that may be useful for the AMF (location management information), e.g. the same or subset of information contained in a “normal” location service request (e.g. MT-LR) such as Location Quality of Service information.

3.	The AMF stores the relevant location management information in the UE location context, to be used for coordination of concurrent LCS requests, e.g. if the AMF receives an MT-LR while there is an active location session at the LMC for the same UE.

EN:	Coordination with the 5GC, if needed, is FFS.

4a.	The LMC instigates location procedures internal to the NG-RAN node – e.g. to obtain positioning measurements or assistance data. The LMC may also instigate location procedures with neighbor NG-RAN nodes – e.g. to obtain assistance data such as PRS configuration of TPs served by the neighbor NG-RAN node(s).

EN:	Enhancements to XnAP, if needed, are FFS.

4b.	In addition to step 4a or instead of step 4a, for downlink positioning the LMC instigates location procedures with the UE – e.g. to obtain a location estimate or positioning measurements or to transfer location assistance data to the UE.

5.	The LMC responds to the requesting function in the NG-RAN node and includes any needed results – e.g. success or failure indication and, if requested and obtained, a location estimate for the UE.

6.	If the response in step 5 ends the local LCS session, the NG-RAN node (LMC) sends an LCS Indication message to the AMF indicating that positioning is no longer being performed locally for the target UE (LCS session stop).

5.2.1.2	5GC Mobile Terminated Location Request (5GC-MT-LR)

Figure 5.2.1.2-1 illustrates the steps applicable to the UE and NG-RAN node in case of 5GC-MT-LR involving LMC. In this scenario, the UE is assumed to be in connected mode prior to step 1.

Figure 5.2.1.2-1: 5GC-MT-LR involving LMC

The steps of Figure X.2.1.2-1 are as follows:

1.	Some LCS entity in the 5GC (e.g. GMLC) requests some location service (e.g. positioning) for a target UE to the serving AMF (see step 4 of Figure 6.1.1-1 in TS.23.273 [4]).

2.	The AMF selects the serving NG-RAN node (LMC) to perform the positioning, based on e.g. knowledge of the NG-RAN node’s location management capabilities, Requested Quality of Service information, etc.

Note 1:	The details of whether to select LMC are SA2 scope, including whether it is part of the LMF Selection function or a new function of the AMF.

3.	The AMF sends a location service request to the NG-RAN node (LMC) via NGAP, instructing it to perform the UE positioning.

4.	Same as steps 4a/4b of the RI-LR procedure in clause X.Y2.1.

5.	The NG-RAN node (LMC) provides a location service response to the AMF via NGAP and includes any needed results – e.g. success or failure indication and, if requested and obtained, a location estimate for the UE.

6.	The AMF returns a location service response to the 5GC entity of step 1 and includes any needed results – e.g. a location estimate for the UE (see step 10 of Figure 6.1.1-1 in TS.23.273 [4]).

5.3	Coordination and coexistence with LMF in the 5GC

Editor Note: Function split between LMC in NG-RAN and LMF in 5GC is FFS

Editor Note: Whether functions supported by LMC can be different per architecture alternative is FFS.

Editor Note: How/who to trigger location request to LMC and whether the location request can come from internal function of NG-RAN node is FFS

6	LCS client in NG-RAN

Editor note: Specification impact of NG RAN acting as LCS client

6.1	Architecture

6.2		Impact on existing protocols and interfaces

7	Conclusion

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-8-26 | RAN3 #105 | R3-184476 |  |  |  | First Skeleton provided | 0.0.0 |
| 2019-8-31 | RAN3 #105 |  |  |  |  | Include the agreed TP in R3-194760 | 0.1.0 |