3GPP TSG-RAN WG3 Meeting #105bis	R3-196078

Chongqing, China, 14-18 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0178 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on header compression |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was agreed that if the PDU session Type is "ethernet" or "unstructured", then the header compression for the concerned PDU session shall not be performed. This captured in TS38.413 as the following: “For each PDU session, if the PDU Session Type IE included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message is set to "ethernet" or "unstructured", the NG-RAN node shall not perform header compression for the concerned PDU session.” However, this is not captured in the TS 38.463, such that the CU-UP cannot detect the abnormal case, e.g., the PDU Session Type IE included in the BEARER CONTEXT SETUP REQUEST message is set to "ethernet" or "unstructured", but the ROHC Parameters IE is also included in the PDCP Configuration IE of the BEARER CONTEXT SETUP REQUEST message, and then the CU-UP may perform header compression which is not a correct behavior, i.e.if PDU session type is set to "ethernet" or "unstructured", gNB-CU-CP shall not include ROHC Parameters IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the following text in section 8.3.1 For each PDU session, if the PDU Session Type IE included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message is set to "ethernet" or "unstructured", the gNB-CU-CP shall not include the ROHC Parameters IE and the gNB-CU-UP shall not perform header compression, for the concerned PDU session. - Add the following text in section 8.3.2 For each PDU session, if the PDU Session Type IE included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message is set to "ethernet" or "unstructured", the gNB-CU-CP shall not include the ROHC Parameters IE and the gNB-CU-UP shall not perform header compression, for the concerned PDU session. Impact analysis: This CR has no impact with the previous version of the specification, because the impact can be considered isolated, and there is no backward compatible issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CU-UP cannot detect the abnormal case and may result in error behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

8.3.1	Bearer Context Setup

8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

<Unchanged Text Omitted>

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

For each PDU session, if the PDU Session Type IE included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message is set to "ethernet" or "unstructured", the gNB-CU-CP shall not include the ROHC Parameters IE, and the gNB-CU-UP shall ignore the ROHC Parameters IE if included.

<Unchanged Text Omitted>

8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

<Unchanged Text Omitted>

If the S-NSSAI IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store the corresponding information and replace any existing information.

For each PDU session, if the PDU Session Type IE included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message is set to "ethernet" or "unstructured", the gNB-CU-CP shall not include the ROHC Parameters IE, and the gNB-CU-UP shall ignore the ROHC Parameters IE if included.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>