3GPP TSG-RAN WG3 Meeting #105bis	R3-196088

Chongqing, China, 14-18 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.470 | CR |  | rev |  | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR to 37.470 on miscelleneous corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, TIM, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_arch_evo-Core |  |  |  |  |  | Date: |  |  | 2019-10-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This is the rapporteur updates for non-controversial issues found in the latest version of the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added description of: - References - Definitions and abbreviations - General aspects - QoS notification - inactivifty notification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ng-eNB CU/DU architecture does not work well. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 4.1, 4.2, 4.3, 5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x2]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[x3]	3GPP TS 38.425: "NG-RAN; Xn interface user plane protocol".

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

W1: interface between an ng-eNB-CU and an ng-eNB-DU, providing an interconnection point between the ng-eNB-CU and the eNB-DU.

W1-C: Reference point for the control plane protocol between ng-eNB-CU and ng-eNB-DU.

ng-eNB-CU: a logical node hosting RRC, SDAP and PDCP protocols of the ng-eNB or RRC and PDCP protocols of the ng-eNB that controls the operation of one or more ng-eNB-DUs. The ng-eNB-CU terminates the W1 interface connected with the ng-eNB-DU.

ng-eNB-DU: a logical node hosting RLC, MAC and PHY layers of the ng-eNB, and its operation is partly controlled by ng-eNB-CU. One ng-eNB-DU supports one or multiple cells. One cell is supported by only one ng-eNB-DU. The ng-eNB-DU terminates the W1 interface connected with the ng-eNB-CU.

ng-eNB: as defined in TS 38.300 [x2].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DRB	Data Radio Bearers

W1-U	W1 User plane interface

W1-C	W1 Control plane interface

W1AP	W1 Application Protocol

GTP-U	GPRS Tunnelling Protocol

IP	Internet Protocol

NR-MIB	NR-Master Information Block

O&M	Operation and Maintenance

PA	Paging Area

PF	Paging Frame

PO	Paging Occasion

QoS	Quality of Service

RRC	Radio Resource Control

SCTP	Stream Control Transmission Protocol

SRB	Signalling Radio Bearers

SIB1	System Information Block 1

TNL				Transport Network Layer

4	General aspects

This clause captures the W1 interface principles and characteristics.

4.1	W1 interface general principles

The general principles for the specification of the W1 interface are as follows:

-	An ng-eNB may consist of an ng-eNB-CU and ng-eNB-DUs. An ng-eNB-CU and an ng-eNB-DU is connected via W1 logical interface.

-	One ng-eNB-CU controls one or more ng-eNB-DUs.

-	One ng-eNB-DU supports one or multiple cells. One cell is supported by only one ng-eNB-DU.

-	eNB-DU ID isused to identify ng-eNB-DU only over W1AP procedures, ng-eNB-DU ID is not connected to cell identifier.

-	The ng-eNB-CU terminates W1 interface connected with the ng-eNB-DU.

-	The ng-eNB-DU terminates W1 interface connected with the ng-eNB-CU.

-	The W1 interface shall separate Radio Network Layer and Transport Network Layer.

-	The W1 interface shall enable exchange of UE associated information and non-UE associated information.

-	The W1 interface is open;

-	From a logical standpoint, the W1 is a point-to-point interface between a ng-eNB-CU and a ng-eNB-DU.

NOTE 1: A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

-	The W1 interface supports control plane and user plane separation;

-	The W1 interface enables exchange of UE associated information and non-UE associated information;-	The standard should not prevent to separated CP and UP.

4.2	W1 interface specification objectives

The W1 interface specifications facilitate the following:

-	inter-connection of a ng-eNB-CU and a ng-eNB-DU supplied by different manufacturers.

4.3	W1 interface capabilities

The W1 interface supports:

-	procedures to establish, maintain and release radio bearers for the NG-RAN part of PDU sessions and for E-UTRAN Radio Access Bearers;

-	the separation of each UE on the protocol level for user specific signalling management;

-	the transfer of RRC signalling messages between the UE and the ng-eNB-CU.

4.4	W1 interface characteristics

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

### 5.2.4	UE context management function

The W1 UE context management function supports the establishment and modification of the necessary overall UE context.

The establishment of the W1 UE context is initiated by the ng-eNB-CU and accepted or rejected by the ng-eNB-DU based on admission control criteria (e.g., resource not available).

The modification of the W1 UE context can be initiated by either ng-eNB-CU or ng-eNB-DU. The receiving node can accept or reject the modification. The W1 UE context management function also supports the release of the context previously established in the ng-eNB-DU. The release of the context is triggered by the ng-eNB-CU either directly or following a request received from the ng-eNB-DU. The ng-eNB-CU requests the ng-eNB-DU to remove the UE Context when the UE enters RRC_IDLE or RRC_INACTIVE.

This function can be also used to manage DRBs and SRBs, i.e., establishing, modifying and releasing DRB and SRB resources. The establishment and modification of DRB resources are triggered by the ng-eNB-CU and accepted/rejected by the ng-eNB-DU based on resource reservation information and QoS information to be provided to the ng-eNB-DU. For each DRB to be setup or modified, the S-NSSAI may be provided by ng-eNB-CU to the ng-eNB-DU in the UE Context Setup procedure and the UE Context Modification procedure.

For NG-RAN, the mapping between QoS flows and radio bearers is performed by ng-eNB-CU and the granularity of bearer related management over W1 is radio bearer level, and the ng-eNB-CU decides an aggregated DRB QoS profile for each radio bearer based on received QoS flow profile, and provides both aggregated DRB QoS profile and QoS flow profile to the ng-eNB-DU, and the ng-eNB-DU either accepts the request or rejects it with appropriate cause value. With this function, ng-eNB-DU could also notify ng-eNB-CU whether the QoS for already established DRBs is not fulfilled any longer or it is fulfilled again.

The UE Inactivity Notification function is initiated by the ng-eNB-DU to indicate the UE activity event.

The Notify function is to enable the ng-eNB-DU to inform the ng-eNB-CU that the QoS of an already established GBR DRB cannot by fulfilled any longer or that it can be fulfilled again.

With this function, the ng-eNB-CU indicates the UL UE AMBR limit to the ng-eNB-DU, and the ng-eNB-DU enforces the indicated limit.

With this function, ng-eNB-CU requests the ng-eNB-DU to setup or change of the SpCell for the UE, and the ng-eNB-DU either accepts or rejects the request with appropriate cause value.

With this function, the ng-eNB-CU requests the setup of the SCell(s) at the ng-eNB-DU side, and the ng-eNB-DU accepts all, some or none of the SCell(s) and replies to the ng-eNB-CU. The ng-eNB-CU requests the removal of the SCell(s) for the UE.

With this function, the ng-eNB-DU indicates that a bearer, or a UE is inactive or active. The ng-eNB-CU consolidates all the serving ng-eNB-DUs for the UE and takes further action.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>