3GPP TSG-RAN WG3 Meeting #105bis	R3-196132

Chongqing, China, October 14th – 18th, 2019

Agenda item:		15.4.1

Source:	Intel Corporation, ZTE, ETRI

Title:	(TP for NR_Mob_enh-Core BL CR for TS 38.300): Data Forwarding and SN Status Transfer for 0ms Interruption

Document for:		Other

# 1		Introduction

This contribution proposes TP for TS 38.300 based on enhancements proposed for RUDI (Reduction in User Data Interruption) data forwarding and SN status continuation in [1] and [2].

# 2		Reference

[1] R3-195664, “Data Forwarding and SN Status Transfer for 0ms Interruption”, Intel Corporation

[2] R3-196129, “Summary of offline – Data Forwarding and SN Status Transfer for 0ms Interruption”, Intel Corporation

# 3		TP for BL CR for TS 38.300

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

##### 9.2.3.2.1	C-Plane Handling

The intra-NR RAN handover performs the preparation and execution phase of the handover procedure performed without involvement of the 5GC, i.e. preparation messages are directly exchanged between the gNBs. The release of the resources at the source gNB during the handover completion phase is triggered by the target gNB. The figure below depicts the basic handover scenario where neither the AMF nor the UPF changes:

Figure 9.2.3.2.1-1: Intra-AMF/UPF Handover

0.	The UE context within the source gNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1.	The source gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

2.	The source gNB decides to handover the UE, based on MeasurementReport and RRM information.

3.	The source gNB issues a Handover Request message to the target gNB passing a transparent RRC container with necessary information to prepare the handover at the target side. The information includes at least the target cell ID, KgNB*, the C-RNTI of the UE in the source gNB, RRM-configuration including UE inactive time, basic AS-configuration including antenna Info and DL Carrier Frequency, the current QoS flow to DRB mapping rules applied to the UE, the SIB1 from source gNB, the UE capabilities for different RATs, PDU session related information, and can include the UE reported measurement information including beam-related information if available. The PDU session related information includes the slice information (if supported) and QoS flow level QoS profile(s).

NOTE:	After issuing a Handover Request, the source gNB should not reconfigure the UE, including performing Reflective QoS flow to DRB mapping.

4.	Admission Control may be performed by the target gNB. Slice-aware admission control shall be performed if the slice information is sent to the target gNB. If the PDU sessions are associated with non-supported slices the target gNB shall reject such PDU Sessions.

5.	The target gNB prepares the handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB, which includes a transparent container to be sent to the UE as an RRC message to perform the handover.

NOTE:	As soon as the source gNB receives the HANDOVER REQUEST ACKNOWLEDGE, or as soon as the transmission of the handover command is initiated in the downlink, data forwarding may be initiated.

NOTE:	For non-split bearer handover, downlink PDCP SDUs are forwarded with SN assigned by the source gNB for bearers subject to be simultaneously served by both the source and target gNBs, regardless of RLC-AM or RLC-UM, until SN assignment is handed over to the target gNB in step 8b, for which the normal data forwarding follows as defined in 9.2.3.2.3.

Editor’s note: FFS on the terminology to be used for non-split bearer handover solution. To be confirmed by RAN2.

6.	The source gNB triggers the Uu handover by sending an RRCReconfiguration message to the UE, containing the information required to access the target cell: at least the target cell ID, the new C-RNTI, the target gNB security algorithm identifiers for the selected security algorithms. It can also include a set of dedicated RACH resources, the association between RACH resources and SSB(s), the association between RACH resources and UE-specific CSI-RS configuration(s), common RACH resources, and system information of the target cell, etc.

NOTE:	In case of non-split bearer handover, the source gNB does not stop transmitting downlink packets until it receives the HO SUCCESS message from the target gNB in step 8a.

7.	The source gNB sends the SN STATUS TRANSFER message to the target gNB.

NOTE:	In case of non-split bearer handover, multiple SN STATUS TRANSFER messages are triggered by the source gNB. The downlink PDCP SN transmitter status conveyed for the first time for a bearer indicates PDCP SN and HFN of the first PDCP SDU that the source gNB forwards to the target gNB. The source gNB does not stop assigning PDCP SNs to downlink packets or delivering uplink packets to the UPF until it sends the last SN STATUS TRANSFER message to the target gNB in step 8b.

Editor’s note: FFS whether/how discarding of already forwarded PDCP SDUs is executed.

8.	The UE synchronises to the target cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to target gNB.

NOTE:	In case of non-split bearer handover, for duplication avoidance, the target gNB does not forward QoS flows of the uplink PDCP SDUs successfully received in-sequence to the UPF until it receives the SN STATUS TRANSFER message in step 8b, in which UL HFN and the first missing SN in the uplink PDCP SN receiver status indicates the start of uplink PDCP SDUs to be delivered to the UPF. The target gNB does not deliver any uplink packet which has an UL COUNT lower than the provided.

Editor’s note: FFS whether we allow the source to send an intermediate SN STATUS TRANSFER (between the moment UE disconnects from the souce and the last SN STATUS TRANSFER) to convey uplink out-of-sequence receiving status, for the target to send the PDCP status report to the UE immeditealy when accessed.

8a/b	In case of non-split bearer handover, the target gNB sends the HO SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the last SN STATUS TRANSFER message to the target gNB as described in step 7 and the normal data forwarding follows as defined in 9.2.3.2.3.

Editor’s note: The introduced step 8a and 8b are based on the working assumption, subject to change.

9.	The target gNB sends a PATH SWITCH REQUEST message to AMF to trigger 5GC to switch the DL data path towards the target gNB and to establish an NG-C interface instance towards the target gNB.

10.	5GC switches the DL data path towards the target gNB. The UPF sends one or more "end marker" packets on the old path to the source gNB per PDU session/tunnel and then can release any U-plane/TNL resources towards the source gNB.

11.	The AMF confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

12.	Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message from the AMF, the target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the handover. The source gNB can then release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

The RRM configuration can include both beam measurement information (for layer 3 mobility) associated to SSB(s) and CSI-RS(s) for the reported cell(s) if both types of measurements are available. Also, if CA is configured, the RRM configuration can include the list of best cells on each frequency for which measurement information is available. And the RRM measurement information can also include the beam measurement for the listed cells that belong to the target gNB.

The common RACH configuration for beams in the target cell is only associated to the SSB(s). The network can have dedicated RACH configurations associated to the SSB(s) and/or have dedicated RACH configurations associated to CSI-RS(s) within a cell. The target gNB can only include one of the following RACH configurations in the Handover Command to enable the UE to access the target cell:

i)	Common RACH configuration;

ii)	Common RACH configuration + Dedicated RACH configuration associated with SSB;

iii)	Common RACH configuration + Dedicated RACH configuration associated with CSI-RS.

The dedicated RACH configuration allocates RACH resource(s) together with a quality threshold to use them. When dedicated RACH resources are provided, they are prioritized by the UE and the UE shall not switch to contention-based RACH resources as long as the quality threshold of those dedicated resources is met. The order to access the dedicated RACH resources is up to UE implementation.