3GPP TSG-RAN WG3 Meeting #105bis	R3-196135

Chongqing, China, 14-18 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.473 | CR |  | rev | - | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR to 37.473 on RRC transfer over W1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, TIM, China Unicom, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_arch_evo-Core |  |  |  |  |  | Date: |  |  | 2019-10-16 |
| Category: | - |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The agreed RRC message Transfer procedures in RAN3 have not been captured in the current TS37.473. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following RRC message Transfer related procedures with corresponding tabular: - Initial UL RRC Message Transfer - UL RRC Message Transfer - DL RRC Message Transfer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC message Transfer procedures in case of disaggregated Ng-eNB architecture are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.x, 9.x1.y1, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | For the purpose of readability, some of the IEs which are introduced in other TPs, are also included in this TP, including: Message Type, Tansaction ID, E-UTRAN CGI, PLMN Identity. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

[x]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRAN); Radio Resource Control (RRC) Protocol Specification".

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.X	RRC Message Transfer procedures

8.X.1	Initial UL RRC Message Transfer

8.X.1.1	General

The purpose of the Initial UL RRC Message Transfer procedure is to transfer the initial RRC message to the ng-eNB-CU. The procedure uses non-UE-associated signaling.

8.X.1.2	Successful operation

Figure 8.X.1.2-1: Initial UL RRC Message Transfer procedure.

The establishment of the UE-associated logical W1-connection shall be initiated as part of the procedure.

8.X.1.3	Abnormal Conditions

Not applicable.

8.X.2	DL RRC Message Transfer

8.X.2.1	General

The purpose of the DL RRC Message Transfer procedure is to transfer an RRC message. The procedure uses UE-associated signalling.

8.X.2.2	Successful operation

Figure 8.X.2.2-1: DL RRC Message Transfer procedure

If a UE-associated logical W1-connection exists, the DL RRC MESSAGE TRANSFER message shall contain the ng-eNB-DU UE W1AP ID IE, which should be used by ng-eNB-DU to lookup the stored UE context. If no UE-associated logical W1-connection exists, the UE-associated logical W1-connection shall be established at reception of the DL RRC MESSAGE TRANSFER message.

The DL RRC MESSAGE TRANSFER message shall include, if available, the old ng-eNB-DU UE W1AP ID IE so that the ng-eNB-DU can retrieve the existing UE context in RRC connection reestablishment procedure, as defined in TS 38.401 [x].

If the ng-eNB-DU identifies the UE-associated logical W1-connection by the ng-eNB-DU UE W1AP ID IE in the DL RRC MESSAGE TRANSFER message and the old ng-eNB-DU UE W1AP ID IE is included, it shall release the old ng-eNB-DU UE W1AP ID and the related configurations associated with the old ng-eNB-DU UE W1AP ID.

8.X.2.3	Abnormal Conditions

Not applicable.

8.X.3	UL RRC Message Transfer

8.X.3.1	General

The purpose of the UL RRC Message Transfer procedure is to transfer an uplink RRC message as UL RLC SDU to the ng-eNB-CU. The procedure uses UE-associated signalling.

8.X.3.2	Successful operation

Figure 8.X.3.2-1: UL RRC Message Transfer procedure

When the ng-eNB-DU has received from the radio interface an RRC message to which a UE-associated logical W1-connection for the UE exists, the ng-eNB-DU shall send the UL RRC MESSAGE TRANSFER message to the ng-eNB-CU including the RRC message as a RRC-Container IE.

8.X.3.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

9.x1.y1	RRC Message Transfer messages

9.x1.y1.1	INITIAL UL RRC MESSAGE TRANSFER

This message is sent by the ng-eNB-DU to transfer the initial layer 3 message to the ng-eNB-CU over the W1 interface.

Direction: ng-eNB-DU ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | ignore |
| ng-eNB-DU UE W1AP ID | M |  | 9.x2.y2.2 |  | YES | reject |
| E-UTRAN CGI | M |  | 9.x2.y2.3 | NG-RAN Cell Global Identifier | YES | reject |
| C-RNTI | M |  | 9.x2.y2.4 | C-RNTI allocated at the ng-eNB-DU | YES | reject |
| RRC-Container | M |  | 9.x2.y2.5 | Includes the UL-CCCH-Message IE as defined in subclause 6.2 of TS 36.331 [x]. | YES | reject |
| DU to CU RRC Container | O |  | OCTET STRING | RadioResourceConfigDedicated IE as defined in subclause 6.3.2 in TS 36.331 [xx]. Required at least to carry SRB1 configuration. | YES | reject |
| Transaction ID | M |  | 9.x2.y2.6 |  | YES | Ignore |

9.x1.y1.2	DL RRC MESSAGE TRANSFER

This message is sent by the ng-eNB-CU to transfer the layer 3 message to the ng-eNB-DU over the W1 interface.

Direction: ng-eNB-CU ng-eNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | ignore |
| ng-eNB-CU UE W1AP ID | M |  | 9.x2.y2.7 |  | YES | reject |
| ng-eNB-DU UE W1AP ID | M |  | 9.x2.y2.2 |  | YES | reject |
| old ng-eNB-DU UE W1AP ID | O |  | 9.x2.y2.2 | Include it if RRCConnectionReestablishment is included in RRC-Container | YES | reject |
| SRB ID | M |  | 9.x2.y2.8 |  | YES | reject |
| RRC-Container | M |  | 9.x2.y2.5 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 36.331 [x] encapsulated in a PDCP PDU, or the DL-CCCH-Message IE as defined in subclause 6.2 of TS 36.331 [x]. | YES | reject |

9.x1.y1.3	UL RRC MESSAGE TRANSFER

This message is sent by the ng-eNB-DU to transfer the layer 3 message to the ng-eNB-CU over the W1 interface.

Direction: ng-eNB-DU ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | ignore |
| ng-eNB-CU UE W1AP ID | M |  | 9.x2.y2.7 |  | YES | reject |
| ng-eNB-DU UE W1AP ID | M |  | 9.x2.y2.2 |  | YES | reject |
| SRB ID | M |  | 9.x2.y2.8 |  | YES | reject |
| RRC-Container | M |  | 9.x2.y2.5 | Includes the UL-DCCH-Message IE as defined in subclause 6.2 of TS 36.331 [8], encapsulated in a PDCP PDU. | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

9.x2.y2.1	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Code | M |  | INTEGER (0..255) |  |
| >Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, ...) |  |

9.x2.y2.2	ng-NB-DU UE W1AP ID

The ng-eNB-DU UE W1AP ID uniquely identifies the UE association over the W1 interface within the ng-eNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ng-eNB-DU UE W1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

9.x2.y2.3	E-UTRAN CGI

This information element is used to globally identify a cell (see TS 36.401 [z]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.x2.y2.9 |  |
| Cell Identity | M |  | BIT STRING (SIZE(28)) | The leftmost bits of the Cell Identity correspond to the eNB ID. |

9.x2.y2.4	C-RNTI

This IE contains the C-RNTI information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| C-RNTI | M |  | INTEGER (0..65535, ...) | C-RNTI as defined in TS 36.331 [x]. |

9.x2.y2.5	RRC-Container

This information element contains an ng-eNB-CUUE or a UE  ng-eNB-CU message that is transferred without interpretation in the ng-eNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC-Container | M |  | OCTET STRING |  |

9.x2.y2.6	Transaction ID

The Transaction ID IE uniquely identifies a procedure among all ongoing parallel procedures of the same type initiated by the same protocol peer. Messages belonging to the same procedure shall use the same Transaction ID. The Transaction ID is determined by the initiating peer of a procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transaction ID | M |  | INTEGER (0..255, ...) |  |

9.x2.y2.7	ng-eNB-CU UE W1AP ID

The ng-eNB-CU UE W1AP ID uniquely identifies the UE association over the W1 interface within the ng-eNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ng-eNB-CU UE W1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

9.x2.y2.8	SRB ID

This IE uniquely identifies a SRB for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRB ID | M |  | INTEGER (0..2, ...) | Corresponds to the SRB-Identity defined in TS 36.331 [x]. |

9.x2.y2.9	PLMN Identity

This information element indicates the PLMN Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | OCTET STRING (SIZE(3)) | - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bits 4 to 1 of octet n encoding digit 2n-1 - bits 8 to 5 of octet n encoding digit 2n -The PLMN identity consists of 3 digits from MCC followed by either -a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or -3 digits from MNC (in case of a 3 digit MNC). |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>