3GPP TSG-RAN3 Meeting #105bis	R3-196168

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0258 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing procedural texts for NG interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Rev1: Only procedural text for the Time to Wait IE is added to comply with specification principles in section 4.1 for optional IEs. Rev0: Several optional IEs are missing the procedural texts, including: - UE Radio Capability for Paging IE in the INITIAL CONTEXT SETUP REQUEST message; - Default Paging DRX IE in the RAN CONFIGURATION UPDATE message; - Time to Wait IE in the RAN CONFIGURATION UPDATE FAILURE message and AMF CONFIGURATION UPDATE FAILURE message; - UE Presence in Area of Interest List IE in the LOCATION REPORT message; - RAN Node Name IE in the NG SETUP REQUEST message and RAN CONFIGURATION UPDATE message. In addtion, as described in section 4.1 of TS 38.413, the following procedure specification principles are given. - Any required inclusion of an optional IE in a response message is explicitly indicated in the procedure text. If the procedure text does not explicitly indicate that an optional IE shall be included in a response message, the optional IE shall not be included Following the principle above, the optional IEs in the response message should be described clearly. While for IEs in the request message, the procedural texts are needed as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedural texts on the Time to wait IE. Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release) since only procedural texts are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the inclusion of optional IEs is not explicitly indicated in the related procedue text, these IEs shall not be included, according to section 4.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.2.3; 8.7.3.3; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | First version: R3-195340 . |  |  |  |  |  |  |  |  |

Change Begins

### 8.7.2	RAN Configuration Update

<Unchanged Text Omitted>

#### 8.7.2.3	Unsuccessful Operation

Figure 8.7.2.3-1: RAN configuration update: unsuccessful operation

If the AMF cannot accept the update, it shall respond with a RAN CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the Time to Wait IE is included in the RAN CONFIGURATION UPDATE FAILURE message, the NG-RAN node shall wait at least for the indicated time before reinitiating the RAN Configuration Update procedure towards the same AMF.

Nex Change

### 8.7.3	AMF Configuration Update

<Unchanged Text Omitted>

#### 8.7.3.3	Unsuccessful Operation

Figure 8.7.3.3-1: AMF configuration update: unsuccessful operation

If the NG-RAN node cannot accept the update, it shall respond with an AMF CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the Time to Wait IE is included in the AMF CONFIGURATION UPDATE FAILURE message, the AMF shall wait at least for the indicated time before reinitiating the AMF Configuration Update procedure towards the same NG-RAN node.

Change Ends