3GPP TSG-RAN WG3 Meeting #105bis	R3-196268

Chongqing, China, 14 – 18 October, 2019

Agenda item:	10.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for SON BL CR for TS 38.463): MLB for E1

Document for:	Text Proposal

# 1	Introduction

This TP captures agreements from RAN3#105bis.

# References

[1]	R3-194772, BL CR to TS 38.463

# Annex	- TP for BL CR for TS 38.463

Beginning of Text Proposal to TS 38.463

#### 9.2.1.X1	RESOURCE STATUS REQUEST

Editor’s note: The content of this section is FFS

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStoporRemoveorAdd |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | Ignore |
| Registration Request | M |  | ENUMERATED(start, stop, partial stop, add, …,) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-CU-UP is requested to report. First Bit = TNL Available Capacity Ind Periodic, Second Bit = HW Load Ind Periodic [FFS]. Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 12000ms, …) | Periodicity that can be used for reporting. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporPartialStoporAdd | This IE shall be present if the Registration Request IE is set to the value “stop”, “partial stop” or “add”. |

#### 9.2.1.X2	RESOURCE STATUS RESPONSE

Editor’s note: The content of this section is FFS

This message is sent by the gNB-CU-UP to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | Ignore |

#### 9.2.1.X3	RESOURCE STATUS FAILURE

Editor’s note: The content of this section is FFS

This message is sent by the gNB-CU-UP to indicate that for none of the requested measurement objects the measurement can be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStoporRemoveorAdd |  |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  |  | 9.3.1.2 | Ignored by the receiver when the Complete Failure Cause Information IE is included | YES | ignore |
| Criticality Diagnostics |  | O |  | 9.3.1.3 |  | YES | Ignore |

#### 9.2.1.X4	RESOURCE STATUS UPDATE

Editor’s note: The content of this section is FFS

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | Reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | reject |
| >>TNL Available Capacity Indicator | O |  | 9.3.x.x |  |  |  |
| >>HW Load Indicator [FFS] | O |  | 9.3.x.x |  |  |  |
| Slice Measurement Result [FFS] |  | 1 |  |  | YES | ignore |
| >Slice Measurement Result Item |  | 1 .. < maxnoofSliceItems> |  |  | EACH | ignore |
| >>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| >>TNL Available Capacity Indicator | O |  | 9.3.x.x |  |  |  |
| >>HW Load Indicator [FFS] | O |  | 9.3.x.x |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

Next Text Proposal to TS 38.463

#### 9.3.1.x1	TNL Available Capacity Indicator

[The definition of TNL Available Capacity Indicator is FFS]

The TNL Available Capacity Indicator IE indicates the status of the Transport Network Layer available capacity at the gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL TNL Load | M |  | INTEGER(0..100, …) | Indicates the amount of downlink transport network level available capacity |
| UL TNL Load | M |  | INTEGER(0..100, …) | Indicates the amount of uplink transport network level available capacity |

#### 9.3.1.x1	HW Load Indicator

[The definition of HW Load is FFS]

The HW Load Indicator IE indicates the status of the Hardware Load experienced by the gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Hardware Load | M |  | INTEGER(0..100, …) | Indicates the amount of hardware resources utilized |

End of Text Proposal to TS 38.463