3GPP TSG-RAN3 Meeting #105b	R3-196527

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0237 | rev | 4 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SON features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of SON features |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new Inter-system SON Configuration Transfer IE in the DL and UL configuration transfer for inter-system exchange Addition of a SON Information Report for intra system exchange |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SON features not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.8.1.2, 8.8.2.2, 9.2.7.1, 9.2.7.2, 9.3.1.x, 9.3.3.7, 9.3.3.x1, 9.3.3.x2, 9.3.3.Y1, 9.3.3.Y2, 9.3.3.Y3, 9.3.3.Y4, 9.3.3.Y5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.300 CR TS 36.413 CR 1710 TS 36.423 CR 1373 TS 38.423 CR 0221 TS 38.463 CR 0142 TS 38.473 CR 0441 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Editorial update and added an FFS on scenario of an RLF report generated by NG-RAN RRC being sent from E-UTRAN to NG-RAN. Rev2: resubmitted to RAN3#105b Rev3: includes the agreed TP in R3-196155 Rev4: Resubmitted to RAN3#106 |  |  |  |  |  |  |  |  |

Start of the first change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture description".

[3]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[4]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[5]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[6]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[7]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[8]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[9]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[11]	3GPP TS 32.422: "Trace control and configuration management".

[12]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode and in RRC inactive state".

[13]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[14]	3GPP TS 38.414: "NG-RAN; NG data transport".

[15]	3GPP TS 29.281: "General Packet Radio System (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[16]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[17]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[18]	3GPP TS 38.331: "NG-RAN; Radio Resource Control (RRC) Protocol Specification".

[19]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[20]	3GPP TS 23.007: "Technical Specification Group Core Network Terminals; Restoration procedures".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[25]	IETF RFC 5905 (2010-06): "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[26]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[27]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[28]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[29]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[r1]	3GPP TS 36.401: "E-UTRAN Architecture Description".

[r2]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

Start of the next change

## 8.8	Configuration Transfer Procedures

### 8.8.1	Uplink RAN Configuration Transfer

#### 8.8.1.1	General

The purpose of the Uplink RAN Configuration Transfer procedure is to transfer RAN configuration information from the NG-RAN node to the AMF. The AMF does not interpret the transferred RAN configuration information. This procedure uses non-UE associated signalling.

#### 8.8.1.2	Successful Operation

Figure 8.8.1.2-1: Uplink RAN configuration transfer

The NG-RAN node initiates the procedure by sending the UPLINK RAN CONFIGURATION TRANSFER message to the AMF.

If the AMF receives the SON Configuration Transfer IE, it shall transparently transfer the SON Configuration Transfer IE towards the NG-RAN node indicated in the Target RAN Node ID IE which is included in the SON Configuration Transfer IE.

If the AMF receives the EN-DC SON Configuration Transfer IE, it shall transparently transfer the EN-DC SON Configuration Transfer IE towards an MME serving the eNB indicated in the Target eNB-ID IE which is included in the EN-DC SON Configuration Transfer IE.

If the AMF receives the Inter-system SON Configuration Transfer IE, it shall transparently transfer the Inter-system SON Configuration Transfer IE towards an MME serving the eNB indicated in the Target eNB-ID IE which is included in the Inter-system SON Configuration Transfer IE.

#### 8.8.1.3	Abnormal Conditions

Void.

### 8.8.2	Downlink RAN Configuration Transfer

#### 8.8.2.1	General

The purpose of the Downlink RAN Configuration Transfer procedure is to transfer RAN configuration information from the AMF to the NG-RAN node. This procedure uses non-UE associated signalling.

#### 8.8.2.2	Successful Operation

Figure 8.8.2.2-1: Downlink RAN configuration transfer

The procedure is initiated with an DOWNLINK RAN CONFIGURATION TRANSFER message sent from the AMF to the NG-RAN node.

If the NG-RAN node receives, in the SON Configuration Transfer IE or in the EN-DC SON Configuration Transfer IE, the SON Information IE containing the SON Information Request IE, it may transfer back the requested information either towards the NG-RAN node indicated in the Source RAN Node ID IE of the SON Configuration Transfer IE or towards an eNB indicated in the Source eNB-ID IE of the EN-DC SON Configuration Transfer IE by initiating the Uplink RAN Configuration Transfer procedure.

If the NG-RAN node receives, in the SON Configuration Transfer IE, the Xn TNL Configuration Info IE containing the Xn Extended Transport Layer Addresses IE, it may use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the NG-RAN node receives, in the SON Configuration Transfer IE, the SON Information IE containing the SON Information Reply IE including the Xn TNL Configuration Info IE as an answer to a former request, it may use it to initiate the Xn TNL establishment.

In case the IP-Sec Transport Layer Address IE is present and the GTP Transport Layer Addresses IE within the Xn Extended Transport Layer Addresses IE is not empty, GTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE.

In case the IP-Sec Transport Layer Address IE is not present, GTP traffic is terminated at the endpoints given by the list of addresses in the Xn GTP Transport Layer Addresses IE within the Xn Extended Transport Layer Addresses IE.

In case the Xn GTP Transport Layer Addresses IE is empty and the IP-Sec Transport Layer Address IE is present, SCTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE, within the Xn Extended Transport Layer Addresses IE.

If the NG-RAN node receives the SON Information IE containing the SON Information Report IE it may use it as specified in TS 38.300 [8].

If the NG-RAN node receives the Inter-system SON Information IE containing the Inter-System SON Information Report IE it may use it as specified in TS 38.300 [8].

Editor’s note: The scenario of an RLF report generated by NG-RAN RRC being sent from E-UTRAN to NG-RAN is FFS

If the NG-RAN node is configured to use one IPsec tunnel for all NG and Xn traffic (IPsec star topology) then the traffic to the peer NG-RAN node shall be routed through this IPsec tunnel and the IP-Sec Transport Layer Address IE shall be ignored.

#### 8.8.2.3	Abnormal Conditions

Void.

Start of the first change

#### 9.2.7.1	UPLINK RAN CONFIGURATION TRANSFER

This message is sent by the NG-RAN node in order to transfer RAN configuration information.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.3.3.6 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | OCTET STRING | Contains the EN-DC SON Configuration Transfer IE as defined in TS 36.413 [16]. | YES | ignore |
| Inter-system SON Configuration Transfer | O |  | 9.3.3.X1 |  | YES | ignore |

#### 9.2.7.2	DOWNLINK RAN CONFIGURATION TRANSFER

This message is sent by the AMF in order to transfer RAN configuration information.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.3.3.6 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | OCTET STRING | Contains the EN-DC SON Configuration Transfer IE as defined in TS 36.413 [16]. | YES | ignore |
| Inter-system SON Configuration Transfer | O |  | 9.3.3.X1 |  | YES | ignore |

Start of the next change

#### 9.3.1.x	Global eNB ID

This information element is used to globally identify an eNB (see TS 36.401 [r1]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.3.8 |  |
| CHOICE eNB ID | M |  |  |  |
| >Macro eNB ID |  |  |  |  |
| >>Macro eNB ID | M |  | BIT STRING (SIZE(20)) | Equal to the 20 leftmost bits of the Cell Identity IE contained in the E-UTRAN CGI IE (see TS 36.401 [r1] subclause 9.3.1.9) of each cell served by the eNB. |
| >Home eNB ID |  |  |  |  |
| >>Home eNB ID | M |  | BIT STRING (SIZE(28)) | Equal to the Cell Identity IE contained in the E-UTRAN CGI IE (see TS 36.401 [r1] subclause 9.3.1.9) of the cell served by the eNB. |
| >Short Macro eNB ID |  |  |  |  |
| >> Short Macro eNB ID | M |  | BIT STRING (SIZE(18)) | Equal to the 18 leftmost bits of the Cell Identity IE (see TS 36.401 [r1] subclause 9.3.1.9) of each cell served by the eNB. |
| >Long Macro eNB ID |  |  |  |  |
| >> Long Macro eNB ID | M |  | BIT STRING (SIZE(21)) | Equal to the 21 leftmost bits of the Cell Identity IE (see TS 36.401 [r1] subclause 9.3.1.9) of each cell served by the eNB. |

Start of the next change

#### 9.3.3.7	SON Information

This IE identifies the nature of the configuration information transferred, i.e., a request, a reply or a report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information | M |  |  |  |
| >SON Information Request |  |  |  |  |
| >>SON Information Request | M |  | ENUMERATED (Xn TNL Configuration Info, …) |  |
| >SON Information Reply |  |  |  |  |
| >>SON Information Reply | M |  | 9.3.3.8 |  |
| >SON Information Report |  |  |  |  |
| >>SON Information Report | M |  | 9.3.3.Y1 |  |

Start of the next change

#### 9.3.3.X1	Inter-system SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, transmitted between an NG-RAN node and an eNB and additionally includes the node identifier of the destination of this configuration information and the node identifier of the source of this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-system SON Configuration Transfer |  |  |  |  |
| >CHOICE Transfer Type | M |  |  |  |
| >>from E-UTRAN to NG-RAN |  |  |  |  |
| >>>Source eNB-ID |  | 1 |  |  |
| >>>>Global eNB ID | M |  | Global eNB ID 9.3.1.x |  |
| >>>>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  |
| >>>Target NG-RAN node ID |  | 1 |  |  |
| >>>>Global RAN node ID | M |  | 9.3.1.5 |  |
| >>>>Selected TAI | M |  | TAI 9.3.3.11 |  |
| >>from NR to E-UTRAN |  |  |  |  |
| >>>Source NG-RAN node ID |  | 1 |  |  |
| >>>>Global RAN node ID | M |  | 9.3.1.5 |  |
| >>>>Selected TAI | M |  | TAI 9.3.3.11 |  |
| >>>Target eNB-ID |  | 1 |  |  |
| >>>>Global eNB ID | M |  | Global eNB ID 9.3.1.x |  |
| >>>>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  |
| >Inter-system SON Information | M |  | 9.3.3.X2 |  |

Start of the next change

#### 9.3.3.X2	Inter-system SON Information

This IE identifies the nature of the configuration information transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information | M |  |  |  |
| >Inter-system SON Information Report |  |  |  |  |
| >>Inter-system SON Information Report | M |  | 9.3.3.Y2 |  |

#### 9.3.3.Y1	SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| > Failure Indication Information |  |  |  |  |
| >> Failure Indication | M |  | 9.3.3.Y3 |  |
| >HO Report Information |  |  |  |  |
| >>HO Report | M |  | 9.3.3.Y4 |  |

#### 9.3.3.Y2	Inter-system SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >HO Report Information |  |  |  |  |
| >> Inter-system HO Report | M |  | 9.3.3.Y5 |  |

#### 9.3.3.Y3	Failure Indication

This IE contains the failure indication to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE RLF Report Container | O |  | OCTET STRING | FFS |

#### 9.3.3.Y4	HO Report

This IE contains the HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter System ping-pong, …) |  |
| Handover Cause | M |  | Cause 9.2.3.1 | Indicates handover cause employed for handover from source cell |
| Source cell CGI | M |  | NG-RAN CGI 9.3.1.73 | NG-RAN CGI of the source cell for handover procedure |
| Target cell CGI | C- ifUERLFReportContainerAbsent |  | NG-RAN CGI 9.3.1.73 | NG-RAN CGI of the target cell for handover procedure. If the Handover Report Type is set to “Inter System ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node cell |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | NG-RAN CGI 9.3.1.73 | NG-RAN CGI of the cell where UE attempted re-establishment |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | E-UTRA CGI 9.3.1.9 | E-UTRA CGI of the E-UTRAN target cell for handover procedure |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message from the source NG-RAN node |
| UE RLF Report Container | O |  | OCTET STRING | The UE RLF Report Container IE received in the FAILURE INDICATION message. |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportTypeHoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportTypeIntersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter System ping-pong" |
| ifUERLFReportContainerAbsent | This IE shall be present if the UE RLF Report Container IE is absent |

#### 9.3.3.Y5	Inter-system HO Report

This IE contains the Inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |
| >Too early Inter-system HO |  |  |  |  |
| >>Source cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |
| >>Failure cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |
| >>UE RLF Report Container | FFS |  | OCTET STRING |  |
| >Inter-system Unnecessary HO |  |  |  |  |
| >>Source cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |
| >>Target cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |
| >>Early IRAT HO | M |  | ENUMERATED (no, yes, ...) | Is set to “yes” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “no” |
| >>Candidate Cell List |  | 1 |  |  |
| >>>Candidate Cell Item |  | 1,, .. <maxnoofCandidateCells> |  |  |
| >>>>Candidate Cell ID | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI. |
| >>>Candidate PCI List |  | 0..1 |  |  |
| >>>>Candidate PCI Item |  | 1 .. <maxnoofCandidateCells> |  |  |
| >>>>>Candidate PCI | M |  | INTEGER (0..1007) | This IE includes the Primary Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NR CGI could not be derived. |
| >>>>>Candidate NR ARFCN | M |  | INTEGER (0.. maxNARFCN) | RF Reference Frequency as defined in TS 38.104 [r2], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. |
| maxNARFCN | Maximum value of NR carrier frequency, defined in TS 38.331 [18] |