3GPP TSG-RAN WG3 #106							                                                R3-196627

Reno, USA, 18th –22nd November 2019

Agenda item:			8.3.2

Source:					ZTE

Title:	 (TP for Introduction of NR_IIOT support to TS 38.413): Extending the maximum value range of MDBV

Document for:		Approval

- Introduction

As defined in TS 23.501, Maximum Data Burst Volume (MDBV) denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB. Based on the S2-1910663, SA2 would like to request RAN3, CT3 and CT4 to update the corresponding specifications to support the increased maximum allowed value of MDBV for signalled 5QI characteristics, to 2000kbytes.

To support this, the maximum value range of MDBV should be extended from 4095 bytes to 2000000bytes, and Maximum Data Burst Volume should be provided from SMF to 5G-AN.

Detailed analysis can be seen in the related discussion paper (R3-196626).

- Text Proposal (TS38.413 15.5.0)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

  - 9.3.1.83	Maximum Data Burst Volume

This IE indicates the Maximum Data Burst Volume for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Data Burst Volume | M |  | INTEGER (0..4095,..., 4096..2000000, …) | Unit: byte. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>