3GPP TSG-RAN WG3 Meeting #106	R3-196766

Reno,  USA, 18 – 22 November 2019

Agenda item:	20.2.2.

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for 5G_V2X_NRSL BL CR for TS 38.473) V2X SIB

WID/SID:	NR_IAB-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Based on the discussion in ([1]), this contribution proposes the TP to TS38.473 to support V2X SIB.

# References

[1]	R3-196765, Discussion on V2X support over F1 interface

Proposed TP for TS38.473 BL CR:

-------------------------------------------Start of Change -------------------------------------------

#### 9.3.1.18	gNB-DU System Information

This IE contains the system information generated by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MIB message | M |  | OCTET STRING | MIB message, as defined in TS 38.331 [8]. | - |  |
| SIB1 message | M |  | OCTET STRING | SIB1 message, as defined in TS 38.331 [8]. | - |  |
| SIBx message | O |  | OCTET STRING | V2X SIBx message, as defined in TS 38.331 [8]. | - | ignore |

Editor’s Note: The V2X SIB message name will be renamed after RAN2 finalize the V2X SIB.

-------------------------------------------Next Change -------------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

>>>	Unaffected parts skipped    <<<

id-CG-Config,

id-Ph-InfoMCG,

(FFS)id-LTEV2XSidelinkInfoList,

(FFS)id-NRSidelinkInfoList,

id-V2XSIB,

maxNRARFCN,

>>>	Unaffected parts skipped    <<<

GNB-DU-Served-Cells-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-DU-System-Information ::= SEQUENCE {

mIB-message		MIB-message,

sIB1-message		SIB1-message,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-System-Information-ExtIEs } } OPTIONAL,

...

}

GNB-DU-System-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-V2XSIB CRITICALITY ignore EXTENSION V2XSIB PRESENCE optional},

...

}

GNB-DUConfigurationQuery ::= ENUMERATED {true, ...}

>>>	Unaffected parts skipped    <<<

#### -- V

V2XSIB ::= OCTET STRING

#### -- W

--------------------------------------------------------------Next CHANGE --------------------------------------------------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

>>>	Unaffected parts skipped    <<<

id-Cell-Type										ProtocolIE-ID ::= 232

id-IgnorePRACHConfiguration							ProtocolIE-ID ::= 233

id-NRV2XServicesAuthorized							ProtocolIE-ID ::= xxx

id-LTEV2XServicesAuthorized							ProtocolIE-ID ::= xxy

id-NRUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= xyy

id-LTEUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= yyy

(FFS)id-LTEV2XSidelinkInfoList						ProtocolIE-ID ::= xxz

(FFS)id-NRSidelinkInfoList							ProtocolIE-ID ::= xzz

id-V2XSIB											ProtocolIE-ID ::= xz1