3GPP TSG-RAN WG3 Meeting #106	R3-196847

Reno (NV), USA, 18 – 22 November, 2019

Agenda item:	10.2.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP for SON BL CR for TS 38.423] Introduction of RACH failure information

Document for:	Text Proposal

# Introduction

This TP includes updates to the SON BL CR for TS 38.423, introducing RACH failure outcome information in the XnAP FAILURE INDICATION message as proposed in [1].

# References

[1]	R3-196845, Further discussion on RACH optimization, Nokia, Nokia Shanghai Bell

# TP for BL CR for TS 38.423

<<< First change >>>

#### 9.1.3.x	FAILURE INDICATION

This message is sent by the NG-RAN node2 either to indicate an RRC re-establishment attempt or a reception of an RLF Report from a UE that suffered a connection failure at NG-RAN node 1, or to provide RACH failure information to NG-RAN node1.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.X |  | YES | ignore |
| CHOICE Initiating condition | M |  |  |  |  |  |
| > RRC Reestab |  |  |  |  |  |  |
| >>Failure cell PCI | C- ifUERLFReportContainerAbsent |  | 9.2.2.10 | Physical Cell Identifier | YES | ignore |
| >>Re-establishment cell CGI | C- ifUERLFReportContainerAbsent |  | NG-RAN CGI 9.2.2.x |  | YES | ignore |
| >>C-RNTI | C- ifUERLFReportContainerAbsent |  | BIT STRING (SIZE (16)) | C-RNTI contained in the RRC Re-establishment Request message (TS 38.331) [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | YES | ignore |
| >>ShortMAC-I | C- ifUERLFReportContainerAbsent |  | BIT STRING (SIZE (16)) | ShortMAC-I contained in the RRC Re-establishment Request message (TS 38.331) [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | YES | ignore |
| >>UE RLF Report Container | O |  | OCTET STRING | RLF -Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10] or TS 36.331 [14]) FFS | YES | ignore |
| > RRC Setup |  |  |  |  |  |  |
| >>UE RLF Report Container | O |  | OCTET STRING | RLF -Report-r16 IE contained in the UEInformationResponse message (TS 38.331) FFS | YES | ignore |
| > RACH Outcome |  |  |  |  |  |  |
| >> RACH Outcome Information |  |  | 9.2.2.K |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifUERLFReportContainerAbsent | This IE shall be present if the UE RLF Report Container IE is absent |

<<< next change >>>

#### 9.2.2.K	RACH Outcome Information

[The information is FFS]

The RACH Outcome Information IE provides information accumulated from UE RACH Reports received at the NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Cell ID | M |  | 9.2.3.25 |  |  |  |
| >>SSB Item |  | 1..< maxnoofSSBtems > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBtems-1) |  |  |  |
| >>>Failed RACH Attempts | M |  | INTEGER (0..1000) | Number of failed RACH attempts per SSB ID during the reporting period. If the value exceeds 1000, value 1000 is reported. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBItems [FFS] | Maximum no. of SSB items. Value is 64. [FFS] |
| maxnoofSSBItems-1 [FFS] | Maximum no. of SSB items minus 1. Value is 63. [FFS] |

<<< End of changes >>>