3GPP TSG-RAN WG3 Meeting #106                                                                           	R3-197037

Reno, USA, 18th - 22th Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP on BL CR for 36.413)Support of MDT configuration for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently,there is no support of NR MDT configruation for EN-DC capable UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NR MDT configuration in Trace activation IE. Add SgNB configured IE in E-RAB Modification Indication message. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the MDT function. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MDT for NR could not be supported for EN-DC sceanrio. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.13,9.2.1.4,9.2.1.X,9.2.1.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----First change----

9.1.4.13	UE CONTEXT MODIFICATION INDICATION

This message is sent by the eNB to request the MME to modify the UE Context information.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| CSG Membership Info |  | 0..1 |  |  | YES | reject |
| >CSG Membership Status | M |  | 9.2.1.73 |  | - |  |
| >CSG Id | M |  | 9.2.1.62 |  | - |  |
| >Cell Access Mode | O |  | 9.2.1.74 |  | - |  |
| >PLMN Identity | O |  | 9.2.3.8 |  | - |  |
| SgNB Configured | O |  | ENUMRATED(True,False…) |  |  |  |

-----Next change----

9.2.1.4	Trace Activation

Defines parameters related to a trace activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRAN Trace ID | M |  | OCTET STRING (SIZE(8)) | The E-UTRAN Trace ID IE is composed of the following: Trace Reference defined in TS 32.422 [10] (leftmost 6 octets, with PLMN information coded as in 9.2.3.8), and Trace Recording Session Reference defined in TS 32.422 [10] (last 2 octets). |  |  |
| Interfaces To Trace | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents an eNB or en-gNB interface: first bit =S1-MME, second bit =X2, third bit =Uu, fourth bit =F1-C, fifth bit =E1: other bits reserved for future use. Value ‘1’ indicates ‘should be traced’. Value ‘0’ indicates ‘should not be traced’. |  |  |
| Trace depth | M |  | ENUMERATED( minimum, medium, maximum, MinimumWithoutVendorSpecificExtension, MediumWithoutVendorSpecificExtension, MaximumWithoutVendorSpecificExtension, …) | Defined in TS 32.422 [10]. |  |  |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.2.2.1 | Defined in TS 32.422 [10]. |  |  |
| MDT Configuration | O |  | 9.2.1.81 |  | YES | ignore |
| UE Application layer measurement configuration | O |  | 9.2.1.128 |  | YES | Ignore |
| NR MDT Configuration | O |  | 9.2.1.X |  | YES | Ignore |

-----Next change----

9.2.1.X	NR MDT Configuration

The IE defines the MDT configuration parameters for NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED(Immediate MDT only, Logged MDT only,…,) |  | - | - |
| CHOICE Area Scope of MDT | M |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for MDT |  | 1 .. <maxnoofNRCellIDforMDT> |  |  |  | - |
| >>>NR CGI | M |  | 9.2.1.Y |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for MDT |  | 1 .. <maxnoofNRTAforMDT> |  |  |  | - |
| >>>5GS TAC | M |  | 9.2.3.53 | The TAI is derived using the current serving PLMN. | - | - |
| >PLMN Wide |  |  | NULL |  | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for MDT |  | 1 .. <maxnoofNRTAforMDT> |  |  | - | - |
| >>>5GS TAI | M |  | 9.2.3.52 |  | - | - |
| CHOICE MDT Mode | M |  |  |  | - | - |
| >Immediate MDT |  |  |  |  |  | - |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [31]. First Bit = M1, Second Bit= M2, Third Bit = M3, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - | - |
| >>M1 Reporting Trigger | M |  | ENUMERATED (periodic, A2event-triggered, …, A2event-triggered periodic) | This IE shall be ignored if the Measurements to Activate IE has the first bit set to “0”. | - | - |
| >>M1 Threshold Event A2 | C-ifM1A2trigger |  |  | Included in case of event-triggered or event-triggered periodic reporting for measurement M1. | - | - |
| >>>CHOICE Threshold | M |  |  |  | - | - |
| >>>>RSRP |  |  |  |  |  | - |
| >>>>>Threshold RSRP | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [16]. | - | - |
| >>>>RSRQ |  |  |  |  |  | - |
| >>>>>Threshold RSRQ | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [16]. | - | - |
| >>M1 Periodic reporting | C-ifperiodicMDT |  |  | Included in case of periodic or event-triggered periodic reporting for measurement M1. | - | - |
| >>>Report interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30) | This IE is defined in TS 38.331 [16]. | - | - |
| >>>Report amount | M |  | ENUMERATED (r1, r2, r4, r8, r16, r32, r64, infinity) | Number of reports. This IE is defined in TS 38.331 [16]. | - | - |
| >>M3 Configuration | C-ifM3 |  | 9.2.1.86 (It depends on the discussion in RAN2 on whether update on 9.2.1.86 is needed) |  | YES | ignore |
| >>M4 Configuration | C-ifM4 |  | 9.2.1.87(It depends on the discussion in RAN2 on whether update on 9.2.1.87 is needed) |  | YES | ignore |
| >>M5 Configuration | C-ifM5 |  | 9.2.1.88(It depends on the discussion in RAN2 on whether update on 9.2.1.88 is needed) |  | YES | ignore |
| >>MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [31]. First Bit = GNSS Second Bit = E-CID information. Other bits are reserved for future use and are ignored if received. Value “1” indicates “activate” and value “0” indicates “do not activate”. The gNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to “1”. | YES | ignore |
| >>M6 Configuration | C-ifM6 |  | 9.2.1.101(It depends on the discussion in RAN2 on whether update on 9.2.1.101 is needed) |  | YES | ignore |
| >>M7 Configuration | C-ifM7 |  | 9.2.1.102(It depends on the discussion in RAN2 on whether update on 9.2.1.102 is needed) |  | YES | ignore |
| >>Bluetooth Measurement Configuration | O |  | 9.2.1.137 |  | YES | Ignore |
| >>WLAN Measurement Configuration | O |  | 9.2.1.138 |  | YES | Ignore |
| >Logged MDT |  |  |  |  |  | - |
| >>Signalling quality | O |  |  |  |  |  |
| >>>Logging interval | M |  | TBD | This IE should be defined in TS 38.331 [16]. Unit: [second]. | - | - |
| >>>Logging duration | M |  | TBD | This IE should be defined in TS 38.331 [16]. Unit: [minute]. | - | - |
| >>Bluetooth Measurement Configuration | O |  | 9.2.1.137 |  | YES | Ignore |
| >>WLAN Measurement Configuration | O |  | 9.2.1.138 |  | YES | Ignore |
| >>Out of coverage | O |  | ENUMERATED(TRUE,…) |  |  |  |
| >>>Logging interval | M |  | TBD | This IE should be defined in TS 38.331 [16]. Unit: [second]. |  |  |
| >>>Logging duration | M |  | TBD | This IE should be defined in TS 38.331 [16]. Unit: [minute]. |  |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.1.89 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifM1A2trigger | This IE shall be present if the Measurements to Activate IE has the first bit set to “1” and the M1 Reporting Trigger IE is set to “A2event-triggered” or to “A2event-triggered periodic”. |
| ifperiodicMDT | This IE shall be present if the M1 Reporting Trigger IE is set to “periodic”, or to “A2event-triggered periodic”. |
| ifM3 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fifth bit set to “1”. |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to “1”. |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the eighth bit set to “1”. |

-----Next change----

9.2.1.Y	NR CGI

This IE is used to globally identify an NR cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.3.8 |  |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) | The leftmost bits of the NR Cell Identity IE correspond to the gNB ID |