3GPP TSG-RAN WG3 Meeting #106	R3-197301

# Reno, U.S.A., 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.413 | CR | 1326 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On QoE measurement collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI 16 |  |  |  |  |  | Date: |  |  | 2019-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Take SA5, SA4 agreements on QoE into account. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce QMC ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete QoE funcationality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.127, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.2.1.127	UE Application layer measurement configuration

The purpose of the UE Application layer measurement configuration IE is to provide configuration information for the QoE function.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Container for application layer measurement configuration | M |  | Octet string (1..1000) |  | Yes | ignore |
| Area scope for UE application layer measurement configuration | M |  | Area scope for UE application layer measurement configuration 9.2.1.128 |  | Yes | ignore |
| Service Type | M |  | ENUMERATED (QMC for streaming service, QMC for MSTI service, ...) | This IE indicates the service type of UE application layer measurements. | Yes | ignore |
| QMC ID | O |  | OCTET STRING (SIZE(4)) | The content is defined in TS 24.405 [x] | Yes | ignore |

************ Next Change **************

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

RANAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

maxNrOfCSGs,

maxNrOfErrors,

maxNrOfPDPDirections,

maxNrOfPoints,

maxNrOfRABs,

maxNrOfSRBs,

maxNrOfSeparateTrafficDirections,

maxRAB-Subflows,

maxRAB-SubflowCombination,

maxNrOfLevels,

maxNrOfAltValues,

maxNrOfSNAs,

maxNrOfLAs,

maxNrOfPLMNsSN,

maxSet,

maxNrOfHSDSCHMACdFlows-1,

maxNrOfUEsToBeTraced,

maxNrOfInterfaces,

************ Next Change **************

UE-Application-Layer-Measurement-Configuration	::= SEQUENCE {

applicationLayerContainerForMeasurementConfiguration		OCTET STRING ( SIZE (1..1000)),

areaScopeForUEApplicationLayerMeasurementConfiguration					AreaScopeForUEApplicationLayerMeasurementConfiguration,

...,

serviceType													ServiceType,

qMCID														QMC-ID

}

QMC-ID					::= OCTET STRING (SIZE (4))