3GPP TSG-RAN3 Meeting #106	R3-197397

Reno, Nevada, US, 18th November – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR |  | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of RLF in the gNB-DU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The gNB-DU can detect RLF events that are due to root causes monitored by the gNB-DU only. An example is of RLF causes detected by the gNB-DU only are maximum number of RLC retransmissions and maximum number of out-of-sync events. Without knowledge of these root cause of failures the gNB-CU does not have a complete view of why RLF events occurred |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new procedure in F1AP to indicate from the gNB-DU to gNB-CU that an RLF has occurred and to specify the root cause of the RLF. Impact assessment towards the previous version of the specification (same release): This CR has limited impact since a procedure is added that can be ignored by the receiving node if not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The gNB-CU does not have a full view of the root cause of occurred RLF events and it cannot correctly judge the best correction to apply to prevent further RLFs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.x (new), 9.2.1.x (new), 9.3.1.xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

### 8.2.x	Radio Link Failure Indication

#### 8.2.x.1	General

The Radio Link Failure Indication procedure is initiated by gNB-DU in order to report a detected Radio Link Failure.

#### 8.2.x.2	Successful Operation

Figure 8.2.x.2-1: Radio Link Failure Indication procedure. Successful operation

When gNB-DU detects Radio Link Failure, the Radio Link Failure Indication procedure is initiated by a RADIO LINK FAILURE INDICATION message sent from the gNB-DU.

#### 8.2.x.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.x	RADIO LINK FAILURE INDICATION

This message is sent by the gNB-DU and is used to indicate that a Radio Link Failure has been detected in the gNB-DU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RLF Type | M |  | 9.3.1.x |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< 3th Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.xx	RLF Type

This IE indicates the type of RLF detected by the gNB-DU for a UE connection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RLF Type | M |  | ENUMERATED (max num RLC retransmissions, out of sync, ...) |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>