3GPP TSG-RAN WG3 Meeting #106	R3-197493

# Reno, NV, USA, November 18th – 22nd 2019

Agenda Item:	13.1

Source:	Ericsson

Title:		Response to R3-196502, R3-196503 and R3-196505

Document for:	Agreement

# Discussion

The common understanding across different WGs is that the IAB-MT is a subset of UE functionality, and not a full-fledged UE. Nevertheless, the IAB BL CRs for TS 36.413, TS 38.413 and TS 36.423 contain misleading statements that do not accurately reflect the above common understanding. The proper reformulations can be executed via Editor’s corrections. The misleading statements and the proposed reformulations are:

- In S1AP IAB BL CR (R3-196502):

  - Clause 8.6.2.1: “If the IAB Node Indication IE is included in the INITIAL UE MESSAGE message, the MME shall consider the UE as an IAB node.”

  - We propose the following reformulation:

    - “If the IAB Node Indication IE is included in the INITIAL UE MESSAGE, the MME shall consider that the message refers to an IAB node.”

- In X2AP IAB BL CR (R3-196503):

  - Clause 8.7.4.2: “If the SGNB ADDITION REQUEST message contains the IAB Node Indication IE, the en-gNB shall, if supported, consider the UE as an IAB node.”

  - We propose the following reformulation:

    - “If the SGNB ADDITION REQUEST message contains the IAB Node Indication IE, the en-gNB shall, if supported, consider that the request is for an IAB node.”

  - Clause 8.7.6.2: “If the SGNB MODIFICATION REQUEST message contains the IAB Node Indication IE, the en-gNB shall, if supported, consider the UE as an IAB node.”

  - We propose the following reformulation:

    - “If the SGNB MODIFICATION REQUEST message contains the IAB Node Indication IE, the en-gNB shall, if supported, consider that the request is for an IAB node.”

- In NGAP IAB BL CR (R3-196505):

  - Clause 8.6.1.2: "If the IAB Node Indication IE is included in the INITIAL UE MESSAGE message, the AMF shall treate the UE as an IAB node."

  - We propose the following reformulation:

    - "If the IAB Node Indication IE is included in the INITIAL UE MESSAGE, the AMF shall consider that the message refers to an IAB node."

Proposal: The Rapporteurs of the above IAB BL CRs to reformulate the abovementioned misleading statements and align them with the fact that the IAB-MT functionality is a subset of UE functionality.