3GPP TSG-RAN WG3 Meeting #106                                                            	R3-197516

Reno, USA, 18th - 22th November 2019

Title:	Reply LS on introduction of handover type indication in UE RLF Report

Response to:	R2-1914087/R3-196409

Release:	Rel-16

Work Item:	          NR_SON_MDT-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

Name:	Liu Aijuan

E-mail Address:	Liuaijuan (at) catt (dot) cn

- Overall Description:

RAN3 thanks RAN2 for their reply LS on RACH Optimization and Mobility Robustness Optimization.

From RAN3 point of view, there is other information which should be included in UE RLF Report for Mobility Robustness Optimization as below:

1) Handover Type Indication: This IE is used to indicate whether the failure related HO procedure is intra-system handover or inter-system handover.

RAN3 understands there may be some misunderstanding that the network already has this information considering the HO procedure is controlled by network node. However, the assumption of MRO is that the UE context may be already removed when the eNB/gNB which experiences connection failure receives the UE RLF report via Xn/NG interface. In this case, it is important to let UE report the handover type to enable the network further detecting the root cause of the failure.

2) TAI of the failed cell

There may be no neighbor cell relationship between the node which retrieve UE RLF Report and the node experiencing failure event, in this case, the network node which retrieves UE RLF Report could not know the TAI of the failed cell and thereby could no send the UE RLF REPORT to the corresponding node via NG/S1 interface. So, it is needed to let UE include the TAI of the failed cell in UE RLF Report.

- 2. Actions:

To RAN2:

ACTION: RAN3 respectfully asks RAN2 to take the above into account and update the corresponding spec.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN3 Meeting #107	    24th – 28th February   2020	      Athens, Greece