3GPP TSG-RAN WG3 Meeting #107	R3-200040

24 February-6 March 2020

E-meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0481 | rev | 4 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Baseline CR for introducing Rel-16 NR mobility enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2020-02-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture agreements for Rel-16 NR mobility enhancement impact on F1AP |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RAN3#106: - From R3-197783: Add Conditional Handover Information IE in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-16 NR mobility enhancements are not supported on F1AP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 8.3.4.2, 9.2.2.1, 9.2.2.7, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.4: Submitted to RAN3#107 as BL CR Rev.3: Align criticality between tabular and ASN.1 Rev.2: Capture agreements of RAN3#106 Rev.1: Resubmission to RAN3#106 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, and DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure.

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the SpCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly.

If the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall use the provided value from the gNB-CU.

If the UL Configuration IE in DRB to Be Setup Item IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB.

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB.

If two UL UP TNL Information IEs are included in UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT SETUP RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activing/deactiving CA based PDCP duplication for the DRB.

If DC Based Duplication Configured IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. If DC Based Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activing/deactiving DC based PDCP duplication for this DRB.

If UL PDCP SN length IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. If the Additional RRM Policy Index IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall, if supported, contain the Additional RRM Policy Index IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20]. The gNB-DU shall, if supported, store the received Additional RRM Policy Index in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is available at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT SETUP REQUEST. The gNB-DU may use it for RRM purposes.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested DRBs and SRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to Setup List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

For EN-DC operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15].

For NG-RAN operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the DRB Information IE.

For DC operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE at the gNB acting as secondary node. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the HandoverPreparationInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU of the gNB acting as master node shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. The gNB-CU of the gNB acting as master node shall only initiate the UE Context Setup procedure for handover or secondary node addition when at least one DRB is setup for the UE. If the received CU to DU RRC Information IE does not include source cell group configuration, the gNB-DU shall generate the cell group configuration using full configuration. Otherwise, delta configuration is allowed.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT SETUP REQUEST message shall be ignored. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the Masked IMEISV IE is contained in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be set up with an appropriate cause value for each SCell failed to setup.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT SETUP REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT SETUP RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

If the CellGroupConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

If the Full Configuration IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

If the C-RNTI IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

The UE Context Setup Procedure is not used to configure SRB0.

If the UE CONTEXT SETUP REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE via SRB1.

If the Notification Control IE is included in the DRB to Be Setup List IE and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store the received UL PDU Session Aggregate Maximum Bit Rate and use it when enforcing uplink traffic policing for non-GBR Bearers for the concerned UE as specified in TS 23.501 [21].

The gNB-DU shall store the received gNB-DU UE Aggregate Maximum Bit Rate Uplink and use it for non-GBR Bearers for the concerned UE.

If the UE CONTEXT SETUP REQUEST message contains the QoS Flow Mapping Indication IE, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the UE CONTEXT SETUP REQUEST message contains the New gNB-CU UE F1AP ID IE, the gNB-DU shall, if supported, replace the value received in the gNB-CU UE F1AP ID IE by the value of the New gNB-CU UE F1AP ID and use it for further signalling.

If the RAN UE ID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store and replace any previous information received.

If the Trace Activation IE is included in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, initiate the requested trace function as described in TS 32.422 [29].

If the Conditional Handover Information IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider that the request concerns a conditional handover.

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: UE Context Setup Request procedure: unsuccessful Operation

If the gNB-DU is not able to establish an F1 UE context, or cannot even establish one bearer it shall consider the procedure as failed and reply with the UE CONTEXT SETUP FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT SETUP REQUEST message, it shall reply with the UE CONTEXT SETUP FAILURE message with an appropriate cause value. Further, if the Candidate SpCell List IE is included in the UE CONTEXT SETUP REQUEST message and the gNB-DU is not able to accept the SpCell ID IE, the gNB-DU shall, if supported, include the Potential SpCell List IE in the UE CONTEXT SETUP FAILURE message and the gNB-CU should take this into account for selection of an opportune SpCell. The gNB-DU shall include the cells in the Potential SpCell List IE in a priority order, where the first cell in the list is the one most desired and the last one is the one least desired (e.g., based on load conditions). If the Potential SpCell List IE is present but no Potential SpCell Item IE is present, the gNB-CU should assume that none of the cells in the Candidate SpCell List IE are acceptable for the gNB-DU.

#### 8.3.1.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value. If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT SETUP REQUEST message and is set to the value “delay critical” but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< End of 1st Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

If the SpCell ID IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace any previously received value and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the ServCellIndex IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take this into account for the indicated SpCell. If the SpCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the SCell To Be Setup List IE or SCell To Be Removed List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message and the indicated SCell(s) are already setup, the gNB-DU shall replace any previously received value. If the SCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the DRX Cycle IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall use the provided value from the gNB-CU. If the DRX configuration indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and set to "release", the gNB-DU shall release DRX configuration.

If the SRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4], and replace any previously received value. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB if the value is set to be "true", or delete the RLC entity of secondary path if the value is set to be "false".

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4].

If two UL UP TNL Information IEs are included in UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT MODIFICATION RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activing/deactiving CA based PDCP duplication for the DRB.

If DC Based Duplication Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. Otherwise, the gNB-DU shall regard that DC based PDCP duplication is de-configured for this DRB id the value is set to be "false", and it should stop PDCP duplication activation/deactivation by MAC CE. If DC Based Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activing/deactiving DC based PDCP duplication for this DRB.

For a certain DRB which was allocated with two GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the UE Context Modification procedure, the gNB-DU shall consider that the CA based PDCP duplication for the concerned DRB is de-configured. If such UE Context Modification procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the UL Configuration IE in DRB to Be Setup Item IE or DRB to Be Modified Item IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the ongoing reconfiguration procedure involves changes of the L1/L2 configuration at the gNB-DU signalled to the gNB-CU via the CellGroupConfig IE, the gNB-CU shall include the RRC Reconfiguration Complete Indicator IE in the UE CONTEXT MODIFICATION REQUEST message to inform the gNB-DU that the ongoing reconfiguration procedure, including CellGroupConfig IE, has been successfully or unsuccesfully performed. In the case that the ongoing reconfiguration procedure has failed, the gNB-DU shall continue to use the old UE configuration.

If DL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If UL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If the RLC Failure Indication IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU should consider that the RLC entity indicated by such IE needs to be re-established when the CA-based packet duplication is active, and the gNB-DU may include the Associated SCell List IE in UE CONTEXT MODIFICATION RESPONSE by containing a list of SCell(s) associated with the RLC entity indicated by the RLC Failure Indication IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE. If the UE CONTEXT MODIFICATION REQUEST message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the Transmission Action Indicator IE, the gNB-DU shall stop or restart (if already stopped) data transmission for the UE, according to the value of this IE. It is up to gNB-DU implementation when to stop or restart the UE scheduling.

For EN-DC operation, if the DRB to Be Setup List IE is present in the UE CONTEXT MODIFICATION REQUEST message the gNB-CU shall include the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15]. For NG-RAN operation, the gNB-CU shall include the DRB Information IE in the UE CONTEXT MODIFICATION REQUEST message.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

For DC operation, if the gNB-CU includes the CG-Config IE in the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may initiate low layer parameters coordination taking this information into account.

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT MODIFICATION REQUEST message shall be ignored.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT MODIFICTION REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. If the Additional RRM Policy Index IE is received from an MeNB, the UE CONTEXT MODIFICATION REQUEST message shall , if supported, contain the Additional RRM Policy Index IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20]. The gNB-DU shall, if supported, store the received Additional RRM Policy Index in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is modified at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT MODIFICATION REQUEST. The gNB-DU may use it for RRM purposes.

If the UE CONTEXT MODIFICATION REQUEST message contains the Uplink TxDirectCurrentList Information IE, the gNB-DU may take that into account when selecting L1 configuration.

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT MODIFICATION RESPONSE message, the result for all the requested or modified DRBs and SRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to be Setup List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed to be Modified List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to be Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of successfully modified SRBs with logical channel identities for primary path shall be included in the SRB Modified List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the CellGroupConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall perform RRC Reconfiguration as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP MODOFOCATION REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be set up with an appropriate cause value for each SCell failed to setup.

If the C-RNTI IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT MODIFICATION RESPONSE message and set to “Not-supported”, the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

The UE Context Modify Procedure is not used to configure SRB0.

If the Notification Control IE is included in the DRB to Be Setup List IE or the DRB to Be Modified List IE and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace the received UL PDU Session Aggregate Maximum Bit Rate and use it as specified in TS 23.501 [21].

If the gNB-DU UE Aggregate Maximum Bit Rate Uplink IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall:

-	replace the previously provided gNB-DU UE Aggregate Maximum Bit Rate Uplink with the new received gNB-DU UE Aggregate Maximum Bit Rate Uplink;

-	use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink for non-GBR Bearers for the concerned UE.

The UL PDU Session Aggregate Maximum Bit Rate IE shall be sent if DRB to Be Setup List IE is included and the gNB-CU has not previously sent it. The gNB-DU shall store and use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink.

If the RLC Status IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall assume that RLC has been reestablished at the gNB-DU and may trigger PDCP data recovery.

If the GNB-DU Configuration Query IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall include the CellGroupConfig IE in the DU To CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Bearer Type Change IE is included in DRB to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall either reset the lower layers or generate a new LCID for the affected bearer as specified in TS 37.340[7].

For NE-DC operation, if NeedforGap IE is included in the UE CONTEXT MODIFICATION REQUEST message,the gNB-DU shall generate measurement gap for the SeNB.

If the QoS Flow Mapping Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Lower Layer presence status change IE set to "suspend lower layers" is included in the UE Context Modification message, the gNB-DU shall keep all lower layer configuration for UEs, and not transmit or receive data from UE.

If the Lower Layer presence status change IE set to "resume lower layers" is included in the UE Context Modification message, the gNB-DU shall use the previously stored lower layer configuration for the UE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate a CellGroupConfig IE using full configuration and include it in the UE CONTEXT MODIFICATION RESPONSE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

If the Conditional Handover Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider that the request concerns a conditional handover.

#### 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the UE context can be successfully performed, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT MODIFICATION REQUEST message, it shall reply with the UE CONTEXT MODIFICATION FAILURE message.

#### 8.3.4.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT MODIFICATION REQUESTmessage and is set to the value “delay critical” but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< End of 2nd Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 3rd Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. | - |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >> UL Configuration | O |  | UL Configuraiton 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | - |  |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN ID 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| Trace Activation | O |  | 9.3.1.88 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Conditional Handover Information | O |  | ENUMERATED (CHO-initiation, …) |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |

<<<<<<<<<<<<<<<<<<<< End of 3rd Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 4th Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) |  | - |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | - |  |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >> UL UP TNL Information to be setup List |  | 0..1 |  |  | - |  |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >> RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | YES | reject |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | Yes | ignore |
| Conditional Handover Information | O |  | ENUMERATED (CHO-initiation, …) |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |

<<<<<<<<<<<<<<<<<<<< End of 4th Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 5th Change >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Candidate-SpCell-Item,

Cause,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBID,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DRXConfigurationIndicator,

DUtoCURRCInformation,

EUTRANQoS,

ExecuteDuplication,

FullConfiguration,

GNB-CU-UE-F1AP-ID,

GNB-DU-UE-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-DU-System-Information,

GNB-CU-Name,

GNB-DU-Name,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

LowerLayerPresenceStatusChange,

NotificationControl,

NRCGI,

NRPCI,

UEContextNotRetrievable,

Potential-SpCell-Item,

RAT-FrequencyPriorityInformation,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCContainer-RRCSetupComplete,

RRCReconfigurationCompleteIndicator,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

ServCellIndex,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

ServingCellMO,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

TimeToWait,

TransactionID,

TransmissionActionIndicator,

UE-associatedLogicalF1-ConnectionItem,

DUtoCURRCContainer,

PagingCell-Item,

SItype-List,

UEIdentityIndexValue,

GNB-CU-TNL-Association-Setup-Item,

GNB-CU-TNL-Association-Failed-To-Setup-Item,

GNB-CU-TNL-Association-To-Add-Item,

GNB-CU-TNL-Association-To-Remove-Item,

GNB-CU-TNL-Association-To-Update-Item,

MaskedIMEISV,

PagingDRX,

PagingPriority,

PagingIdentity,

Cells-to-be-Barred-Item,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

NR-CGI-List-For-Restart-Item,

PWS-Failed-NR-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

RLCFailureIndication,

UplinkTxDirectCurrentListInformation,

SULAccessIndication,

Protected-EUTRA-Resources-Item,

GNB-DUConfigurationQuery,

BitRate,

RRC-Version,

GNBDUOverloadInformation,

RRCDeliveryStatusRequest,

NeedforGap,

RRCDeliveryStatus,

ResourceCoordinationTransferInformation,

Dedicated-SIDelivery-NeededUE-Item,

Associated-SCell-Item,

IgnoreResourceCoordinationContainer,

PagingOrigin,

UAC-Assistance-Info,

RANUEID,

GNB-DU-TNL-Association-To-Remove-Item,

NotificationInformation,

TraceActivation,

TraceID,

Neighbour-Cell-Information-Item,

SymbolAllocInSlot,

Slot-Configuration-Item,

NumDLULSymbols,

AdditionalRRMPriorityIndex,

DUCURadioInformationType,

CUDURadioInformationType,

Transport-Layer-Addresses-Info,

ConditionalHandoverInformation

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-ConfirmedUEID,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRB-Activity-Item,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRB-Notify-Item,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ExecuteDuplication,

id-FullConfiguration,

id-gNB-CU-UE-F1AP-ID,

id-gNB-DU-UE-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-CU-Name,

id-gNB-DU-Name,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-new-gNB-CU-UE-F1AP-ID,

id-new-gNB-DU-UE-F1AP-ID,

id-oldgNB-DU-UE-F1AP-ID,

id-PLMNAssistanceInfoForNetShar,

id-Potential-SpCell-Item,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-RedirectedRRCmessage,

id-ResetType,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCContainer-RRCSetupComplete,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetup-Item,

id-SCell-FailedtoSetupMod-List,

id-SCell-FailedtoSetupMod-Item,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-SelectedPLMNID,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-ServingCellMO,

id-SpCell-ID,

id-SpCellULConfigured,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-Item,

id-SRBs-Modified-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-SetupMod-Item,

id-SRBs-SetupMod-List,

id-TimeToWait,

id-TransactionID,

id-TransmissionActionIndicator,

id-UEContextNotRetrievable,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

id-DUtoCURRCContainer,

id-NRCGI,

id-PagingCell-Item,

id-PagingCell-List,

id-PagingDRX,

id-PagingPriority,

id-SItype-List,

id-UEIdentityIndexValue,

id-GNB-CU-TNL-Association-Setup-List,

id-GNB-CU-TNL-Association-Setup-Item,

id-GNB-CU-TNL-Association-Failed-To-Setup-List,

id-GNB-CU-TNL-Association-Failed-To-Setup-Item,

id-GNB-CU-TNL-Association-To-Add-Item,

id-GNB-CU-TNL-Association-To-Add-List,

id-GNB-CU-TNL-Association-To-Remove-Item,

id-GNB-CU-TNL-Association-To-Remove-List,

id-GNB-CU-TNL-Association-To-Update-Item,

id-GNB-CU-TNL-Association-To-Update-List,

id-MaskedIMEISV,

id-PagingIdentity,

id-Cells-to-be-Barred-List,

id-Cells-to-be-Barred-Item,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-To-Be-Broadcast-Item,

id-Cells-Broadcast-Completed-List,

id-Cells-Broadcast-Completed-Item,

id-Broadcast-To-Be-Cancelled-List,

id-Broadcast-To-Be-Cancelled-Item,

id-Cells-Broadcast-Cancelled-List,

id-Cells-Broadcast-Cancelled-Item,

id-NR-CGI-List-For-Restart-List,

id-NR-CGI-List-For-Restart-Item,

id-PWS-Failed-NR-CGI-List,

id-PWS-Failed-NR-CGI-Item,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-Protected-EUTRA-Resources-List,

id-RequestType,

id-ServingPLMN,

id-DRXConfigurationIndicator,

id-RLCFailureIndication,

id-UplinkTxDirectCurrentListInformation,

id-SULAccessIndication,

id-Protected-EUTRA-Resources-Item,

id-GNB-DUConfigurationQuery,

id-GNB-DU-UE-AMBR-UL,

id-GNB-CU-RRC-Version,

id-GNB-DU-RRC-Version,

id-GNBDUOverloadInformation,

id-NeedforGap,

id-RRCDeliveryStatusRequest,

id-RRCDeliveryStatus,

id-Dedicated-SIDelivery-NeededUE-List,

id-Dedicated-SIDelivery-NeededUE-Item,

id-ResourceCoordinationTransferInformation,

id-Associated-SCell-List,

id-Associated-SCell-Item,

id-IgnoreResourceCoordinationContainer,

id-UAC-Assistance-Info,

id-RANUEID,

id-PagingOrigin,

id-GNB-DU-TNL-Association-To-Remove-Item,

id-GNB-DU-TNL-Association-To-Remove-List,

id-NotificationInformation,

id-TraceActivation,

id-TraceID,

id-Neighbour-Cell-Information-List,

id-Neighbour-Cell-Information-Item,

id-Slot-Configuration-Item,

id-SymbolAllocInSlot,

id-NumDLULSymbols,

id-AdditionalRRMPriorityIndex,

id-DUCURadioInformationType,

id-CUDURadioInformationType,

id-LowerLayerPresenceStatusChange,

id-Transport-Layer-Addresses-Info,

id-ConditionalHandoverInformation,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofslots

FROM F1AP-Constants;

-- TEXT OMITTED –

-- **************************************************************

--

##### -- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional 	}|

{ ID id-SpCell-ID								CRITICALITY reject	TYPE NRCGI												PRESENCE mandatory	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE mandatory	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE mandatory}|

{ ID id-Candidate-SpCell-List					CRITICALITY ignore	TYPE Candidate-SpCell-List							PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-SCell-ToBeSetup-List					CRITICALITY ignore	TYPE SCell-ToBeSetup-List								PRESENCE optional	}|

{ ID id-SRBs-ToBeSetup-List						CRITICALITY reject	TYPE SRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-DRBs-ToBeSetup-List						CRITICALITY reject	TYPE DRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY ignore	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV										PRESENCE optional	}|

{ ID id-ServingPLMN								CRITICALITY ignore	TYPE PLMN-Identity										PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE conditional }|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-new-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional }|

{ ID id-RANUEID									CRITICALITY ignore	TYPE RANUEID											PRESENCE optional	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-ConditionalHandoverInformation			CRITICALITY reject	TYPE ConditionalHandoverInformation				PRESENCE optional},

...

}

Candidate-SpCell-List::= SEQUENCE (SIZE(1..maxnoofCandidateSpCells)) OF ProtocolIE-SingleContainer { { Candidate-SpCell-ItemIEs} }

SCell-ToBeSetup-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetup-ItemIEs} }

SRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetup-ItemIEs} }

DRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetup-ItemIEs} }

Candidate-SpCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Candidate-SpCell-Item					CRITICALITY ignore	TYPE Candidate-SpCell-Item						PRESENCE mandatory	},

...

}

SCell-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetup-Item						CRITICALITY ignore	TYPE SCell-ToBeSetup-Item					PRESENCE mandatory	},

...

}

SRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetup-Item		CRITICALITY reject		TYPE SRBs-ToBeSetup-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetup-Item					CRITICALITY reject	TYPE DRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

-- TEXT OMITTED –

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE NRCGI												PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE optional	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE optional	}|

{ ID id-TransmissionActionIndicator				CRITICALITY ignore	TYPE TransmissionActionIndicator					PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-RRCReconfigurationCompleteIndicator		CRITICALITY ignore	TYPE RRCReconfigurationCompleteIndicator			PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY reject	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-SCell-ToBeSetupMod-List					CRITICALITY ignore	TYPE SCell-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-SCell-ToBeRemoved-List					CRITICALITY ignore	TYPE SCell-ToBeRemoved-List 							PRESENCE optional }|

{ ID id-SRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE SRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE DRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeModified-List					CRITICALITY reject	TYPE DRBs-ToBeModified-List							PRESENCE optional	}|

{ ID id-SRBs-ToBeReleased-List					CRITICALITY reject	TYPE SRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeReleased-List					CRITICALITY reject	TYPE DRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-DRXConfigurationIndicator				CRITICALITY ignore	TYPE DRXConfigurationIndicator						PRESENCE optional	}|

{ ID id-RLCFailureIndication					CRITICALITY ignore	TYPE RLCFailureIndication								PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentListInformation	CRITICALITY ignore	TYPE UplinkTxDirectCurrentListInformation		PRESENCE optional	}|

{ ID id-GNB-DUConfigurationQuery				CRITICALITY reject	TYPE GNB-DUConfigurationQuery						PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE optional	}|

{ ID id-ExecuteDuplication						CRITICALITY ignore	TYPE ExecuteDuplication									PRESENCE optional}|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-NeedforGap								CRITICALITY ignore	TYPE NeedforGap											PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-LowerLayerPresenceStatusChange			CRITICALITY ignore	TYPE LowerLayerPresenceStatusChange				PRESENCE optional	}|

{ ID id-ConditionalHandoverInformation			CRITICALITY reject	TYPE ConditionalHandoverInformation				PRESENCE optional},

...

}

SCell-ToBeSetupMod-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetupMod-ItemIEs} }

SCell-ToBeRemoved-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeRemoved-ItemIEs} }

SRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetupMod-ItemIEs} }

DRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetupMod-ItemIEs} }

DRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeModified-ItemIEs} }

SRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeReleased-ItemIEs} }

DRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeReleased-ItemIEs} }

SCell-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetupMod-Item			CRITICALITY ignore	TYPE SCell-ToBeSetupMod-Item			PRESENCE mandatory	},

...

}

SCell-ToBeRemoved-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeRemoved-Item			CRITICALITY ignore	TYPE SCell-ToBeRemoved-Item			PRESENCE mandatory	},

...

}

SRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE SRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE DRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeModified-Item		CRITICALITY reject	TYPE DRBs-ToBeModified-Item			PRESENCE mandatory},

...

}

SRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeReleased-Item	CRITICALITY reject	TYPE SRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

DRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeReleased-Item		CRITICALITY reject	TYPE DRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

-- TEXT OMITTED –

### 9.4.5	Information Element Definitions

-- TEXT OMITTED –

CNUEPagingIdentity ::= CHOICE {

fiveG-S-TMSI			BIT STRING (SIZE(48)),

choice-extension			ProtocolIE-SingleContainer { { CNUEPagingIdentity-ExtIEs } }

}

CNUEPagingIdentity-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ConditionalHandoverInformation ::= ENUMERATED {

cho-initiation,

...

}

CP-TransportLayerAddress ::= CHOICE {

endpoint-IP-address				TransportLayerAddress,

endpoint-IP-address-and-port	Endpoint-IP-address-and-port,

choice-extension			ProtocolIE-SingleContainer { { CP-TransportLayerAddress-ExtIEs } }

}

CP-TransportLayerAddress-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

-- TEXT OMITTED –

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-F1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-gNBDUConfigurationUpdate					ProcedureCode ::= 3

id-gNBCUConfigurationUpdate					ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification					ProcedureCode ::= 7

id-UEContextModificationRequired			ProcedureCode ::= 8

id-UEMobilityCommand						ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-privateMessage							ProcedureCode ::= 14

id-UEInactivityNotification					ProcedureCode ::= 15

id-GNBDUResourceCoordination				ProcedureCode ::= 16

id-SystemInformationDeliveryCommand			ProcedureCode ::= 17

id-Paging									ProcedureCode ::= 18

id-Notify									ProcedureCode ::= 19

id-WriteReplaceWarning						ProcedureCode ::= 20

id-PWSCancel								ProcedureCode ::= 21

id-PWSRestartIndication						ProcedureCode ::= 22

id-PWSFailureIndication						ProcedureCode ::= 23

id-GNBDUStatusIndication 					ProcedureCode ::= 24

id-RRCDeliveryReport		 				ProcedureCode ::= 25

id-F1Removal								ProcedureCode ::= 26

id-NetworkAccessRateReduction				ProcedureCode ::= 27

id-TraceStart								ProcedureCode ::= 28

id-DeactivateTrace							ProcedureCode ::= 29

id-DUCURadioInformationTransfer				ProcedureCode ::= 30

id-CUDURadioInformationTransfer				ProcedureCode ::= 31

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNRminus1					INTEGER ::= 11

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofslots							INTEGER ::= 320

maxnoofTLAs								INTEGER ::=	16

maxnoofGTPTLAs							INTEGER ::=	16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

id-CUtoDURRCInformation								ProtocolIE-ID ::= 9

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 12

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 13

id-DRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 14

id-DRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 15

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 16

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 17

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 18

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 19

id-DRBs-Modified-Item								ProtocolIE-ID ::= 20

id-DRBs-Modified-List								ProtocolIE-ID ::= 21

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 22

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 23

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 24

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 25

id-DRBs-Setup-Item									ProtocolIE-ID ::= 26

id-DRBs-Setup-List									ProtocolIE-ID ::= 27

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 28

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 29

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 30

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 31

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 32

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 33

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 34

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 35

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 36

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 37

id-DRXCycle											ProtocolIE-ID ::= 38

id-DUtoCURRCInformation								ProtocolIE-ID ::= 39

id-gNB-CU-UE-F1AP-ID								ProtocolIE-ID ::= 40

id-gNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 41

id-gNB-DU-ID										ProtocolIE-ID ::= 42

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 43

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 44

id-gNB-DU-Name										ProtocolIE-ID ::= 45

id-NRCellID											ProtocolIE-ID ::= 46

id-oldgNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 47

id-ResetType										ProtocolIE-ID ::= 48

id-ResourceCoordinationTransferContainer			ProtocolIE-ID ::= 49

id-RRCContainer										ProtocolIE-ID ::= 50

id-SCell-ToBeRemoved-Item							ProtocolIE-ID ::= 51

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 52

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 53

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 54

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 55

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 56

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 57

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 58

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 59

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 60

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 61

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 62

id-SpCell-ID										ProtocolIE-ID ::= 63

id-SRBID											ProtocolIE-ID ::= 64

id-SRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 65

id-SRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 66

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 67

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 68

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 69

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 70

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 71

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 72

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 73

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 74

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 75

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 76

id-TimeToWait										ProtocolIE-ID ::= 77

id-TransactionID									ProtocolIE-ID ::= 78

id-TransmissionActionIndicator						ProtocolIE-ID ::= 79

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 80

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 81

id-gNB-CU-Name										ProtocolIE-ID ::= 82

id-SCell-FailedtoSetup-List							ProtocolIE-ID ::= 83

id-SCell-FailedtoSetup-Item							ProtocolIE-ID ::= 84

id-SCell-FailedtoSetupMod-List						ProtocolIE-ID ::= 85

id-SCell-FailedtoSetupMod-Item						ProtocolIE-ID ::= 86

id-RRCReconfigurationCompleteIndicator 				ProtocolIE-ID ::= 87

id-Cells-Status-Item								ProtocolIE-ID ::= 88

id-Cells-Status-List								ProtocolIE-ID ::= 89

id-Candidate-SpCell-List							ProtocolIE-ID ::= 90

id-Candidate-SpCell-Item							ProtocolIE-ID ::= 91

id-Potential-SpCell-List							ProtocolIE-ID ::= 92

id-Potential-SpCell-Item							ProtocolIE-ID ::= 93

id-FullConfiguration								ProtocolIE-ID ::= 94

id-C-RNTI											ProtocolIE-ID ::= 95

id-SpCellULConfigured								ProtocolIE-ID ::= 96

id-InactivityMonitoringRequest						ProtocolIE-ID ::= 97

id-InactivityMonitoringResponse						ProtocolIE-ID ::= 98

id-DRB-Activity-Item								ProtocolIE-ID ::= 99

id-DRB-Activity-List								ProtocolIE-ID ::= 100

id-EUTRA-NR-CellResourceCoordinationReq-Container 		ProtocolIE-ID ::= 101

id-EUTRA-NR-CellResourceCoordinationReqAck-Container 	ProtocolIE-ID ::= 102

id-Protected-EUTRA-Resources-List					ProtocolIE-ID ::= 105

id-RequestType 										ProtocolIE-ID ::= 106

id-ServCellIndex									ProtocolIE-ID ::= 107

id-RAT-FrequencyPriorityInformation					ProtocolIE-ID ::= 108

id-ExecuteDuplication								ProtocolIE-ID ::= 109

id-NRCGI											ProtocolIE-ID ::= 111

id-PagingCell-Item									ProtocolIE-ID ::= 112

id-PagingCell-List									ProtocolIE-ID ::= 113

id-PagingDRX										ProtocolIE-ID ::= 114

id-PagingPriority 									ProtocolIE-ID ::= 115

id-SItype-List										ProtocolIE-ID ::= 116

id-UEIdentityIndexValue								ProtocolIE-ID ::= 117

id-gNB-CUSystemInformation							ProtocolIE-ID ::= 118

id-HandoverPreparationInformation					ProtocolIE-ID ::= 119

id-GNB-CU-TNL-Association-To-Add-Item				ProtocolIE-ID ::= 120

id-GNB-CU-TNL-Association-To-Add-List				ProtocolIE-ID ::= 121

id-GNB-CU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 122

id-GNB-CU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 123

id-GNB-CU-TNL-Association-To-Update-Item			ProtocolIE-ID ::= 124

id-GNB-CU-TNL-Association-To-Update-List			ProtocolIE-ID ::= 125

id-MaskedIMEISV										ProtocolIE-ID ::= 126

id-PagingIdentity									ProtocolIE-ID ::= 127

id-DUtoCURRCContainer								ProtocolIE-ID ::= 128

id-Cells-to-be-Barred-List							ProtocolIE-ID ::= 129

id-Cells-to-be-Barred-Item							ProtocolIE-ID ::= 130

id-TAISliceSupportList								ProtocolIE-ID ::= 131

id-GNB-CU-TNL-Association-Setup-List				ProtocolIE-ID ::= 132

id-GNB-CU-TNL-Association-Setup-Item				ProtocolIE-ID ::= 133

id-GNB-CU-TNL-Association-Failed-To-Setup-List		ProtocolIE-ID ::= 134

id-GNB-CU-TNL-Association-Failed-To-Setup-Item		ProtocolIE-ID ::= 135

id-DRB-Notify-Item									ProtocolIE-ID ::= 136

id-DRB-Notify-List									ProtocolIE-ID ::= 137

id-NotficationControl								ProtocolIE-ID ::= 138

id-RANAC											ProtocolIE-ID ::= 139

id-PWSSystemInformation								ProtocolIE-ID ::= 140

id-RepetitionPeriod									ProtocolIE-ID ::= 141

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 142

id-Cells-To-Be-Broadcast-List						ProtocolIE-ID ::= 144

id-Cells-To-Be-Broadcast-Item						ProtocolIE-ID ::= 145

id-Cells-Broadcast-Completed-List 					ProtocolIE-ID ::= 146

id-Cells-Broadcast-Completed-Item 					ProtocolIE-ID ::= 147

id-Broadcast-To-Be-Cancelled-List 					ProtocolIE-ID ::= 148

id-Broadcast-To-Be-Cancelled-Item 					ProtocolIE-ID ::= 149

id-Cells-Broadcast-Cancelled-List 					ProtocolIE-ID ::= 150

id-Cells-Broadcast-Cancelled-Item 					ProtocolIE-ID ::= 151

id-NR-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 152

id-NR-CGI-List-For-Restart-Item 					ProtocolIE-ID ::= 153

id-PWS-Failed-NR-CGI-List 							ProtocolIE-ID ::= 154

id-PWS-Failed-NR-CGI-Item 							ProtocolIE-ID ::= 155

id-ConfirmedUEID									ProtocolIE-ID ::= 156

id-Cancel-all-Warning-Messages-Indicator			ProtocolIE-ID ::= 157

id-GNB-DU-UE-AMBR-UL								ProtocolIE-ID ::= 158

id-DRXConfigurationIndicator						ProtocolIE-ID ::= 159

id-RLC-Status										ProtocolIE-ID ::= 160

id-DLPDCPSNLength									ProtocolIE-ID ::= 161

id-GNB-DUConfigurationQuery							ProtocolIE-ID ::= 162

id-MeasurementTimingConfiguration					ProtocolIE-ID ::= 163

id-DRB-Information									ProtocolIE-ID ::= 164

id-ServingPLMN										ProtocolIE-ID ::= 165

id-Protected-EUTRA-Resources-Item					ProtocolIE-ID ::= 168

id-GNB-CU-RRC-Version								ProtocolIE-ID ::= 170

id-GNB-DU-RRC-Version								ProtocolIE-ID ::= 171

id-GNBDUOverloadInformation							ProtocolIE-ID ::= 172

id-CellGroupConfig									ProtocolIE-ID ::= 173

id-RLCFailureIndication								ProtocolIE-ID ::= 174

id-UplinkTxDirectCurrentListInformation				ProtocolIE-ID ::= 175

id-DC-Based-Duplication-Configured					ProtocolIE-ID ::= 176

id-DC-Based-Duplication-Activation					ProtocolIE-ID ::= 177

id-SULAccessIndication								ProtocolIE-ID ::= 178

id-AvailablePLMNList								ProtocolIE-ID ::= 179

id-PDUSessionID										ProtocolIE-ID ::= 180

id-ULPDUSessionAggregateMaximumBitRate				ProtocolIE-ID ::= 181

id-ServingCellMO									ProtocolIE-ID ::= 182

id-QoSFlowMappingIndication							ProtocolIE-ID ::= 183

id-RRCDeliveryStatusRequest							ProtocolIE-ID ::= 184

id-RRCDeliveryStatus								ProtocolIE-ID ::= 185

id-BearerTypeChange									ProtocolIE-ID ::= 186

id-RLCMode											ProtocolIE-ID ::= 187

id-Duplication-Activation							ProtocolIE-ID ::= 188

id-Dedicated-SIDelivery-NeededUE-List				ProtocolIE-ID ::= 189

id-Dedicated-SIDelivery-NeededUE-Item				ProtocolIE-ID ::= 190

id-DRX-LongCycleStartOffset							ProtocolIE-ID ::= 191

id-ULPDCPSNLength									ProtocolIE-ID ::= 192

id-SelectedBandCombinationIndex						ProtocolIE-ID ::= 193

id-SelectedFeatureSetEntryIndex						ProtocolIE-ID ::= 194

id-ResourceCoordinationTransferInformation			ProtocolIE-ID ::= 195

id-ExtendedServedPLMNs-List							ProtocolIE-ID ::= 196

id-ExtendedAvailablePLMN-List						ProtocolIE-ID ::= 197

id-Associated-SCell-List							ProtocolIE-ID ::= 198

id-latest-RRC-Version-Enhanced						ProtocolIE-ID ::= 199

id-Associated-SCell-Item							ProtocolIE-ID ::= 200

id-Cell-Direction									ProtocolIE-ID ::= 201

id-SRBs-Setup-List									ProtocolIE-ID ::= 202

id-SRBs-Setup-Item									ProtocolIE-ID ::= 203

id-SRBs-SetupMod-List								ProtocolIE-ID ::= 204

id-SRBs-SetupMod-Item								ProtocolIE-ID ::= 205

id-SRBs-Modified-List								ProtocolIE-ID ::= 206

id-SRBs-Modified-Item								ProtocolIE-ID ::= 207

id-Ph-InfoSCG										ProtocolIE-ID ::= 208

id-RequestedBandCombinationIndex					ProtocolIE-ID ::= 209

id-RequestedFeatureSetEntryIndex					ProtocolIE-ID ::= 210

id-RequestedP-MaxFR2								ProtocolIE-ID ::= 211

id-DRX-Config										ProtocolIE-ID ::= 212

id-IgnoreResourceCoordinationContainer				ProtocolIE-ID ::= 213

id-UEAssistanceInformation							ProtocolIE-ID ::= 214

id-NeedforGap										ProtocolIE-ID ::= 215

id-PagingOrigin										ProtocolIE-ID ::= 216

id-new-gNB-CU-UE-F1AP-ID							ProtocolIE-ID ::= 217

id-RedirectedRRCmessage								ProtocolIE-ID ::= 218

id-new-gNB-DU-UE-F1AP-ID							ProtocolIE-ID ::= 219

id-NotificationInformation							ProtocolIE-ID ::= 220

id-PLMNAssistanceInfoForNetShar						ProtocolIE-ID ::= 221

id-UEContextNotRetrievable							ProtocolIE-ID ::= 222

id-BPLMN-ID-Info-List								ProtocolIE-ID ::= 223

id-SelectedPLMNID									ProtocolIE-ID ::= 224

id-UAC-Assistance-Info								ProtocolIE-ID ::= 225

id-RANUEID											ProtocolIE-ID ::= 226

id-GNB-DU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 227

id-GNB-DU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 228

id-TNLAssociationTransportLayerAddressgNBDU			ProtocolIE-ID ::= 229

id-portNumber										ProtocolIE-ID ::= 230

id-AdditionalSIBMessageList							ProtocolIE-ID ::= 231

id-Cell-Type										ProtocolIE-ID ::= 232

id-IgnorePRACHConfiguration							ProtocolIE-ID ::= 233

id-CG-Config										ProtocolIE-ID ::= 234

id-PDCCH-BlindDetectionSCG							ProtocolIE-ID ::= 235

id-Requested-PDCCH-BlindDetectionSCG				ProtocolIE-ID ::= 236

id-Ph-InfoMCG										ProtocolIE-ID ::= 237

id-MeasGapSharingConfig								ProtocolIE-ID ::= 238

id-systemInformationAreaID							ProtocolIE-ID ::= 239

id-areaScope										ProtocolIE-ID ::= 240

id-RRCContainer-RRCSetupComplete					ProtocolIE-ID ::= 241

id-TraceActivation									ProtocolIE-ID ::= 242

id-TraceID											ProtocolIE-ID ::= 243

id-Neighbour-Cell-Information-List					ProtocolIE-ID ::= 244

id-Slot-Configuration-Item							ProtocolIE-ID ::= 245

id-SymbolAllocInSlot								ProtocolIE-ID ::= 246

id-NumDLULSymbols									ProtocolIE-ID ::= 247

id-AdditionalRRMPriorityIndex						ProtocolIE-ID ::= 248

id-DUCURadioInformationType							ProtocolIE-ID ::= 249

id-CUDURadioInformationType 						ProtocolIE-ID ::= 250

id-AggressorgNBSetID								ProtocolIE-ID ::= 251

id-VictimgNBSetID									ProtocolIE-ID ::= 252

id-LowerLayerPresenceStatusChange					ProtocolIE-ID ::= 253

id-Transport-Layer-Addresses-Info					ProtocolIE-ID ::= 254

id-Neighbour-Cell-Information-Item					ProtocolIE-ID ::= 255

id-ConditionalHandoverInformation					ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>