# 3GPP TSG-RAN WG3 #107-e																		R3-200185

# E-Meeting, 24 February-6 March 2020

Title: 	(TP to BL CR#0156 “Introduction of NB-IoT related NG-AP procedures” for TS 38.413) 5G-S-TMSI in Error Indication

Source: 	Huawei

Agenda item:	14.3.2

Document for:	other

- Introduction

Due to the limitation of the Quota, in this contribution, we discuss three leftover issue “5G-S-TMSI in Error Indication " in NGAP BL CR#0156, and provide corresponding Text Proposal.

- Discussion

In RAN3#105bis meeting, it was agreed to introduce 5G-S-TMSI in Error Indication Procedure to align with EPS, and it was commented and one Editor’s Note were added:

Editor’s Note: It should be confirm that the benefit of the presence of 5G-S-TMSI for 5GC is not equivalent at EPS.

For EPS, the scenario to introduce the S-TMSI in S1AP: ERROR INDICATION message was for the case in which the MME restart and lost the stored S-TMSI, as NB-IoT inter UE QoS related procedures (Retrieve UE Information and UE Information Transfer procedures) are two non-UE-associated S1AP class 2 procedures, linked by using S-TMSI, once the eNB triggers Retrieve UE Information procedure towards the MME, the MME is not able to provide S-TMSI information to the eNB to indicate the failure.

From our understanding, similar case may happen in 5GS, therefore it is proposed to remove the Editor’s Note.

Proposal: Remove the Editor’s Note about the 5G-S-TMSI in NGAP: ERROR INDICATION message

- Text Proposal to the NGAP BL CR#0156

----Start of the First Change----

9.2.6.13	ERROR INDICATION

This message is sent by both the NG-RAN node and the AMF to indicate that some error has been detected in the node.

Direction: NG-RAN node  AMF and AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | O |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | O |  | 9.3.3.2 |  | YES | ignore |
| Cause | O |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | ignore |

----End of the Change----

- Reference

- BL CR#0156 “Introduction of NB-IoT related NG-AP procedures” for TS 38.413