3GPP TSG-RAN3 Meeting #107-e	R3-200333

24 February - 6 March 2020 E-Meeting

Agenda item:	15.3.1.4

Source:	ZTE

Title:	TP for NR_Mob_enh BL CR for TS 38.423 Correction of Conditional HO Cancel

Document for:	Agreement

1	Introduction

In the agreed R3-197614 at RAN3#106, some confusions regarding (Conditional) HO Cancel procedure were still left.

For the “ Conditional Handover Cancel”, the following abnormal condition has been captured  in section 8.2.B.4:

“If the Candidate Cells To Be Cancelled List IE included in CONDITIONAL HANDOVER CANCEL are not associated to the same UE-associated signaling connection used for creating before, the source NG-RAN node shall ignore the message.”

However, the counterpart description of abnormal condition is still missing for “Handover Cancel” case in section 8.2.3.4, so should be specified.

Note: it makes big difference between “ignore the message” and “ignore the xxx IE”.

The new IE “Candidate Cells To Be Cancelled List” is optional, so may not be included in the “CONDITIONAL HANDOVER CANCEL” message. For  “HANDOVER CANCEL”, in case of above IE absence, it can still be used to cancel immediate HO, hence there is no issue there.

For  “CONDITIONAL HANDOVER CANCEL”, since it can only be used for conditional HO, hence in case of above IE absence, it means that all candidate cells associated with the UE signaling connection are supposed to be canceled.  However, per following baseline statement, it seems that regardless of whether invalid Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message or not, the source NG-RAN node shall both consider that the target NG-RAN node is about to remove all candidate cells. This is conflicting with the abnormal condition  in section 8.2.B.4 where it specifies the source NG-RAN node shall ignore the message.

“At the reception of the CONDITIONAL HANDOVER CANCEL message the source NG-RAN node shall consider that the target NG-RAN node is about to remove any reference to, and release any resources previously reserved to the concerned UE context for the concerned UE-associated signalling.”

In addition, there are some more cleanups, e.g. NR CGI —>Target Cell Global ID.

2	Text proposal

This TP is based on the BL CR endorsed after RAN3 #106 [R3-197814].

*** First change, skipped text not changed ***

8.2.3	Handover Cancel

8.2.3.1	General

The Handover Cancel procedure is used to enable a source NG-RAN node to cancel an ongoing handover preparation or an already prepared handover.

The procedure uses UE-associated signalling.

8.2.3.2	Successful Operation

Figure 8.2.3.2-1: Handover Cancel, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER CANCEL message to the target NG-RAN node. The source NG-RAN node shall indicate the reason for cancelling the handover by means of an appropriate cause value.

If the Candidate Cells To Be Cancelled List IE is included in HANDOVER CANCEL message, the target NG-RAN node shall consider that the source NG-RAN node is cancelling only the handover associated to the cells identified by the included NG-RAN CGI.

If the Candidate Cells To Be Cancelled List IE is included in HANDOVER CANCEL message, the target NG-RAN node shall consider that the source NG-RAN node is cancelling the prepared conditional handovers in the candidate cells associated to the same UE-associated signaling identified by the Source NG-RAN node UE XnAP ID and Target NG-RAN node UE XnAP ID, if included.

8.2.3.3	Unsuccessful Operation

Not applicable.

8.2.3.4	Abnormal Conditions

If the HANDOVER CANCEL message refers to a context that does not exist, the target NG-RAN node shall ignore the message.

If the Candidate Cells To Be Cancelled List IE is included in HANDOVER CANCEL and the handover is not associated to a conditional handover, the target NG-RAN node shall ignore the Candidate Cells To Be Cancelled List IE, but shall still cancel the target cell associated to immediate handover if any.

If the Candidate Cells To Be Cancelled List IE included in HANDOVER CANCEL are not associated to the same UE-associated signaling connection used for creating before, the target NG-RAN node shall ignore the message.

*** Next change, skipped text not changed ***

### 8.2.B	Conditional Handover Cancel

#### 8.2.B.1	General

The Conditional Handover Cancel procedure is used to enable a target NG-RAN node to cancel an already prepared conditional handover.

The procedure uses UE-associated signalling.

#### 8.2.B.2	Successful Operation

Figure 8.2.B.2-1: Conditional Handover Cancel, successful operation

The target NG-RAN node initiates the procedure by sending the CONDITIONAL HANDOVER CANCEL message to the source NG-RAN node. The target NG-RAN node shall indicate the reason for cancelling the conditional handover by means of an appropriate cause value.

At the reception of the CONDITIONAL HANDOVER CANCEL message without including the Candidate Cells To Be Cancelled List IE, the source NG-RAN node shall consider that the target NG-RAN node is about to remove any reference to, and release any resources previously reserved to the concerned UE context for the concerned UE-associated signalling.

If the Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that only the resources reserved for the cells identified by the included NG-RAN CGI are about to be released.

If the Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that only the resources reserved for the candidate cells associated to the same UE-associated signaling identified by the Source NG-RAN node UE XnAP ID and Target NG-RAN node UE XnAP ID are about to be released.

#### 8.2.B.3	Unsuccessful Operation

Not applicable.

#### 8.2.B.4	Abnormal Conditions

If the CONDITIONAL HANDOVER CANCEL message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

If the Candidate Cells To Be Cancelled List IE included in CONDITIONAL HANDOVER CANCEL are not associated to the same UE-associated signaling connection used for creating before, the source NG-RAN node shall ignore the message.

*** Next change, skipped text not changed ***

9.1.1.6	HANDOVER CANCEL

This message is sent by the source NG-RAN node to the target NG-RAN node to cancel an ongoing handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional handover. Value is FFS. |

*** Next change, skipped text not changed ***

9.1.1.BB	CONDITIONAL HANDOVER CANCEL

This message is sent by the target NG-RAN node to the source NG-RAN node to cancel an already prepared conditional handover.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional handover. Value is FFS. |

*** Remaining text not changed ***