3GPP TSG-RAN3 Meeting #107-e	R3-200367

24 February-6 March 2020, E-Meeting

Title: 	(TP for SON BL CR for TS 38.413): Clarification to Inter-System unnecessary HO

Source: 	Huawei

Agenda item:	10.2.1.2

Document Type:	other

# 1. Introduction

MRO was discussed during RAN3#105b and a set of BL CRs were endorsed. In this paper we propose some further refinement for MRO for Inter-system unnecessary HO.

# 2. Discussion

For the PCI defined in NR, it refers to the NR Physical Cell Identifier of the cell, not the Primary Cell Identifier. The corresponding description of the IE need change.

For the Early IRAT HO in inter-system HO report, the enumerated values were changed to “true and false”. The corresponding ASN.1 need change.

# 3. Conclusion

Based on the discussion in this paper, the semantics description of PCI and the ASN.1 for Early IRAT HO should be revised.

# 4. Annex – TP

Start of the first change

#### 9.3.3.Y5	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |
| >Too early Inter-system HO |  |  |  |  |
| >>Source cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |
| >>Failure cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |
| >>UE RLF Report Container | FFS |  | OCTET STRING |  |
| >Inter-system Unnecessary HO |  |  |  |  |
| >>Source cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |
| >>Target cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |
| >>Early IRAT HO | M |  | ENUMERATED (true, false, ...) | Is set to “true” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “false” |
| >>Candidate Cell List |  | 1 |  |  |
| >>>Candidate CGI List |  | 1 |  |  |
| >>>>Candidate CGI Item |  | 1,, .. <maxnoofCandidateCells> |  |  |
| >>>>>Candidate Cell ID | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI. |
| >>>Candidate PCI List |  | 0..1 |  |  |
| >>>>Candidate PCI Item |  | 1 .. <maxnoofCandidateCells> |  |  |
| >>>>>Candidate PCI | M |  | INTEGER (0..1007, …) | This IE includes the NR Physical Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NR CGI could not be derived. |
| >>>>>Candidate NR ARFCN | M |  | INTEGER (0.. maxNARFCN) | RF Reference Frequency as defined in TS 38.104 [r2], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. |
| maxNARFCN | Maximum value of NR carrier frequency, defined in TS 38.331 [18] |

Start of next change

### 9.4.5	Information Element Definitions

<partially omitted>

IntersystemUnnecessaryHO ::= SEQUENCE {

sourcecellID			EUTRA-CGI,

failurecellID			NGRAN-CGI,

earlyIRATHO				ENUMERATED{true, false},

candidateCellList		CandidateCellList,

iE-Extensions			ProtocolExtensionContainer { { IntersystemUnnecessaryHO-ExtIEs} }			OPTIONAL

}

IntersystemUnnecessaryHO-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

End of the last change