3GPP TSG-RAN WG3 Meeting #107-e	R3-200386

E-meeting, 24 February – 6 March, 2020

Agenda item:	10.2.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for SON BL CR for TS 38.473) Introduction of RACH Assistance Information

Document for:	Text Proposal

# 1	Introduction

This TP includes updates to the SON BL CR for TS 38.473, by introducing support for transfer of RACH assistance information on the F1 interface according to discussion in [1].

The TP is based on TS 38.473 v16.0.0, and proposed included in the BL CR to TS 38.473 [2].

# References

[1]		R3-200385, RACH Assistance Information Exchange, Nokia, Nokia Shanghai Bell

[2]	R3-200014, BL CR to TS 38.473

# Annex	- TP for TR 38.473

<<< start of changes >>>

#### 9.2.8.1	DU-CU RADIO INFORMATION TRANSFER

This message is sent by a gNB-DU to a gNB-CU, to convey radio-related information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CHOICE DU-CU Radio Information Type | M |  |  |  | YES | ignore |
| >RIM |  |  |  |  |  |  |
| >>DU-CU RIM Information | M |  | 9.3.1.91 |  | - | - |
| >RACH Assistance Information Transfer |  |  |  |  |  |  |
| >>DU-CU RACH Failure Rate Information | O |  | 9.2.X.Z1 |  | YES | ignore |

<< next change >>>

#### 9.2.8.2	CU-DU RADIO INFORMATION TRANSFER

This message is sent by a gNB-CU to a gNB-DU, to convey radio-related information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CHOICE CU-DU Radio Information Type | M |  |  |  | YES | ignore |
| >RIM |  |  |  |  |  |  |
| >>CU-DU RIM Information | M |  | 9.3.1.92 |  | - | - |
| > RACH Assistance Information Transfer |  |  |  |  |  |  |
| >> UE RACH Report Information | O |  | 9.2.X.Z3 |  | YES | ignore |
| >> CU-DU RACH Failure Rate Information | O |  | 9.2.X.Z2 |  | YES | ignore |

<< next change >>>

#### 9.2.X.Z1	DU-CU RACH Failure Rate Information

The DU-CU RACH Failure Rate Information IE contains RACH failure rate calculated at the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  |  |  |
| >>Cell ID | M |  | 9.3.1.12 |  |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>RACH Failure Rate on NUL Carrier | M |  | INTEGER (0..100) | Percentage of failed attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>RACH Failure Rate on SUL Carrier | O |  | INTEGER (0..100) | Percentage of failed attempted RACH accesses, for the SSB, on the SUL carrier |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBIds | Maximum no. of SSB items. Value is 64 |
| maxnoofSSBIds-1 | Maximum no. of SSB items minus 1. Value is 63 |

<< next change >>>

#### 9.2.X.Z2	CU-DU RACH Failure Rate Information

The CU-DU RACH Failure Rate Information IE provides RACH failure rate calculated at cells that are neighbor to cells served by the the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report |  | 1 |  | Reporting cell list |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Reporting Cell ID | M |  | NR CGI 9.3.1.12 | The cell for which the RACH failure rate is calculated. |  |  |
| >>Neighbour Cell List |  | 1 |  | List of cells in the gNB-DU that are neighbours to the reporting cell |  |  |
| >>>Neighbour Cell Item |  | 1.. <maxCellingNBDU> |  |  |  |  |
| >>>Neighbour Cell ID | M |  | NR CGI 9.3.1.12 |  |  |  |
| >>SSB List |  |  |  | List of SSBs in the cell for which the RACH failure rate is calculated |  |  |
| >>>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>>RACH Failure Rate on NUL Carrier | M |  | INTEGER (0..100) | Percentage of failed attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>>RACH Failure Rate on SUL Carrier | O |  | INTEGER (0..100) | Percentage of failed attempted RACH accesses, for the SSB, on the SUL carrier |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBIds | Maximum no. of SSB items. Value is 64 |
| maxnoofSSBIds-1 | Maximum no. of SSB items minus 1. Value is 63 |

<< next change >>>

#### 9.2.X.Z3	UE RACH Report Information

The UE RACH Report Information IE contains information about failed RACH attempts at cells served by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Destination Cell ID | M |  | NR CGI 9.3.1.12 | The cell in which the RACH attempts were performed |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>Number of Failed RACH Attempts on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of reported failed RACH attempts on the NUL carrier, for the SSB, extracted from multiple UE RACH Reports |  |  |
| >>>Number of Failed RACH Attempts on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of reported failed RACH attempts on the SUL carrier, for the SSB, extracted from multiple UE RACH Reports |  |  |

<<< end of changes >>>