3GPP TSG-RAN WG3 Meeting #107-e	R3-200387

E-meeting, 24 February – 6 March, 2020

Agenda item:	10.2.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for SON BL CR for TS 38.423) Introduction of RACH Assistance Information

Document for:	Text Proposal

# 1	Introduction

This TP includes updates to the SON BL CR for TS 38.423, by introducing support for transfer of UE RACH report information and RACH assistance information on the Xn interface according to discussion in [1].

The TP is based on BL CR to TS 38.423 [2].

# References

[1]	R3-200385, RACH Assistance Information Exchange, Nokia, Nokia Shanghai Bell

[2]	R3-200012, BL CR to TS 38.423

# Annex	- TP for TS 38.423

<<< start of changes >>>

#### 9.1.3.x	FAILURE INDICATION

This message is sent by the NG-RAN node2 either to indicate an RRC re-establishment attempt or a reception of an RLF Report from a UE that suffered a connection failure at NG-RAN node1, or to provide information relative to RACH access attempts on NG-RAN node1 or on NG-RAN node2.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.X |  | YES | ignore |
| CHOICE Initiating condition | M |  |  |  |  |  |
| > RRC Reestab |  |  |  |  |  |  |
| >>Failure cell PCI | C- ifUERLFReportContainerAbsent |  | 9.2.2.10 | Physical Cell Identifier | YES | ignore |
| >>Re-establishment cell CGI | C- ifUERLFReportContainerAbsent |  | NG-RAN CGI 9.2.2.x |  | YES | ignore |
| >>C-RNTI | C- ifUERLFReportContainerAbsent |  | BIT STRING (SIZE (16)) | C-RNTI contained in the RRC Re-establishment Request message (TS 38.331) [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | YES | ignore |
| >>ShortMAC-I | C- ifUERLFReportContainerAbsent |  | BIT STRING (SIZE (16)) | ShortMAC-I contained in the RRC Re-establishment Request message (TS 38.331) [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | YES | ignore |
| >>UE RLF Report Container | O |  | OCTET STRING | RLF -Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10] or TS 36.331 [14]) FFS | YES | ignore |
| > RRC Setup |  |  |  |  |  |  |
| >>UE RLF Report Container | O |  | OCTET STRING | RLF -Report-r16 IE contained in the UEInformationResponse message (TS 38.331) FFS | YES | ignore |
| > RACH Assistance Information Transfer |  |  |  |  |  |  |
| >> UE RACH Report Information | O |  | 9.2.2.K |  | YES | ignore |
| >> RACH Failure Rate Information | O |  | 9.2.2.KK |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifUERLFReportContainerAbsent | This IE shall be present if the UE RLF Report Container IE is absent |

<<< next change >>>

#### 9.2.2.K	UE RACH Report Information

The UE RACH Report Information IE contains information about failed RACH attempts at cells served by NG-RAN node1 received in UE RACH reports uploaded at NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Destination Cell ID | M |  | NR CGI 9.2.2.7 | The cell in which the RACH attempts were performed, served by NG-RAN node1 |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index | M |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>Number of Failed RACH Attempts on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of reported failed RACH attempts, for the SSB, on the NUL carrier, extracted from multiple UE RACH Reports |  |  |
| >>>Number of Failed RACH Attempts on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of reported failed RACH attempts, for the SSB, on the SUL carrier, extracted from multiple UE RACH Reports |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBIDs | Maximum no. SSB IDs within an NR cell. Value is 64. |
| maxnoofSSBIds-1 | Maximum no. of SSB IDs within an NR cell minus 1. Value is 63. |

<<< next change >>>

#### 9.2.2.KK	RACH Failure Rate Information

The RACH Failure Rate Information IE contains RACH failure rate calculated at cells served by the NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Reporting Cell ID | M |  | NR CGI 9.2.2.7 | The cell for which the RACH failure rate is calculated. |  |  |
| >>Cell in gNB-DU | M |  | NR CGI 9.2.2.7 |  |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>RACH Failure Rate on NUL Carrier | M |  | INTEGER (0..100) | Percentage of failed RACH access attempts on the NUL carrier. |  |  |
| >>>RACH Failure Rate on SUL Carrier | O |  | INTEGER (0..100) | Percentage of failed RACH access attempts on the SUL carrier. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBIDs | Maximum no. SSB IDs within an NR cell. Value is 64. |
| maxnoofSSBIds-1 | Maximum no. of SSB IDs within an NR cell minus 1. Value is 63. |

<<< end of changes >>>