3GPP TSG-RAN WG3 Meeting #107e                                   R3-200405

Electronic Meeting, 24 Feb – 6 March, 2020

Agenda item:	10.2.1.6

Source: 		Qualcomm Incorporated

Title: 	(TP for SON BL CR for 38.413) Mobility history from UE

Document for:	Approval

# 1	Introduction

This is TP to implement UE reported mobility history information in NGAP, per R3-200404.

# 4	TP to SON BL CR to 38.413

Start Change

#### 9.3.1.95	UE History Information

This IE contains information about cells that a UE has been visited in RRC_IDLE/RRC_INACTIVE state and served by in active state prior to the target cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Last Visited Cell Item |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list. |
| >Last Visited Cell Information | M |  | 9.3.1.96 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum no. of cells in the UE history information. Value is 16. |