3GPP TSG-RAN WG3 Meeting #107e                                   R3-200406

Electronic Meeting, 24 Feb – 6 March, 2020

Agenda item:	10.2.1.6

Source: 		Qualcomm Incorporated

Title: 	(TP for SON BL CR for 38.423) Mobility history from UE

Document for:	Approval

# 1	Introduction

This is TP to implement UE reported mobility history information in XnAP, per R3-200404.

# 4	TP to SON BL CR to 38.423

Start Change

#### 9.2.3.64	UE History Information

The UE History Information IE contains information about cells that a UE has been visited in RRC_IDLE/RRC_INACTIVE or served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [12].

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 38.413 [5].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list |
| >Last Visited Cell Information | M |  | 9.2.3.65 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |