3GPP TSG-RAN WG3 Meeting #107-e                                                          R3-200415

Electronic Meeting, February 24th – March 6th, 2020

Agenda item:       13.2.1.4

Source: 		Qualcomm Incorporated

Title:	 (TP for NR_IAB BL CR to TS 38.473) F1AP - PHY layer configuration

Document for:	Agreement

# 1	Introduction

This paper discusses the configuration of PHY layer parameters for IAB based on [1] and [2].

RAN1 divides the IAB parameters into the following sub-feature groups:

- SSB-based discovery and measurement: Tx Configuration (F1AP)

- SSB-based discovery and measurement: Rx Configuration (RRC)

- PRACH (RRC)

- Resource multiplexing (F1AP)

- Resource multiplexing (MAC CE)

- Resource multiplexing (RRC)

- Case-1 timing (MAC CE)

The configuration protocols in brackets have been provided by RAN1 as guidance to RAN2 and RAN3, where:

- F1AP was considered for configuration of DU-related parameters.

- RRC was considered for configuration of MT-related parameters by the CU.

- MAC CE was considered for local signaling between DU and MT.

RAN3 would only have to be concerned with the configuration of the DU. For that purpose, RAN3 should assess RAN1’s guidance on the configuration protocols proposed.

RAN1 assumed that all DU configuration would be performed via F1AP.  However, DU configuration is generally split between OAM and CU. Some of the OAM-based DU configurations are reported by the DU to the CU via F1AP, and a subset of those can subsequently be changed by the CU. RAN3 therefore has to decide for these PHY layer parameters, which of these configuration options is used. These decisions should consider the following factors:

- The configuration of PHY-layer parameters for IAB should align with existing configuration procedures used, e.g., for access-related PHY-layer parameters, if possible.

- Since the CU is responsible for resource-, route- and topology management it needs to be able to configure or reconfigure PHY-layer parameters affected by these management tasks.

The following discussion outlines the background of the relevant feature groups, provides a recommendation of the configuration procedure, and allow companies to add their respective views.

# 2	Discussion

## 2.1	IAB-specific SSB-based discovery and measurement

Background

IAB-specific SSB-based discovery and measurement is used by network-integrated IAB-nodes to discover and perform measurements on neighbour IAB-nodes and IAB-donor DUs. This feature is essential for the support of topology adaptation. It uses a separate SSB resource, which is not used for access by UEs or for initial access by IAB-MTs.

IAB-DUs and IAB-donor DUs need to be configured with the IAB-specific SSB Tx resource. Network-integrated IAB-node MTs further need to obtain a measurement configuration to listen to the IAB-specific SSB transmissions by other IAB-nodes and IAB-donor DUs.

IAB-specific SSB-based discovery and measurement is subject to half-duplexing constraint, i.e., the IAB-DU cannot transmit SSB at the same time as the collocated IAB-MT listens to SSB transmissions from neighbour IAB-nodes and IAB-donor DUs. This means that IAB-specific SSB transmission and SSB measurements use different time-domain resources. Further, IAB-specific SSB transmissions by IAB-DUs and IAB-donor DUs within overlapping coverage area must have relative time offset so that they can mutually discover and measure each other.

Considerations for IAB-specific SSB transmission:

- It would be beneficial to align the IAB-specific SSB Tx configuration (STC) with the SSB Tx configuration used for UE access, which is presently performed via OAM.

- The IAB-specific SSB Tx configuration should be known to the CU to be taken into consideration for resource planning and for the measurement configuration of IAB-MTs, which is conducted via RRC.

- When new IAB-nodes are deployed or operating IAB-nodes are converged to IAB-donor DUs, the SSB Tx resource configuration may be subject to change with topology-wide repercussions. It would be beneficial if the CU could perform reconfiguration of the STC for that reason.

Proposal 1:IAB-specific SSB Tx configuration is optionally provided by OAM, reported to the CU via non-UE-associated F1AP messages, and it can be (re-)configured by the CU via non-UE-associated F1AP messages.

### 2.2	IAB-specific PRACH configuration

Background

The IAB-specific PRACH is used by IAB-MTs for random access to IAB-DUs and IAB-donor DUs. It is used by network-integrated IAB-nodes as well as network-integrating IAB-nodes.

IAB-nodes requires a special PRACH configuration to account for the larger RRT associated with the larger mutual distance (up to multiple ISDs) compared to UE-access (typically ½ ISD).

For integrated IAB-nodes, IAB-specific PRACH resource configuration can further account for half-duplexing constraints between IAB-nodes’ Tx and Rx operation.

Considerations:

- It is beneficial to align IAB-specific PRACH configuration with UE-access-specific PRACH configuration, which is contained in SIB1 and reported to the CU as part of the SIB1 container.

- The CU needs to know the resource used for IAB-specific PRACH to take it into considerations for resource management.

- For initial IAB-MT access, IAB-specific PRACH configuration may not be necessary and UE-specific PRACH resources could be used (dependent on deployment scenario).

Recommendation:

IAB-specific PRACH configuration is part of SIB1, configured via OAM and reported to the CU via F1AP as part of the SIB1 container. The CU may reconfigure IAB-specific PRACH configuration via RRC.

This means that there is no impact on RAN3.

### 2.4	IAB resource configuration

Background

Resource multiplexing in the time domain is applied between backhaul links as well as between access and backhaul links to account for half-duplexing constraint. Resource multiplexing can occur with symbol-level granularity. The CU is responsible for the topology-wide management of the semi-static time-domain resource configuration. Further, local DUMT signaling via PDCCH (DCI format 2-5) allows the IAB-DU or IAB-donor DU to release semi-statically configured resources to the child-node DU.

The semi-static resource configuration by the CU to the DU is cell-specific, and it contains a sequence of time resources. Each time resource contains one or multiple symbols together with an H/S/NA and a D/U/F predicate. H/S/NA refers to:

- H: Hard (i.e. DU can use this time resource unconditionally)

- S: Soft (i.e. DU can use this time resource if indicated by the parent node via DUMT PDCCH signaling)

- NA: Not available (i.e. DU cannot use this time resource)

and D/U/F refers to:

- D: Time resource must be used for DL

- U: Time resource must be used for UL

- F: Time resource can be used for either DL or UL

The D/U/F pattern of the DU’s time resource configuration needs to be aligned with the DU’s transmission of TDD-UL-DL-common in SIB1 and the CU’s configuration of TDD-UL-DL-dedicated and Slot-Format-Indicator for each UE and MT connected to this DU.

Figure 1: Example for semi-static time resource configuration on DU

The DU can use an available time resource for the scheduling of traffic across all UEs and MTs connected. In case the DU does not need an available time resource for traffic exchange with a child node, it can inform the child node via DUMT signaling that the child-node DU can use this time resource instead. For this purpose, however, the time resource must have already been configured on the child-node DU as “Soft”. Also, the granularity of the resource release to the child-node is based on the child-node’s rather than the parent node’s resource configuration, and it is only indicated in the DUMT signaling message via the resource’s D/U/F predicate.

Example: DU signals to child-node MT: “Your collocated DU can use its soft-configured ‘D’ resource in this slot”.

To make all of this happen, the DU must be informed by the CU about each child-node-DU’s time resource configuration. The IAB-node must further be configured on how to relate the content of the PDCCH signaling message received by the IAB-MT to the soft-resources owned by the collocated IAB-DU.

Since time resources are cell-specific, the IAB-node might be able to simultaneously exchange data on child and parent links at different frequencies. Such resource multiplexing depends on the separation of the frequencies, the relative direction of transmission for traffic exchange on child and parent links, and implementation-related factors.

Example: An IAB-node with mmWave backhaul and sub-6 access could simultaneously operate on backhaul and access links. If access and backhaul links are adjacent, it might still be possible to simultaneously transmit or receive on child and parent link.

The CU should be able to take these multiplexing capabilities into consideration for the IAB resource management.

Considerations for resource configuration:

- Resource management is owned by the CU.

- Resource configuration is subject to constraints of half-duplexing between child and parent links, which is implementation-specific and must be known to the CU.

- The resource configuration is confined by TDD-UL-DL-common configured via OAM in SIB1 and reported to the CU. It is further confined by TDD-UL-DL-dedicated and Slot-Format-Indication configured by the CU for each UE/MT connected to the DU.

- The IAB-node’s multiplexing capabilities between serving cells configured on the IAB-DU and serving cells configured on the collocated IAB-MT are implementation-specific and therefore only known to the IAB-node. They must be reported to the CU. Since the IAB-MT may support a large number of frequency bands, the report should only include multiplexing capabilities for those cells that are actually configured on the IAB-MT.

Proposal 2:Resource configuration on the IAB-donor DU and IAB-DU is performed by the CU using non-UE-associated F1AP signalling, and considering constraints due to TDD-UL-DL-common contained in SIB1, which is reported by the DU to the CU.

Proposal 3: Information on the resource configuration of child-node DUs is provided to the DU by the CU using UE-associated F1AP signalling.

Proposal 4: The IAB-node’s multiplexing capabilities between serving cells configured on the IAB-DU and serving cells configured on the collocated IAB-MT are reported by the IAB-DU to the CU via non-UE-associated F1AP signalling.

All other configurations are performed via RRC.

### 2.5	Stage-3 enhancements to F1AP

The TP below includes all enhancements to TS 38.473 to support PHY layer configuration for IAB.

Proposal 5: Include the TP into IAB BL CR for TS 38.473.

# 3	Conclusion

This paper discussed the configuration of PHY layer parameters for IAB.The following proposals have been made:

Proposal 1: IAB-specific SSB Tx configuration is optionally provided by OAM, reported to the CU via non-UE-associated F1AP messages, and it can be (re-)configured by the CU via non-UE-associated F1AP messages.

Proposal 2: Resource configuration on the IAB-donor DU and IAB-DU is performed by the CU using non-UE-associated F1AP signalling, and considering constraints due to TDD-UL-DL-common contained in SIB1, which is reported by the DU to the CU.

Proposal 3: Information on the resource configuration of child-node DUs is provided to the DU by the CU using UE-associated F1AP signalling.

Proposal 4: The IAB-node’s multiplexing capabilities between serving cells configured on the IAB-DU and serving cells configured on the collocated IAB-MT are reported by the IAB-DU to the CU via non-UE-associated F1AP signalling.

Proposal 5: Include the TP into TS 38.473.

# 4	References

[1]	R1-1913674: Updated consolidated parameter list for Rel-16 NR, 3GPP TSG RAN WG1 Meeting #99, Reno, USA, November 18th -22nd, 2019

[2]	R1-1913675: LS on updated Rel-16 LTE and NR parameter lists, 3GPP TSG RAN WG1 Meeting #99, Reno, USA, November 18th -22nd, 2019

# 5	TP for NR_IAB BL CR to TS 38.473

START CHANGE

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

## 8.2	Interface Management procedures

### 8.2.1	Reset

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

The gNB-DU initiates the procedure by sending a F1 SETUP REQUEST message including the appropriate data to the gNB-CU. The gNB-CU responds with a F1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the F1 interface is operational and other F1 messages may be exchanged.

If the F1 SETUP REQUEST message contains the gNB-DU Name IE, the gNB-CU may use this IE as a human readable name of the gNB-DU.

If the F1 SETUP REQUEST message contains the gNB-DU Served Cells List IE, the gNB-CU shall take into account as specified in TS 38.401 [4].

For NG-RAN, the gNB-DU shall include the gNB-DU System Information IE and the TAI Slice Support List IE.

The gNB-CU may include the Cells to be Activated List IE in the F1 SETUP RESPONSE message. The Cells to be Activated List IE includes a list of cells that the gNB-CU requests the gNB-DU to activate. The gNB-DU shall activate the cells included in the Cells to be Activated List IE and reconfigure the physical cell identity for cells for which the NR PCI IE is included.

If Cells to be Activated List Item IE is contained in the F1 SETUP REQUEST message, and the indicated cell includes the IAB Info gNB CU IE, the gNB-DU shall apply the gNB-DU resource configuration included in this IE for this cell.

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the F1 SETUP RESPONSE message.

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

For NG-RAN, the gNB-CU may include Available PLMN List IE, and optionally also Extended Available PLMN List IE, if the available PLMN(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, gNB-DU shall take this into account and only broadcast the PLMN(s) included in the received Available PLMN list(s).

The Latest RRC Version Enhanced IE shall be included in the F1 SETUP REQUEST message and in the F1 SETUP RESPONSE message.

If in F1 SETUP REQUEST message, the Cell Direction IE is present, the gNB-CU should use it to understand whether the cell is for UL or DL only. If in F1 SETUP REQUEST message, the Cell Direction IE is omitted in the Served Cell Information IE it shall be interpreted as that the Cell Direction is Bi-directional.

#### 8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: F1 Setup procedure: Unsuccessful Operation

If the gNB-CU cannot accept the setup, it should respond with a F1 SETUP FAILURE and appropriate cause value.

If the F1 SETUP FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the F1 setup towards the same gNB-CU.

#### 8.2.3.4	Abnormal Conditions

Not applicable.

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If gNB-DU ID IE is contained in the GNB-DU CONFIGURATION UPDATE message for a newly established SCTP association, the gNB-CU will associate this association with the related gNB-DU.

If Served Cells To Add Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall add cell information according to the information in the Served Cell Information IE. For NG-RAN, the gNB-DU shall include the gNB-DU System Information IE.

If Served Cells To Modify Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall modify information of cell indicated by Old NR CGI IE according to the information in the Served Cell Information IE and overwrite the served cell information for the affected served cell. Further, if the gNB-DU System Information IE is present the gNB-CU shall store and replace any previous information received.

If Served Cells To Delete Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall delete information of cell indicated by Old NR CGI IE.

If Cells Status Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall update the information about the cells, as described in TS 38.401 [4].

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for cells for which the NR PCI IE is included.

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, and the indicated cell includes the IAB Info gNB CU IE, the gNB-DU shall apply the gNB-DU resource configuration included in this IE for this cell.

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Deactivated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall deactivate all the cells with NR CGI listed in the IE.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the GNB-DU CONFIGURATION UPDATE message. The gNB-CU shall store and replace any previously provided RAN Area Code IE by the received RAN Area Code IE.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If in GNB-DU CONFIGURATION UPDATE message, the Cell Direction IE is present, the gNB-CU should use it to understand whether the cell is for UL or DL only. If in GNB-DU CONFIGURATION UPDATE message, the Cell Direction IE is omitted in the Served Cell Information IE it shall be interpreted as that the Cell Direction is Bi-directional.

If the GNB-DU CONFIGURATION UPDATE message includes gNB-DU TNL Association To Remove List IE, and the Endpoint-IP-address-and-port IE for both TNL endpoints of the TNL association(s) are included in the gNB-DU TNL Association To Remove List IE, the gNB-CU shall, if supported, consider that the TNL association(s) indicated by both received TNL endpoints will be removed by the gNB-DU. If the Endpoint-IP-address IE for one or both of the TNL endpoints is included in the gNB-DU TNL Association To Remove List IE in GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider that the TNL association(s) indicated by the received endpoint IP address(es) will be removed by the gNB-DU.

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-DU Configuration Update procedure: Unsuccessful Operation

If the gNB-CU cannot accept the update, it shall respond with a GNB-DU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-DU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the GNB-DU CONFIGURATION UPDATE message towards the same gNB-CU.

#### 8.2.4.4	Abnormal Conditions

Not applicable.

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the indicated cell includes the IAB Info gNB CU IE, the gNB-DU shall apply the gNB-DU resource configuration included in this IE for this cell.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

If the gNB-CU TNL Association To Add List IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, use it to establish the TNL association(s) with the gNB-CU. The gNB-DU shall report to the gNB-CU, in the gNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the gNB-CU as follows:

-	A list of TNL address(es) with which the gNB-DU successfully established the TNL association shall be included in the gNB-CU TNL Association Setup List IE;

-	A list of TNL address(es) with which the gNB-DU failed to establish the TNL association shall be included in the gNB-CU TNL Association Failed To Setup List IE.

If the gNB-CU TNL Association To Remove List IE is contained in the gNB-CU CONFIGURATION UPDATE message, and the gNB-CU TNL Association To Remove List IE only includes the Endpoint-IP-Address IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by the received gNB-CU Transport Layer Address towards the gNB-CU.

If the GNB-CU CONFIGURATION UPDATE message includes gNB-CU TNL Association To Remove List IE, and the Endpoint-IP-address-and-port IE for both TNL endpoints of the TNL association(s) is included in the gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by both received TNL endpoints towards the gNB-CU. If the Endpoint-IP-address IE for one or both of the TNL endpoints is included in the gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by the received endpoint IP address(es).

If the gNB-CU TNL Association To Update List IE is contained in the gNB-CU CONFIGURATION UPDATE message the gNB-DU shall, if supported, overwrite the previously stored information for the related TNL Association(s).

If the TNL Association usage IE is included in the gNB-CU TNL Association To Add List IE or the gNB-CU TNL Association To Update List IE, the gNB-DU node shall, if supported, use it as described in TS 38.472 [22].

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

If Protected E-UTRA Resources List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall protect the corresponding resource of the cells indicated by E-UTRA Cells List IE for spectrum sharing between E-UTRA and NR.

If the GNB-CU CONFIGURATION UPDATE message contains the Protected E-UTRA Resource Indication IE, the receiving gNB-DU should forward it to lower layers and use it for cell-level resource coordination. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE when expressing its desired resource allocation during gNB-DU Resource Coordination procedure. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same gNB-DU.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU shall consider that the indicated cells are out-of-service as defined in TS 38.401 [4].

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU Configuration Update: Unsuccessful Operation

If the gNB-DU cannot accept the update, it shall respond with a GNB-CU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the GNB-CU CONFIGURATION UPDATE message towards the same gNB-DU.

#### 8.2.5.4	Abnormal Conditions

Not applicable.

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

## 8.3	UE Context Management procedures

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, and DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure.

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the SpCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly.

If the Child-node IAB Info gNB CU IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU may use the resource information contained in this IE to indicate to the child the availability of one of the child node gNB-DU’s soft resources for the frequency of this cell.

If the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the SCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall use the provided value from the gNB-CU.

If the UL Configuration IE in DRB to Be Setup Item IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB.

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB.

If two UL UP TNL Information IEs are included in UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT SETUP RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activing/deactiving CA based PDCP duplication for the DRB.

If DC Based Duplication Configured IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. If DC Based Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activing/deactiving DC based PDCP duplication for this DRB.

If UL PDCP SN length IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is available at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT SETUP REQUEST. The gNB-DU may use it for RRM purposes.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested DRBs and SRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to Setup List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

For EN-DC operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15].

For NG-RAN operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the DRB Information IE.

For DC operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE at the gNB acting as secondary node. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the HandoverPreparationInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU of the gNB acting as master node shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. The gNB-CU of the gNB acting as master node shall only initiate the UE Context Setup procedure for handover or secondary node addition when at least one DRB is setup for the UE. If the received CU to DU RRC Information IE does not include source cell group configuration, the gNB-DU shall generate the cell group configuration using full configuration. Otherwise, delta configuration is allowed.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT SETUP REQUEST message shall be ignored. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the Masked IMEISV IE is contained in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be established with an appropriate cause value for each SCell failed to setup.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT SETUP REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT SETUP RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

If the CellGroupConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

If the Full Configuration IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

If the C-RNTI IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

The UE Context Setup Procedure is not used to configure SRB0.

If the UE CONTEXT SETUP REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE via SRB1.

If the Notification Control IE is included in the DRB to Be Setup List IE and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store the received UL PDU Session Aggregate Maximum Bit Rate and use it when enforcing uplink traffic policing for non-GBR Bearers for the concerned UE as specified in TS 23.501 [21].

The gNB-DU shall store the received gNB-DU UE Aggregate Maximum Bit Rate Uplink and use it for non-GBR Bearers for the concerned UE.

If the UE CONTEXT SETUP REQUEST message contains the QoS Flow Mapping Indication IE, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the UE CONTEXT SETUP REQUEST message contains the New gNB-CU UE F1AP ID IE, the gNB-DU shall, if supported, replace the value received in the gNB-CU UE F1AP ID IE by the value of the New gNB-CU UE F1AP ID and use it for further signalling.

If the RAN UE ID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store and replace any previous information received.

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

If the SpCell ID IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace any previously received value and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the ServCellIndex IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take this into account for the indicated SpCell. If the SpCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the Child-node IAB Info gNB CU IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall replace any information previously received with the information contained in this IE.

If the SCell To Be Setup List IE or SCell To Be Removed List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message and the indicated SCell(s) are already setup, the gNB-DU shall replace any previously received value. If the SCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the DRX Cycle IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall use the provided value from the gNB-CU. If the DRX configuration indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and set to "release", the gNB-DU shall release DRX configuration.

If the SRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4], and replace any previously received value. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB if the value is set to be "true", or delete the RLC entity of secondary path if the value is set to be "false".

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4].

If two UL UP TNL Information IEs are included in UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT MODIFICATION RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activing/deactiving CA based PDCP duplication for the DRB.

If DC Based Duplication Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. Otherwise, the gNB-DU shall regard that DC based PDCP duplication is de-configured for this DRB id the value is set to be "false", and it should stop PDCP duplication activation/deactivation by MAC CE. If DC Based Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activing/deactiving DC based PDCP duplication for this DRB.

For a certain DRB which was allocated with two GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the UE Context Modification procedure, the gNB-DU shall consider that the CA based PDCP duplication for the concerned DRB is de-configured. If such UE Context Modification procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the UL Configuration IE in DRB to Be Setup Item IE or DRB to Be Modified Item IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the ongoing reconfiguration procedure involves changes of the L1/L2 configuration at the gNB-DU signalled to the gNB-CU via the CellGroupConfig IE, the gNB-CU shall include the RRC Reconfiguration Complete Indicator IE in the UE CONTEXT MODIFICATION REQUEST message to inform the gNB-DU that the ongoing reconfiguration procedure, including CellGroupConfig IE, has been successfully or unsuccesfully performed. In the case that the ongoing reconfiguration procedure has failed, the gNB-DU shall continue to use the old UE configuration.

If DL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If UL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If the RLC Failure Indication IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU should consider that the RLC entity indicated by such IE needs to be re-established when the CA-based packet duplication is active, and the gNB-DU may include the Associated SCell List IE in UE CONTEXT MODIFICATION RESPONSE by containing a list of SCell(s) associated with the RLC entity indicated by the RLC Failure Indication IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE. If the UE CONTEXT MODIFICATION REQUEST message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the Transmission Action Indicator IE, the gNB-DU shall stop or restart (if already stopped) data transmission for the UE, according to the value of this IE. It is up to gNB-DU implementation when to stop or restart the UE scheduling.

For EN-DC operation, if the DRB to Be Setup List IE is present in the UE CONTEXT MODIFICATION REQUEST message the gNB-CU shall include the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15]. For NG-RAN operation, the gNB-CU shall include the DRB Information IE in the UE CONTEXT MODIFICATION REQUEST message.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT MODIFICATION REQUEST message shall be ignored.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT MODIFICTION REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is modified at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT MODIFICATION REQUEST. The gNB-DU may use it for RRM purposes.

If the UE CONTEXT MODIFICATION REQUEST message contains the Uplink TxDirectCurrentList Information IE, the gNB-DU may take that into account when selecting L1 configuration.

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT MODIFICATION RESPONSE message, the result for all the requested or modified DRBs and SRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to be Setup List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed to be Modified List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to be Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of successfully modified SRBs with logical channel identities for primary path shall be included in the SRB Modified List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the CellGroupConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall perform RRC Reconfiguration as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP MODOFOCATION REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be established with an appropriate cause value for each SCell failed to setup.

If the C-RNTI IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT MODIFICATION RESPONSE message and set to “Not-supported”, the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

The UE Context Modify Procedure is not used to configure SRB0.

If the Notification Control IE is included in the DRB to Be Setup List IE or the DRB to Be Modified List IE and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace the received UL PDU Session Aggregate Maximum Bit Rate and use it as specified in TS 23.501 [21].

If the gNB-DU UE Aggregate Maximum Bit Rate Uplink IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall:

-	replace the previously provided gNB-DU UE Aggregate Maximum Bit Rate Uplink with the new received gNB-DU UE Aggregate Maximum Bit Rate Uplink;

-	use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink for non-GBR Bearers for the concerned UE.

The UL PDU Session Aggregate Maximum Bit Rate IE shall be sent if DRB to Be Setup List IE is included and the gNB-CU has not previously sent it. The gNB-DU shall store and use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink.

If the RLC Status IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall assume that RLC has been reestablished at the gNB-DU and may trigger PDCP data recovery.

If the GNB-DU Configuration Query IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall include the CellGroupConfig IE in the DU To CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Bearer Type Change IE is included in DRB to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall either reset the lower layers or generate a new LCID for the affected bearer as specified in TS 37.340[7].

For NE-DC operation, if NeedforGap IE is included in the UE CONTEXT MODIFICATION REQUEST message,the gNB-DU shall generate measurement gap for the SeNB.

If the QoS Flow Mapping Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate a CellGroupConfig IE using full configuration and include it in the UE CONTEXT MODIFICATION RESPONSE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

# 9	Elements for F1AP Communication

## 9.1			General

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

## 9.2			Message Functional Definition and Content

#### 9.2.1	Interface Management messages

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### 9.2.1.4	F1 SETUP REQUEST

This message is sent by the gNB-DU to transfer information associated to an F1-C interface instance.

NOTE:	If a TNL association is shared among several F1-C interface instances, several F1 Setup procedures are issued via the same TNL association after that TNL association has become operational.

Editor’s Note: IAB-related information is included in Serving Cell Information IE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-DU ID | M |  | 9.3.1.9 |  | YES | reject |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) |  | YES | ignore |
| gNB-DU Served Cells List |  | 0.. 1 |  | List of cells configured in the gNB-DU | YES | reject |
| >gNB-DU Served Cells Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| gNB-DU RRC version | M |  | RRC version 9.3.1.70 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information associated to an F1-C interface instance.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Cells to be Activated List |  | 0.. 1 |  |  | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info gNB CU | O |  | 9.3.x.y+1 | IAB configuration by CU | YES | ignore |
| gNB-CU RRC version | M |  | RRC version 9.3.1.70 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instance, this message may transfer updated information associated to several F1-C interface instances.

Editor’s NOTE: IAB-related information is included in Served Cell Information IE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Served Cells To Add List |  | 0..1 |  | Complete list of added cells served by the gNB-DU | YES | reject |
| >Served Cells To Add Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the gNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Delete List |  | 0..1 |  | Complete list of deleted cells served by the gNB-DU | YES | reject |
| >Served Cells To Delete Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| Cells Status List |  | 0..1 |  | Complete list of active cells | YES | reject |
| > Cells Status Item |  | 0 .. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Service Status | M |  | 9.3.1.68 |  | - |  |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| > Dedicated SI Delivery Needed UE Item |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-DU ID | O |  | 9.3.1.9 |  | YES | reject |
| gNB-DU TNL Association To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-DU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | reject |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | - | - |
| >>TNL Association Transport Layer Address gNB-CU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### 9.2.1.8	GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU to a gNB-DU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Editor’s NOTE: The CU can use this message to modify already activated cells with new resource configuration.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0.. 1 |  | List of cells to be activated | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info gNB CU | O |  | 9.3.x.y+1 | IAB configuration by CU | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Cells to be Deactivated List |  | 0.. 1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Editor’s NOTE: The CU can use this message to modify already activated cells with new resource configuration.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info gNB CU | O |  | 9.3.x.y+1 | IAB configuration by CU | YES | ignore |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >> E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>> E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Global Identifier as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum nunmerbs of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE shall be considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| Child node IAB Info gNB CU | O |  | 9.3.x.y+2 | Resource configuration of child node gNB-DU’s cell with same frequency as SpCell. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| >>Child node IAB Info gNB CU | O |  | 9.3.x.y+2 | Resource configuration of child node gNB-DU’s cell with same frequency as SCell. | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. | - |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >> UL Configuration | O |  | UL Configuraiton 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | - |  |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN ID 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 | The value of the gNB-CU UE F1AP ID IE shall be replaced by the value of the New gNB-CU UE F1AP ID and used for further signalling. | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Editor’s NOTE: The CU can use this message to modify already activated cells with new child-node resource information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE shall be considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the RRCConnectionReconfiguration message as defined in TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| Child node IAB Info gNB CU | O |  | 9.3.x.y+2 | Resource configuration of child node gNB-DU’s cell with same frequency as SpCell. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| >>Child node IAB Info gNB CU | O |  | 9.3.x.y+2 | Resource configuration of child node gNB-DU’s cell with same frequency as SCell. | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) |  | - |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | - |  |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >> UL UP TNL Information to be setup List |  | 0..1 |  |  | - |  |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >> RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | YES | reject |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

## 9.3		Information Element Definitions

#### 9.3.1	Radio Network Layer Related IEs

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per TA. | YES | ignore |
| CHOICE NR-Mode-Info | M |  |  |  | - |  |
| >FDD |  |  |  |  | - |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >TDD |  |  |  |  | - |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>> NR FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>> Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8]. | - |  |
| RANAC | O |  | RAN Area Code 9.3.1.57 |  | YES | ignore |
| Extended Served PLMNs List |  | 0..1 |  | This is included if more than 6 Served PLMNs is to be signalled. | YES | ignore |
| >Extended Served PLMNs Item |  | 1 ..<maxnoofExtendedBPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >>TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per TA. | - |  |
| Cell Direction | O |  | 9.3.1.78 |  | YES | ignore |
| Cell Type | O |  | 9.3.1.87 |  | YES | ignore |
| Broadcast PLMN Identity Info List |  | 0..<maxnoofBPLMNsNR-1> |  | This IE corresponds to the PLMN-IdentityInfoList IE in SIB1 as specified in TS 38.331 [8]. The PLMN Identities and associated information contained in this IE shall be provided in the same order as broadcast in SIB1. | YES | ignore |
| >PLMN Identity List | M |  | Available PLMN List 9.3.1.65 |  | - |  |
| >Extended PLMN Identity List | O |  | Extended Available PLMN List 9.3.1.76 |  | - |  |
| >5GS-TAC | O |  | OCTET STRING (3) |  | - |  |
| >NR Cell Identity | M |  | BIT STRING (36) |  | - |  |
| >RANAC | O |  | RAN Area Code 9.3.1.57 |  | - |  |
| IAB Info gNB DU | O |  | 9.3.x.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR-1 | Maximum no. of PLMN Ids.broadcast in an NR cell minus 1. Value is 11. |

#### 9.3.x.y	IAB Info gNB DU

This IE contains cell-specific IAB-related information sent by the gNB DU to the gNB CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MultiplexInfo | M |  | 9.3.x.y+3 | Contains information on multiplexing with cells configured for collocated IAB-MT |
| IAB STC Info | M |  | 9.3.x.y+4 | Contains information on STC configuration on gNB DU |

#### 9.3.x.y+1	IAB Info gNB CU

This IE contains cell-specificIAB-related information sent by the gNB CU to the gNB DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB STC Info | O |  | 9.3.x.y+4 | Contains STC (re-)configuration of gNB DU |
| gNB-DU Resource Configuration | O |  | 9.3.x.y+5 | Contains resource configuration for gNB-DU |

#### 9.3.x.y+2	Child Node IAB Info gNB CU

This IE contains IAB-related information of the child-node’s gNB DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU Resource Configuration | O |  | 9.3.x.y+5 | Resource configuration of child node gNB-DU’s cell with same frequency. |

#### 9.3.x.y+3	Multiplex Info

This IE contains information on the multiplexing capabilities with cells configured on collocated IAB-MT

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB-MT Cell List |  | 0..1 |  |  |
| >IAB-MT Cell Item |  | 1 .. <maxnoofServingCells> | Same as maxNrofServingCells in Ts 38.331 [zz] | All cells configured for collocated IAB-MT |
| >> NR Cell Identity | M |  | BIT STRING (SIZE(36)) | Cell identify of serving cell configured for collocated IAB-MT |
| >>DU_RX/MT_RX | M |  | ENUMERATED{supported, not supported ...} |  |
| >>DU_TX/MT_TX | M |  | ENUMERATED{supported, not supported ...} |  |
| >>DU_TX/MT_RX | M |  | ENUMERATED{supported, not supported ...} |  |
| >>DU_RX/MT_TX | M |  | ENUMERATED{supported, not supported ...} |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofServingCells | Maximum no. of serving cells for IAB-MT. Value is 32. |

#### 9.3.x.y+4	IAB-STC Info

This IE contains cell SSB Tx Configuration information of the IAB-DU used by neighbor IAB-node MTs for discovery and measurements of this IAB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB-STC-Info List |  | 0,,1 |  |  |
| >IAB-STC-Info Item |  | 1 ..<maxnoofIABstcInfo> |  |  |
| >>SSB FreqInfo | M |  | NR Frequency Info 9.3.1.17 | SSB central frequency |
| >>SSB Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | Transmission bandwidth IE including SSB subcarrier spacing |
| >>SSB Transmission Periodicity | M |  | ENUMERATED(sf5, sf10, sf20, sf40, sf80, sf160, sf320, sf640, ,,,) | SSB transmission periodicity |
| >>SSB Transmission Timing Offset | M |  | INTEGER(0..127, …) | SSB transmission timing offset in number of half frames |
| >>SSB Transmission Bitmap | M |  | OCTET STRING | SetupRelease (SSB-ToMeasure) IE as defined in TS 38.331 [zz]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofIABstcInfo | Maximum no. of stc configurations. Value is 4. |

#### 9.3.x.y+5	gNB-DU Resource Configuration

This IE contains cell configuration information of a cell in the gNB-DU.

The IE gNB-DU RESOURCE CONFIGATION is used to configure the TDD resource parameters of DL/UL/Flexible and Hard/Soft/Notavailable for each serving cell (see TS 38.213 [13], clause 11.1.1).

Editor’s NOTE: This IE has been derived from TS38.331 IEs on TDD-UL-DL-dedicated and SlotFormatCombinatoinsPerCell. Note that TS38.331 specifies SlotFormatCombinationsPerCell over a set of slots and per cell, while TDD-UL-DL-dedicated is defined over the transmission periodicity of TDD-UL-DL-common and for all cells. For DU configuration, both were combined to a configuration per cell over transmission periodicity.

Editor’s Note: In TS 38.213 Table 11.1.1, SlotFormatIndex = 255 indicates that TDD-UL-DL-dedicated should be used. Since the DU configuration provides explicit choice between either SlotFormatConfiguration or TDD-UL-DL-Dedicated for each slot, the range of the SlotFormatCombinationID has been reduced to 254.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| IAB-SubcarrierSpacing | M |  | ENUMERATED{kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1} | Subcarrier spacing used as reference for the TDD slot configuration | YES | reject |
| IAB-DL-UL-TransmissionPeriodicity | M |  | ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10} | DL-UL-TransmissionPeriodicity | YES | reject |
| IAB-SlotConfigList |  | 0..1 |  |  |  |  |
| >IAB-SlotConfigItem |  | 1..<maxnoofSlots> |  | Same as maxNrofSlots in TS 38.331 [zz] | - |  |
| >>DownlinkSlotConfig | O |  | ENUMERATED(HARD, SOFT, NOTAVAILABLE) |  | - |  |
| >>UplinkSlotConfig | O |  | ENUMERATED(HARD, SOFT, NOTAVAILABLE) |  | - |  |
| >>FlexibleSlotConfig | O |  | ENUMERATED(HARD, SOFT, NOTAVAILABLE) |  | - |  |
| >>CHOICE IAB-SlotConfig |  |  |  |  | - |  |
| >>>Explicit |  |  |  |  | - |  |
| >>>>permutation | M |  | ENUMERATED(DFU, UFD) |  | - |  |
| >>>>noofDownlinkSymbols | M |  | INTEGER(0.. maxnoofSymbols-1) |  | - |  |
| >>>>noofUplinkSymbols | M |  | INTEGER(0.. maxnoofSymbols-1) |  | - |  |
| >>>IAB-SlotFormat |  |  |  |  | - |  |
| >>>>IAB-SlotFormatIndex | M |  | INTEGER(0..254) | Index into Table 11.1.1-x and Table 14-1 in TS 38.213 [zz], excluding last row | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSlots | Maximum no. of slots in 10ms. According to TS 38.331 [zz], this value is 320. |
| maxnoofSymbols | Maximum no. of symbols in a slot. According to TS 38.331 [zz], this value is 14. |

NEXT CHANGE

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

### 9.4.4		PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Candidate-SpCell-Item,

Cause,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBID,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DRXConfigurationIndicator,

DUtoCURRCInformation,

EUTRANQoS,

ExecuteDuplication,

FullConfiguration,

GNB-CU-UE-F1AP-ID,

GNB-DU-UE-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-DU-System-Information,

GNB-CU-Name,

GNB-DU-Name,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

NotificationControl,

NRCGI,

NRPCI,

UEContextNotRetrievable,

Potential-SpCell-Item,

RAT-FrequencyPriorityInformation,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCReconfigurationCompleteIndicator,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

SCell-ToBeModified-Item,

ServCellIndex,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

ServingCellMO,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

TimeToWait,

TransactionID,

TransmissionActionIndicator,

UE-associatedLogicalF1-ConnectionItem,

DUtoCURRCContainer,

PagingCell-Item,

SItype-List,

UEIdentityIndexValue,

GNB-CU-TNL-Association-Setup-Item,

GNB-CU-TNL-Association-Failed-To-Setup-Item,

GNB-CU-TNL-Association-To-Add-Item,

GNB-CU-TNL-Association-To-Remove-Item,

GNB-CU-TNL-Association-To-Update-Item,

MaskedIMEISV,

PagingDRX,

PagingPriority,

PagingIdentity,

Cells-to-be-Barred-Item,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

NR-CGI-List-For-Restart-Item,

PWS-Failed-NR-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

RLCFailureIndication,

UplinkTxDirectCurrentListInformation,

SULAccessIndication,

Protected-EUTRA-Resources-Item,

GNB-DUConfigurationQuery,

BitRate,

RRC-Version,

GNBDUOverloadInformation,

RRCDeliveryStatusRequest,

NeedforGap,

RRCDeliveryStatus,

ResourceCoordinationTransferInformation,

Dedicated-SIDelivery-NeededUE-Item,

Associated-SCell-Item,

IgnoreResourceCoordinationContainer,

PagingOrigin,

UAC-Assistance-Info,

RANUEID,

GNB-DU-TNL-Association-To-Remove-Item,

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-ConfirmedUEID,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRB-Activity-Item,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRB-Notify-Item,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ExecuteDuplication,

id-FullConfiguration,

id-gNB-CU-UE-F1AP-ID,

id-gNB-DU-UE-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-CU-Name,

id-gNB-DU-Name,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-new-gNB-CU-UE-F1AP-ID,

id-new-gNB-DU-UE-F1AP-ID,

id-oldgNB-DU-UE-F1AP-ID,

id-PLMNAssistanceInfoForNetShar,

id-Potential-SpCell-Item,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-RedirectedRRCmessage,

id-ResetType,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetup-Item,

id-SCell-FailedtoSetupMod-List,

id-SCell-FailedtoSetupMod-Item,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-SelectedPLMNID,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-ServingCellMO,

id-SpCell-ID,

id-SpCellULConfigured,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-Item,

id-SRBs-Modified-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-SetupMod-Item,

id-SRBs-SetupMod-List,

id-TimeToWait,

id-TransactionID,

id-TransmissionActionIndicator,

id-UEContextNotRetrievable,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

id-DUtoCURRCContainer,

id-NRCGI,

id-PagingCell-Item,

id-PagingCell-List,

id-PagingDRX,

id-PagingPriority,

id-SItype-List,

id-UEIdentityIndexValue,

id-GNB-CU-TNL-Association-Setup-List,

id-GNB-CU-TNL-Association-Setup-Item,

id-GNB-CU-TNL-Association-Failed-To-Setup-List,

id-GNB-CU-TNL-Association-Failed-To-Setup-Item,

id-GNB-CU-TNL-Association-To-Add-Item,

id-GNB-CU-TNL-Association-To-Add-List,

id-GNB-CU-TNL-Association-To-Remove-Item,

id-GNB-CU-TNL-Association-To-Remove-List,

id-GNB-CU-TNL-Association-To-Update-Item,

id-GNB-CU-TNL-Association-To-Update-List,

id-MaskedIMEISV,

id-PagingIdentity,

id-Cells-to-be-Barred-List,

id-Cells-to-be-Barred-Item,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-To-Be-Broadcast-Item,

id-Cells-Broadcast-Completed-List,

id-Cells-Broadcast-Completed-Item,

id-Broadcast-To-Be-Cancelled-List,

id-Broadcast-To-Be-Cancelled-Item,

id-Cells-Broadcast-Cancelled-List,

id-Cells-Broadcast-Cancelled-Item,

id-NR-CGI-List-For-Restart-List,

id-NR-CGI-List-For-Restart-Item,

id-PWS-Failed-NR-CGI-List,

id-PWS-Failed-NR-CGI-Item,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-Protected-EUTRA-Resources-List,

id-RequestType,

id-ServingPLMN,

id-DRXConfigurationIndicator,

id-RLCFailureIndication,

id-UplinkTxDirectCurrentListInformation,

id-SULAccessIndication,

id-Protected-EUTRA-Resources-Item,

id-GNB-DUConfigurationQuery,

id-GNB-DU-UE-AMBR-UL,

id-GNB-CU-RRC-Version,

id-GNB-DU-RRC-Version,

id-GNBDUOverloadInformation,

id-NeedforGap,

id-RRCDeliveryStatusRequest,

id-RRCDeliveryStatus,

id-Dedicated-SIDelivery-NeededUE-List,

id-Dedicated-SIDelivery-NeededUE-Item,

id-ResourceCoordinationTransferInformation,

id-Associated-SCell-List,

id-Associated-SCell-Item,

id-IgnoreResourceCoordinationContainer,

id-UAC-Assistance-Info,

id-RANUEID,

id-PagingOrigin,

id-GNB-DU-TNL-Association-To-Remove-Item,

id-GNB-DU-TNL-Association-To-Remove-List,

id-IAB-MT-Cell-Item,

id-IAB-MT-Cell-List,

id-STC-Info-Item,

id-STC-Info-List,

id-Slot-Config-Item,

id-Slot-Config-List,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofServingCells,

FROM F1AP-Constants;

-- **************************************************************

--

#### -- F1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- F1 Setup Request

--

-- **************************************************************

F1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupRequestIEs} },

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

GNBDUConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateAcknowledgeIEs} },

...

}

GNBDUConfigurationUpdateAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List			CRITICALITY reject	TYPE Cells-to-be-Activated-List				PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List		CRITICALITY reject	TYPE Cells-to-be-Deactivated-List			PRESENCE optional	},

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

-- **************************************************************

--

#### -- GNB-CU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID												PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	 Cells-to-be-Activated-List						PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	 Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	 Cells-to-be-Barred-List							PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	 Protected-EUTRA-Resources-List					PRESENCE optional	},

...

}

Cells-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Deactivated-List-ItemIEs } }

GNB-CU-TNL-Association-To-Add-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Add-ItemIEs } }

GNB-CU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Remove-ItemIEs } }

GNB-CU-TNL-Association-To-Update-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Update-ItemIEs } }

Cells-to-be-Barred-List			::= SEQUENCE(SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { Cells-to-be-Barred-ItemIEs } }

Cells-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-Deactivated-List-Item						CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List-Item					PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-To-Add-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-To-Add-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Add-Item			PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-To-Remove-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-To-Remove-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Remove-Item			PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-To-Update-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-To-Update-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Update-Item			PRESENCE mandatory	},

...

}

Cells-to-be-Barred-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-Barred-Item		CRITICALITY ignore	TYPE	 Cells-to-be-Barred-Item				PRESENCE mandatory	},

...

}

Protected-EUTRA-Resources-List ::= SEQUENCE (SIZE(1.. maxCellineNB))	OF ProtocolIE-SingleContainer { { Protected-EUTRA-Resources-ItemIEs } }

Protected-EUTRA-Resources-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Protected-EUTRA-Resources-Item 					CRITICALITY reject 	TYPE Protected-EUTRA-Resources-Item							PRESENCE mandatory},

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

-- **************************************************************

--

#### -- UE Context Setup ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional	}|

{ ID id-SpCell-ID								CRITICALITY reject	TYPE NRCGI												PRESENCE mandatory	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE mandatory	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE mandatory}|

{ ID id-Candidate-SpCell-List					CRITICALITY ignore	TYPE Candidate-SpCell-List							PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-SCell-ToBeSetup-List					CRITICALITY ignore	TYPE SCell-ToBeSetup-List								PRESENCE optional	}|

{ ID id-SRBs-ToBeSetup-List						CRITICALITY reject	TYPE SRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-DRBs-ToBeSetup-List						CRITICALITY reject	TYPE DRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY ignore	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV										PRESENCE optional	}|

{ ID id-ServingPLMN								CRITICALITY ignore	TYPE PLMN-Identity										PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE conditional }|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest					PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-new-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional }|

{ ID id-RANUEID									CRITICALITY ignore	TYPE RANUEID											PRESENCE optional	}

{ ID id-child-Node-IAB-Info-gNB-CU				CRITICALITY ignore	TYPE

Child-Node-IAB-Info-gNB-CU		PRESENCE optional	}

...

}

Candidate-SpCell-List::= SEQUENCE (SIZE(1..maxnoofCandidateSpCells)) OF ProtocolIE-SingleContainer { { Candidate-SpCell-ItemIEs} }

SCell-ToBeSetup-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetup-ItemIEs} }

SRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetup-ItemIEs} }

DRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetup-ItemIEs} }

Candidate-SpCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Candidate-SpCell-Item					CRITICALITY ignore	TYPE Candidate-SpCell-Item						PRESENCE mandatory	},

...

}

SCell-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetup-Item						CRITICALITY ignore	TYPE SCell-ToBeSetup-Item					PRESENCE mandatory	},

...

}

SRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetup-Item		CRITICALITY reject		TYPE SRBs-ToBeSetup-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetup-Item					CRITICALITY reject	TYPE DRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

-- **************************************************************

--

#### -- UE Context Modification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE NRCGI												PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE optional	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE optional	}|

{ ID id-TransmissionActionIndicator				CRITICALITY ignore	TYPE TransmissionActionIndicator					PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-RRCReconfigurationCompleteIndicator		CRITICALITY ignore	TYPE RRCReconfigurationCompleteIndicator			PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY reject	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-SCell-ToBeSetupMod-List					CRITICALITY ignore	TYPE SCell-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-SCell-ToBeRemoved-List					CRITICALITY ignore	TYPE SCell-ToBeRemoved-List 							PRESENCE optional }|

{ ID id-SRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE SRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE DRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeModified-List					CRITICALITY reject	TYPE DRBs-ToBeModified-List							PRESENCE optional	}|

{ ID id-SRBs-ToBeReleased-List					CRITICALITY reject	TYPE SRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeReleased-List					CRITICALITY reject	TYPE DRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-DRXConfigurationIndicator				CRITICALITY ignore	TYPE DRXConfigurationIndicator						PRESENCE optional	}|

{ ID id-RLCFailureIndication					CRITICALITY ignore	TYPE RLCFailureIndication								PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentListInformation	CRITICALITY ignore	TYPE UplinkTxDirectCurrentListInformation		PRESENCE optional	}|

{ ID id-GNB-DUConfigurationQuery				CRITICALITY reject	TYPE GNB-DUConfigurationQuery						PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE optional	}|

{ ID id-ExecuteDuplication						CRITICALITY ignore	TYPE ExecuteDuplication									PRESENCE optional}|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-NeedforGap								CRITICALITY ignore	TYPE NeedforGap											PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration									PRESENCE optional	}|

{ ID id-child-Node-IAB-Info-gNB-CU				CRITICALITY ignore	TYPE

Child-Node-IAB-Info-gNB-CU		PRESENCE optional	}

...

}

SCell-ToBeSetupMod-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetupMod-ItemIEs} }

SCell-ToBeRemoved-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeRemoved-ItemIEs} }

SRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetupMod-ItemIEs} }

DRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetupMod-ItemIEs} }

DRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeModified-ItemIEs} }

SRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeReleased-ItemIEs} }

DRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeReleased-ItemIEs} }

SCell-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetupMod-Item			CRITICALITY ignore	TYPE SCell-ToBeSetupMod-Item			PRESENCE mandatory	},

...

}

SCell-ToBeRemoved-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeRemoved-Item			CRITICALITY ignore	TYPE SCell-ToBeRemoved-Item			PRESENCE mandatory	},

...

}

SRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE SRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE DRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeModified-Item		CRITICALITY reject	TYPE DRBs-ToBeModified-Item			PRESENCE mandatory},

...

}

SRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeReleased-Item	CRITICALITY reject	TYPE SRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

DRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeReleased-Item		CRITICALITY reject	TYPE DRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

END

-- ASN1STOP

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

id-DRX-LongCycleStartOffset,

id-SelectedBandCombinationIndex,

id-SelectedFeatureSetEntryIndex,

id-Ph-InfoSCG,

id-latest-RRC-Version-Enhanced,

id-RequestedBandCombinationIndex,

id-RequestedFeatureSetEntryIndex,

id-RequestedP-MaxFR2,

id-DRX-Config,

id-UEAssistanceInformation,

id-PDCCH-BlindDetectionSCG,

id-Requested-PDCCH-BlindDetectionSCG,

id-BPLMN-ID-Info-List,

id-NotificationInformation,

id-TNLAssociationTransportLayerAddressgNBDU,

id-portNumber,

id-AdditionalSIBMessageList,

id-IgnorePRACHConfiguration,

id-CG-Config,

id-Ph-InfoMCG,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNRminus1,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN,

maxnoofServingCells,

maxnoofIABstcInfo,

maxnoofSlots

FROM F1AP-Constants

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### -- A

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### -- C

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

Cells-to-be-Activated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-gNB-CUSystemInformation			CRITICALITY reject	EXTENSION GNB-CUSystemInformation			PRESENCE optional }|

{ ID id-AvailablePLMNList				CRITICALITY ignore	EXTENSION AvailablePLMNList					PRESENCE optional }|

{ ID id-ExtendedAvailablePLMN-List		CRITICALITY ignore	EXTENSION ExtendedAvailablePLMN-List		PRESENCE optional }|

{ ID id-IAB-Info-gNB-CU					CRITICALITY ignore	EXTENSION IAB-Info-gNB-CU			PRESENCE optional },

...

}

Cells-to-be-Modified-Item ::= SEQUENCE{

nRCGI 	NRCGI,

iAB-Info-gNB-CU 	IAB-Info-gNB-CU,

...

}

Child-Node-IAB-Info-gNB-CU ::=	SEQUENCE{

bNB-Resource-Configuration 	GNB-Resource-Configuration,

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### -- D

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

MultiplexInfo 	::=	SEQUENCE{

iAB-MT-SCell-List 	IAB-MT-SCell-List,

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### -- E

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

Explicit ::=	SEQUENCE {

Permutation ::=  ENUMERATED{DFU, UFD},

noofDownlinkSymbols	::= INTEGER(0..maxNoofSymbols-1),

noofUplinkSymbols	::= INTEGER(0..maxNoofSymbols-1),

...

}

#### -- G

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

GNB-Resource-Configuration	::= SEQUENCE {

BH-SubcarrierSpacing ::=           ENUMERATED {

kHz15,

kHz30,

kHz60,

kHz120,

kHz240,

spare3,

spare2,

spare1,

...

}

BH-DL-UL-TransmissionPeriodicity ::=      ENUMERATED {

ms0p5,

ms0p625,

ms1,

ms1p25,

ms2,

ms2p5,

ms5,

ms10,

...

}

bH-SlotConfigList	BH-SlotConfigList,

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### --I

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

IAB-Info-gNB-CU ::=	SEQUENCE{

iAB-STC-Info 	 IAB-STC-Info,

gNB-Resource-Configuration 	GNB-Resource-Configuration,

...

}

IAB-Info-gNB-DU ::=	SEQUENCE{

multiplexInfo 		MultiplexInfo,

iAB-STC-Info 	 IAB-STC-Info,

...

}

IAB-MT-SCell-List ::= SEQUENCE (SIZE(1..maxnoofServingCells)) OF IAB-MT-SCell-List-Item

IAB-MT-SCell-List-Item ::= 	SEQUENCE {

nRCellIdentity			NRCellIdentity,

DU-RX-MT-RX ::= ENUMERATED {supported, not-supported, ...}

DU-TX-MT-TX ::= ENUMERATED {supported, not-supported, ...}

DU-RX-MT-TX ::= ENUMERATED {supported, not-supported, ...}

DU-TX-MT-RX ::= ENUMERATED {supported, not-supported, ...}

...

}

IAB-SlotConfig ::=  CHOICE{

explicit 	Explicit,

iAH-SlotFormat	IAB-SlotFormat,

...

}

IAB-SlotConfigList ::= SEQUENCE(SIZE(1..maxnoofSlots)) OF IAB-SlotConfigListItem

IAB-SlotConfigListItem ::= SEQUENCE {

Downlink ::= ENUMERATED {HARD, SOFT, NOTAVAILABLE}

Uplink ::= ENUMERATED {HARD, SOFT, NOTAVAILABLE}

Flexible ::= ENUMERATED {HARD, SOFT, NOTAVAILABLE}

iAB-SlotConfig 	IAB-SlotConfig,

...

}

IAB-SlotFormat ::=	SEQUENCE {

SlotFormatIndex ::=  	INTEGER(0..254)

}

IAB-STC-Info 	::=	SEQUENCE{

iAB-STC-INFO-LIST 	IAB-MT-STC-Info-LIST,

...

}

IAB-MT-STC-Info-List ::= 	SEQUENCE (1..maxnoofIABstcInfo) OF IAB-MT-STC-Info-List-Item

IAB-MT-STC-Info-Item::= 	SEQUENCE {

SSB-NRfreqInfo	NRFreqInfo,

SSB-transmissionBandwidth	Transmission-Bandwidth,

SSB-transmissionPeriodicity	::= ENUMERATED {

sf10,

sf20,

sf40,

sf80,

sf160,

sf320,

sf640,

...}

SSB-transmissionTimingOffset	::= INTEGER (0..127, ...),

SSB-transmisisonBitmap	::= OCTET STRING,

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### -- S

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

SCell-ToBeModified-Item::= SEQUENCE{

sCell-ID			NRCGI,

child-Node-IAB-Info-gNB-CUChild-Node-IAB-Info-gNB-CU,

...

}

Served-Cell-Information ::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

fiveGS-TAC							FiveGS-TAC			OPTIONAL,

configured-EPS-TAC				Configured-EPS-TAC 		OPTIONAL,

servedPLMNs					ServedPLMNs-List,

nR-Mode-Info					NR-Mode-Info,

measurementTimingConfiguration	OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { {Served-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-Cell-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RANAC							CRITICALITY ignore EXTENSION RANAC							PRESENCE optional }|

{	ID id-ExtendedServedPLMNs-List		CRITICALITY ignore EXTENSION ExtendedServedPLMNs-List		PRESENCE optional }|

{	ID id-Cell-Direction				CRITICALITY ignore EXTENSION Cell-Direction					PRESENCE optional }|

{	ID id-BPLMN-ID-Info-List			CRITICALITY ignore EXTENSION BPLMN-ID-Info-List				PRESENCE optional }|

{	ID id-Cell-Type						CRITICALITY ignore EXTENSION CellType						PRESENCE optional} |

{	ID id-IAB-info						CRITICALITY ingore EXTENSION IAB-Info-gNB-DU

PRESENCE optional}

,

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### -- T

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

TDD-Resource-Configuration	::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

SubcarrierSpacing ::=           ENUMERATED {

kHz15,

kHz30, kHz60,

kHz120, kHz240,

spare3,

spare2,

spare1

}

dL-UL-TransmissionPeriodicity ::=      ENUMERATED {

ms0p5,

ms0p625,

ms1,

ms1p25,

ms2,

ms2p5,

ms5,

ms10

}

slotConfigListSlotConfigList,

...

}

SlotConfigList ::= SEQUENCE(SIZE(1..maxnoofSlots)) of SlotConfigItem

SlotConfigItem ::= SEQUENCE {

Downlink ::= ENUMERATED {HARD, SOFT, NOTAVAILABLE}

Uplink ::= ENUMERATED {HARD, SOFT, NOTAVAILABLE}

Flexible ::= ENUMERATED {HARD, SOFT, NOTAVAILABLE}

SlotConfig ::=  CHOICE{

Explicit ::=	SEQUENCE {

Permutation ::=  ENUMERATED{DFU, UFD}

noofDownlinkSymbols	::= INTEGER(0..maxNoofSymbols-1)

noofUplinkSymbols	::= INTEGER(0..maxNoofSymbols-1)

}

SlotForm ::=	SEQUENCE {

SlotFormatIndex ::=  	INTEGER(0..254)

}

}

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

### 9.4.7	Constant Definitions

-- ASN1START

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNRminus1					INTEGER ::= 11

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofIABstcInfo					Integer ::= 4

maxnoofSlots							Integer ::= 320

maxnoofSymbols							Integer ::= 14

maxnoofServingCells						Integer	::=	32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

id-CUtoDURRCInformation								ProtocolIE-ID ::= 9

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 12

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 13

id-DRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 14

id-DRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 15

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 16

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 17

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 18

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 19

id-DRBs-Modified-Item								ProtocolIE-ID ::= 20

id-DRBs-Modified-List								ProtocolIE-ID ::= 21

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 22

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 23

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 24

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 25

id-DRBs-Setup-Item									ProtocolIE-ID ::= 26

id-DRBs-Setup-List									ProtocolIE-ID ::= 27

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 28

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 29

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 30

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 31

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 32

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 33

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 34

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 35

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 36

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 37

id-DRXCycle											ProtocolIE-ID ::= 38

id-DUtoCURRCInformation								ProtocolIE-ID ::= 39

id-gNB-CU-UE-F1AP-ID								ProtocolIE-ID ::= 40

id-gNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 41

id-gNB-DU-ID										ProtocolIE-ID ::= 42

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 43

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 44

id-gNB-DU-Name										ProtocolIE-ID ::= 45

id-NRCellID											ProtocolIE-ID ::= 46

id-oldgNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 47

id-ResetType										ProtocolIE-ID ::= 48

id-ResourceCoordinationTransferContainer			ProtocolIE-ID ::= 49

id-RRCContainer										ProtocolIE-ID ::= 50

id-SCell-ToBeRemoved-Item							ProtocolIE-ID ::= 51

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 52

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 53

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 54

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 55

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 56

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 57

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 58

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 59

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 60

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 61

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 62

id-SpCell-ID										ProtocolIE-ID ::= 63

id-SRBID											ProtocolIE-ID ::= 64

id-SRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 65

id-SRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 66

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 67

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 68

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 69

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 70

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 71

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 72

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 73

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 74

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 75

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 76

id-TimeToWait										ProtocolIE-ID ::= 77

id-TransactionID									ProtocolIE-ID ::= 78

id-TransmissionActionIndicator						ProtocolIE-ID ::= 79

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 80

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 81

id-gNB-CU-Name										ProtocolIE-ID ::= 82

id-SCell-FailedtoSetup-List							ProtocolIE-ID ::= 83

id-SCell-FailedtoSetup-Item							ProtocolIE-ID ::= 84

id-SCell-FailedtoSetupMod-List						ProtocolIE-ID ::= 85

id-SCell-FailedtoSetupMod-Item						ProtocolIE-ID ::= 86

id-RRCReconfigurationCompleteIndicator				ProtocolIE-ID ::= 87

id-Cells-Status-Item								ProtocolIE-ID ::= 88

id-Cells-Status-List								ProtocolIE-ID ::= 89

id-Candidate-SpCell-List							ProtocolIE-ID ::= 90

id-Candidate-SpCell-Item							ProtocolIE-ID ::= 91

id-Potential-SpCell-List							ProtocolIE-ID ::= 92

id-Potential-SpCell-Item							ProtocolIE-ID ::= 93

id-FullConfiguration								ProtocolIE-ID ::= 94

id-C-RNTI											ProtocolIE-ID ::= 95

id-SpCellULConfigured								ProtocolIE-ID ::= 96

id-InactivityMonitoringRequest						ProtocolIE-ID ::= 97

id-InactivityMonitoringResponse						ProtocolIE-ID ::= 98

id-DRB-Activity-Item								ProtocolIE-ID ::= 99

id-DRB-Activity-List								ProtocolIE-ID ::= 100

id-EUTRA-NR-CellResourceCoordinationReq-Container 		ProtocolIE-ID ::= 101

id-EUTRA-NR-CellResourceCoordinationReqAck-Container 	ProtocolIE-ID ::= 102

id-Protected-EUTRA-Resources-List					ProtocolIE-ID ::= 105

id-RequestType 										ProtocolIE-ID ::= 106

id-ServCellIndex									ProtocolIE-ID ::= 107

id-RAT-FrequencyPriorityInformation					ProtocolIE-ID ::= 108

id-ExecuteDuplication								ProtocolIE-ID ::= 109

id-NRCGI											ProtocolIE-ID ::= 111

id-PagingCell-Item									ProtocolIE-ID ::= 112

id-PagingCell-List									ProtocolIE-ID ::= 113

id-PagingDRX										ProtocolIE-ID ::= 114

id-PagingPriority 									ProtocolIE-ID ::= 115

id-SItype-List										ProtocolIE-ID ::= 116

id-UEIdentityIndexValue								ProtocolIE-ID ::= 117

id-gNB-CUSystemInformation							ProtocolIE-ID ::= 118

id-HandoverPreparationInformation					ProtocolIE-ID ::= 119

id-GNB-CU-TNL-Association-To-Add-Item				ProtocolIE-ID ::= 120

id-GNB-CU-TNL-Association-To-Add-List				ProtocolIE-ID ::= 121

id-GNB-CU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 122

id-GNB-CU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 123

id-GNB-CU-TNL-Association-To-Update-Item			ProtocolIE-ID ::= 124

id-GNB-CU-TNL-Association-To-Update-List			ProtocolIE-ID ::= 125

id-MaskedIMEISV										ProtocolIE-ID ::= 126

id-PagingIdentity									ProtocolIE-ID ::= 127

id-DUtoCURRCContainer								ProtocolIE-ID ::= 128

id-Cells-to-be-Barred-List							ProtocolIE-ID ::= 129

id-Cells-to-be-Barred-Item							ProtocolIE-ID ::= 130

id-TAISliceSupportList								ProtocolIE-ID ::= 131

id-GNB-CU-TNL-Association-Setup-List				ProtocolIE-ID ::= 132

id-GNB-CU-TNL-Association-Setup-Item				ProtocolIE-ID ::= 133

id-GNB-CU-TNL-Association-Failed-To-Setup-List		ProtocolIE-ID ::= 134

id-GNB-CU-TNL-Association-Failed-To-Setup-Item		ProtocolIE-ID ::= 135

id-DRB-Notify-Item									ProtocolIE-ID ::= 136

id-DRB-Notify-List									ProtocolIE-ID ::= 137

id-NotficationControl								ProtocolIE-ID ::= 138

id-RANAC											ProtocolIE-ID ::= 139

id-PWSSystemInformation								ProtocolIE-ID ::= 140

id-RepetitionPeriod									ProtocolIE-ID ::= 141

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 142

id-Cells-To-Be-Broadcast-List						ProtocolIE-ID ::= 144

id-Cells-To-Be-Broadcast-Item						ProtocolIE-ID ::= 145

id-Cells-Broadcast-Completed-List 					ProtocolIE-ID ::= 146

id-Cells-Broadcast-Completed-Item 					ProtocolIE-ID ::= 147

id-Broadcast-To-Be-Cancelled-List 					ProtocolIE-ID ::= 148

id-Broadcast-To-Be-Cancelled-Item 					ProtocolIE-ID ::= 149

id-Cells-Broadcast-Cancelled-List 					ProtocolIE-ID ::= 150

id-Cells-Broadcast-Cancelled-Item 					ProtocolIE-ID ::= 151

id-NR-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 152

id-NR-CGI-List-For-Restart-Item 					ProtocolIE-ID ::= 153

id-PWS-Failed-NR-CGI-List 							ProtocolIE-ID ::= 154

id-PWS-Failed-NR-CGI-Item 							ProtocolIE-ID ::= 155

id-ConfirmedUEID									ProtocolIE-ID ::= 156

id-Cancel-all-Warning-Messages-Indicator			ProtocolIE-ID ::= 157

id-GNB-DU-UE-AMBR-UL								ProtocolIE-ID ::= 158

id-DRXConfigurationIndicator						ProtocolIE-ID ::= 159

id-RLC-Status										ProtocolIE-ID ::= 160

id-DLPDCPSNLength									ProtocolIE-ID ::= 161

id-GNB-DUConfigurationQuery							ProtocolIE-ID ::= 162

id-MeasurementTimingConfiguration					ProtocolIE-ID ::= 163

id-DRB-Information									ProtocolIE-ID ::= 164

id-ServingPLMN										ProtocolIE-ID ::= 165

id-Protected-EUTRA-Resources-Item					ProtocolIE-ID ::= 168

id-GNB-CU-RRC-Version								ProtocolIE-ID ::= 170

id-GNB-DU-RRC-Version								ProtocolIE-ID ::= 171

id-GNBDUOverloadInformation							ProtocolIE-ID ::= 172

id-CellGroupConfig									ProtocolIE-ID ::= 173

id-RLCFailureIndication								ProtocolIE-ID ::= 174

id-UplinkTxDirectCurrentListInformation				ProtocolIE-ID ::= 175

id-DC-Based-Duplication-Configured					ProtocolIE-ID ::= 176

id-DC-Based-Duplication-Activation					ProtocolIE-ID ::= 177

id-SULAccessIndication								ProtocolIE-ID ::= 178

id-AvailablePLMNList								ProtocolIE-ID ::= 179

id-PDUSessionID										ProtocolIE-ID ::= 180

id-ULPDUSessionAggregateMaximumBitRate				ProtocolIE-ID ::= 181

id-ServingCellMO									ProtocolIE-ID ::= 182

id-QoSFlowMappingIndication							ProtocolIE-ID ::= 183

id-RRCDeliveryStatusRequest							ProtocolIE-ID ::= 184

id-RRCDeliveryStatus								ProtocolIE-ID ::= 185

id-BearerTypeChange									ProtocolIE-ID ::= 186

id-RLCMode											ProtocolIE-ID ::= 187

id-Duplication-Activation							ProtocolIE-ID ::= 188

id-Dedicated-SIDelivery-NeededUE-List				ProtocolIE-ID ::= 189

id-Dedicated-SIDelivery-NeededUE-Item				ProtocolIE-ID ::= 190

id-DRX-LongCycleStartOffset							ProtocolIE-ID ::= 191

id-ULPDCPSNLength									ProtocolIE-ID ::= 192

id-SelectedBandCombinationIndex						ProtocolIE-ID ::= 193

id-SelectedFeatureSetEntryIndex						ProtocolIE-ID ::= 194

id-ResourceCoordinationTransferInformation			ProtocolIE-ID ::= 195

id-ExtendedServedPLMNs-List							ProtocolIE-ID ::= 196

id-ExtendedAvailablePLMN-List						ProtocolIE-ID ::= 197

id-Associated-SCell-List							ProtocolIE-ID ::= 198

id-latest-RRC-Version-Enhanced						ProtocolIE-ID ::= 199

id-Associated-SCell-Item							ProtocolIE-ID ::= 200

id-Cell-Direction									ProtocolIE-ID ::= 201

id-SRBs-Setup-List									ProtocolIE-ID ::= 202

id-SRBs-Setup-Item									ProtocolIE-ID ::= 203

id-SRBs-SetupMod-List								ProtocolIE-ID ::= 204

id-SRBs-SetupMod-Item								ProtocolIE-ID ::= 205

id-SRBs-Modified-List								ProtocolIE-ID ::= 206

id-SRBs-Modified-Item								ProtocolIE-ID ::= 207

id-Ph-InfoSCG										ProtocolIE-ID ::= 208

id-RequestedBandCombinationIndex					ProtocolIE-ID ::= 209

id-RequestedFeatureSetEntryIndex					ProtocolIE-ID ::= 210

id-RequestedP-MaxFR2								ProtocolIE-ID ::= 211

id-DRX-Config										ProtocolIE-ID ::= 212

id-IgnoreResourceCoordinationContainer				ProtocolIE-ID ::= 213

id-UEAssistanceInformation							ProtocolIE-ID ::= 214

id-NeedforGap										ProtocolIE-ID ::= 215

id-PagingOrigin										ProtocolIE-ID ::= 216

id-new-gNB-CU-UE-F1AP-ID							ProtocolIE-ID ::= 217

id-RedirectedRRCmessage								ProtocolIE-ID ::= 218

id-new-gNB-DU-UE-F1AP-ID							ProtocolIE-ID ::= 219

id-NotificationInformation							ProtocolIE-ID ::= 220

id-PLMNAssistanceInfoForNetShar						ProtocolIE-ID ::= 221

id-UEContextNotRetrievable							ProtocolIE-ID ::= 222

id-BPLMN-ID-Info-List								ProtocolIE-ID ::= 223

id-SelectedPLMNID									ProtocolIE-ID ::= 224

id-UAC-Assistance-Info								ProtocolIE-ID ::= 225

id-RANUEID											ProtocolIE-ID ::= 226

id-GNB-DU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 227

id-GNB-DU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 228

id-TNLAssociationTransportLayerAddressgNBDU			ProtocolIE-ID ::= 229

id-portNumber										ProtocolIE-ID ::= 230

id-AdditionalSIBMessageList							ProtocolIE-ID ::= 231

id-Cell-Type										ProtocolIE-ID ::= 232

id-IgnorePRACHConfiguration							ProtocolIE-ID ::= 233

id-CG-Config										ProtocolIE-ID ::= 234

id-PDCCH-BlindDetectionSCG							ProtocolIE-ID ::= 235

id-Requested-PDCCH-BlindDetectionSCG				ProtocolIE-ID ::= 236

id-Ph-InfoMCG										ProtocolIE-ID ::= 237

END

-- ASN1STOP

END OF CHANGE