3GPP TSG-RAN3 Meeting #107-e	R3-200550

24 February - 6 March 2020 E-Meeting

Agenda item:	15.4.1.2

Source:	ZTE

Title:	TP for LTE_Mob_enh BL CR for TS 36.423 Correction of CHO Per Target Cell TEID

Document for:	Agreement

1	Introduction

In the agreed R3-196699 at RAN3#106, some confusions regarding HO success procedure were still left.

The “Late data forwarding” is purely NW behavior, and it cannot be really configured or conducted by UE. Hence it is suggested to reword relevant statement like “If the conditional handover is configured for this UE...”.

The source node may conditional prepare multiple candidate target nodes in parallel, so once the real target node is fixed by UE via sending the HANDOVER SUCCESS message, the source node should also cancel all other candidate target nodes if any, otherwise the CHO resources shall be kept useless for longer time. Such NW side behavior should be captured.

Some cleanups in IE semantics.

2	Text proposal

This TP is based on the BL CR endorsed after RAN3 #106 [R3-197805].

*** First change, skipped text not changed ***

### 8.2.X	Handover Success

#### 8.2.X.1	General

The Handover Success procedure is used during a conditional handover or a DAPS handover, to enable a target eNB to inform the source eNB that the UE has successfully accessed the target eNB.

The procedure uses UE-associated signalling.

#### 8.2.X.2	Successful Operation

Figure 8.2.X.2-1: Handover Success, successful operation

The target eNB initiates the procedure by sending the HANDOVER SUCCESS message to the source eNB.

If the conditional handover was configured for this UE, the source eNB shall start data forwarding using the tunnel information related to the global target cell ID provided in the HANDOVER SUCCESS message.

When the source eNB receives the HANDOVER SUCCESS message, it shall consider all other CHO preparations accepted for this UE in the target eNB as cancelled, and it shall also cancel all other CHO preparations accepted for this UE in all other target eNBs if any.

#### 8.2.X.3	Unsuccessful Operation

Not applicable.

#### 8.2.X.4	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source eNB shall ignore the message.

*** Next change, skipped text not changed ***

9.1.1.Y	HANDOVER SUCCESS

This message is sent by the target eNB to the source eNB to indicate the successful access of the UE toward the target eNB.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | reject |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | ignore |
| Target Cell ID | M |  | ECGI 9.2.14 | Target cell which UE successfully accessed | YES | reject |

*** Remaining text not changed ***