3GPP TSG-RAN WG3 #107-e 	R3-200561

24 February-6 March 2020 E-Meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0105 | rev | <Rev#> | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | E2E delay measurement for Qos monitoring for URLLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2020-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN3 has agreed to support the QoS monitoring request from CN. DU sends the UL/DL delay measurement results to the CU-UP via the GTP-U. CU-UP calculates the whole UL/DL RAN delay and sends the results to the UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the UL/DL delay measurement results in the GTP-U from DU to DU |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CN will not get the delay between UE and UPF and the delay between NG-RAN and UPF. Also NG-RAN can not get the whole UL/DL delay of RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1,5.5.2.3,5.5.3.a(new),5.5.3.b(new),5.5.3.c(new),5.5.3.d(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.415 CR 0012 TS 38.423 CR 0310 TS 38.413 CR 0329 TS 38.463 CR 0481 TS 38.473 CR 0530 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

### 5.4.3	Transfer of Assistance Information

#### 5.4.3.1	Successful operation

The purpose of the Transfer of Assistance Information procedure is to provide assistance information to the node hosting the NR PDCP entity. Such information may be taken into consideration by the node hosting the NR PDCP entity for UP management and optimisation procedures.

An NR user plane protocol instance making use of the Transfer of Assistance Information procedure is associated to a single data radio bearer only.

The Transfer of Assistance Information procedure may be invoked if the corresponding node decides to send the Radio Quality Assistance Information and/or the PDCP duplication activation suggestion to the node hosting the NR PDCP entity for the concerned data radio bearer.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.

The ASSISTANCE INFORMATION DATA frame may include one or more Radio Quality Assistance Information. The information shall consist of the information indicated in the Assistance Information Type.

The ASSISTANCE INFORMATION DATA shall be sent, if supported, when the corresponding node receives a DL USER DATA PDU including the Assistance Information Report Polling Flag set to 1.

The ASSISTANCE INFORMATION DATA frame may include the PDCP Duplication Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or not activate DL PDCP duplication. The node hosting the NR PDCP entity may take this information into account to take a decision on whether to activate or not activate PDCP duplication.

The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay of RAN.

Figure 5.4.3.1-1: Successful Transfer of Assistance Information Data

Next Change

#### 5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the NR PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 |  | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=2) |  |  |  | PDCP Dupl. Ind. |  | Assistance Info. Ind. |  | UL Delay Ind. | DL Delay Ind. | 1 |
| Spare |  |  |  |  |  |  |  |  | PDCP Duplication Activation Suggestion | 1 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |  |  |
| UL Delay |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL Delay |  |  |  |  |  |  |  |  |  | 0 or 3 |

Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format

Next Change

### 5.5.3	Coding of information elements in frames

#### 5.5.3.a	UL Delay Indicator(UDI)

Description: This parameter indicates the presence of the UL Delay.

Value range: {0= UL Delay not present, 1= UL Delay present}.

Field length: 1 bit.

#### 5.5.3.b	DL Delay Indicator(DDI)

Description: This parameter indicates the presence of the DL Delay.

Value range: {0= DL Delay not present, 1= DL Delay present}.

Field length: 1 bit.

#### 5.5.3.c	UL Delay

Description: This parameter indicates the UL delay of transmitting the UL packet of the concerned data radio bearer in units of 0.1 millisecs.

Value range: {0, 224-1}.

Field length: 3 octets.

#### 5.5.3.d	DL Delay

Description: This parameter indicates the DL delay of transmitting the DL packet of the concerned data radio bearer in units of 0.1 millisecs.

Value range: {0, 224-1}.

Field length: 3 octets.

End Change