3GPP TSG-RAN WG3 #107-e	R3-200651

24 February-6 March 2020 E-Meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.470 | CR | 0001 | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to 37.470 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom, Orange, TIM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are still some editorials in the current specification, and the references related to the other W1 interface specifications are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Fix the editorials, including: - To align the naming issue, e.g. to reflect NG-RAN with addition of “ng-“; - To complete the interface management procedure list; - To remove redundant section. - Add the references related to the other W1 interface specifications - Add other W1 interface specifications Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR has no impact with the previous version of the specification (same release) because it does not add any new IEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are still ambiguities in current spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------------Change Start--------------------------------------------------------

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.425: "NR user plane protocol".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 37.471: "E-UTRAN and NG-RAN; W1 interface: layer 1".

[5]	3GPP TS 37.472: "E-UTRAN and NG-RAN; W1 signalling transport".

[6]	3GPP TS 37.473: "E-UTRAN and NG-RAN; W1 application protocol (W1AP)".

--------------------------------------------------------Next Change--------------------------------------------------------

5	Functions of the W1 interface

5.1	General

The following clauses describe the functions supported over W1-C and W1-U.

5.2	W1-C functions

5.2.1	Interface management function

The W1 setup function allows to exchange application level data needed for the ng-eNB-DU and ng-eNB-CU to interoperate correctly on the W1 interface. The W1 setup is initiated by the ng-eNB-DU.

The ng-eNB-CU Configuration Update and ng-eNB-DU Configuration Update functions allow to update application level configuration data needed between ng-eNB-CU and ng-eNB-DU to interoperate correctly over the W1 interface, and may activate or deactivate cells.

For NG-RAN, the W1 setup and ng-eNB-DU Configuration Update functions allow to inform the S-NSSAI(s) supported by the ng-eNB-DU.

The error indication function is used by the ng-eNB-DU or ng-eNB-CU to indicate to the ng-eNB-CU or ng-eNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the ng-eNB-DU and the ng-eNB-CU.

The W1 resource coordination function is used to transfer information about frequency resource sharing between ng-eNB-CU and ng-eNB-DU.

The ng-eNB-DU status indication function allows the ng-eNB-DU to indicate overload status to ng-eNB-CU.

5.2.2	System Information management function

Scheduling of system broadcast information is carried out in the ng-eNB-DU. The ng-eNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

The ng-eNB-DU is responsible for the encoding of ng-eNB-MIB. In case broadcast of SIB1 and other SI messages is needed, the ng-eNB-DU is responsible for assembling SIB1, SIB2, SIB3, SIB8 and SIB16 and the ng-eNB-CU is responsible for assembling other SIBs.

Editor's note: The handling of NB IoT related system information may be discussed when the support for NB IoT in NG-RAN is discussed

5.2.3	Paging function

The ng-eNB-DU is responsible for transmitting the paging information.

The ng-eNB-CU provides paging information to enable the ng-eNB-DU to calculate the exact PO and PF. The ng-eNB-CU determines the PA. The ng-eNB-DU consolidates all the paging records for a particular PO, PF and PA, and encodes the final RRC message and broadcasts the paging message on the respective PO, PF in the PA.

5.2.4	UE context management function

The W1 UE context management function supports the establishment and modification of the necessary overall UE context.

The establishment of the W1 UE context is initiated by the ng-eNB-CU and accepted or rejected by the ng-eNB-DU based on admission control criteria (e.g., resource not available).

The modification of the W1 UE context can be initiated by either ng-eNB-CU or ng-eNB-DU. The receiving node can accept or reject the modification. The W1 UE context management function also supports the release of the context previously established in the ng-eNB-DU. The release of the context is triggered by the ng-eNB-CU either directly or following a request received from the ng-eNB-DU. The ng-eNB-CU requests the ng-eNB-DU to remove the UE Context when the UE enters RRC_IDLE or RRC_INACTIVE.

This function can be also used to manage DRBs and SRBs, i.e., establishing, modifying and releasing DRB and SRB resources. The establishment and modification of DRB resources are triggered by the ng-eNB-CU and accepted/rejected by the ng-eNB-DU based on resource reservation information and QoS information to be provided to the ng-eNB-DU. For each DRB to be setup or modified, the S-NSSAI may be provided by ng-eNB-CU to the ng-eNB-DU in the UE Context Setup procedure and the UE Context Modification procedure.

For NG-RAN, the mapping between QoS flows and radio bearers is performed by ng-eNB-CU and the granularity of bearer related management over W1 is radio bearer level, and the ng-eNB-CU decides an aggregated DRB QoS profile for each radio bearer based on received QoS flow profile, and provides both aggregated DRB QoS profile and QoS flow profile to the ng-eNB-DU, and the ng-eNB-DU either accepts the request or rejects it with appropriate cause value.

With this function, ng-eNB-DU could also notify ng-eNB-CU whether the QoS for already established DRBs is not fulfilled any longer or it is fulfilled again.

The UE Inactivity Notification function is initiated by the ng-eNB-DU to indicate the UE activity event.

The Notify function is to enable the ng-eNB-DU to inform the ng-eNB-CU that the QoS of an already established GBR DRB cannot by fulfilled any longer or that it can be fulfilled again.

With this function, the ng-eNB-CU indicates the UL UE AMBR limit to the ng-eNB-DU, and the ng-eNB-DU enforces the indicated limit.

With this function, the ng-eNB-CU indicates the UL UE AMBR limit to the ng-eNB-DU, and the ng-eNB-DU enforces the indicated limit.

With this function, ng-eNB-CU requests the ng-eNB-DU to setup or change of the SpCell for the UE, and the ng-eNB-DU either accepts or rejects the request with appropriate cause value.

With this function, the ng-eNB-CU requests the setup of the SCell(s) at the ng-eNB-DU side, and the ng-eNB-DU accepts all, some or none of the SCell(s) and replies to the ng-eNB-CU. The ng-eNB-CU requests the removal of the SCell(s) for the UE.

With this function, the ng-eNB-DU indicates that a bearer, or a UE is inactive or active. The ng-eNB-CU consolidates all the serving ng-eNB-DUs for the UE and takes further action.

5.2.5	RRC message transfer function

This function allows to transfer RRC messages between ng-eNB-CU and ng-eNB-DU. RRC messages are transferred over W1-C. The ng-eNB-CU is responsible for the encoding of the dedicated RRC message with assistance information provided by ng-eNB-DU. This function also allows ng-eNB-DU to report to ng-eNB-CU if the downlink RRC message has been successfully delivered to UE or not.

5.2.6	Warning messages information transfer function

This function allows to cooperate with the warning message transmission procedures over NG interface. The ng-eNB-CU is responsible for encoding the warning related SI message and sending it together with other warning related information for the ng-eNB-DU to broadcast over the radio interface.

5.3	W1-U functions

5.3.1	Transfer of user data

This function allows to transfer of user data between ng-eNB-CU and ng-eNB-DU.

5.3.2	Flow control function

This function allows to control the downlink user data flow to the ng-eNB-DU. The detailed protocol is specified in TS 38.425 [2]

6	Procedures of the W1 interface

6.1	Control plane procedures

6.1.1	Interface Management procedures

The W1 Interface management procedures are listed below:

-	Reset procedure

-	Error Indication procedure

-	W1 Setup procedure

-	ng-eNB-DU Configuration Update procedure

-	ng-eNB-CU Configuration Update procedure

-	ng-eNB-DU Resource Coordination procedure

-	ng-eNB-DU Status Indication procedure

6.1.2	Context Management procedures

The W1 Context management procedures are listed below:

-	UE Context Setup procedure

-	UE context Release Request (ng-eNB-DU initiated) procedure

-	UE context Release (ng-eNB-CU initiated) procedure

-	UE Context Modification (ng-eNB-CU initiated) procedure

-	UE Context Modification Required (ng-eNB-DU initiated) procedure

-	UE Inactivity Notification procedure

-	Notify procedure procedure

6.1.3	RRC Message Transfer procedures

The W1 RRC message transfer procedures are listed below:

-	Initial UL RRC Message Transfer procedure

-	UL RRC Message Transfer procedure

-	DL RRC Message Transfer procedure

-	RRC Delivery Report procedure

6.1.4	Warning Message Transmission procedures

The W1 Warning message transmission procedures are listed below:

-	Write-Replace Warning procedure

-	PWS Cancel procedure

-	PWS Restart Indication procedure

-	PWS Failure Indication procedure

6.1.5	Paging procedures

The W1 Paging procedures are listed below:

-	Paging

6.2	User plane procedures

6.2.1	User Data Transfer

Void

6.2.2	Flow Control

Void

--------------------------------------------------------Next Change--------------------------------------------------------

8	Other W1 interface specifications

This clause contains the description of the other related 3GPP specifications.

8.1	E-UTRAN and NG-RAN; W1 interface: layer 1 (3GPP TS 37.471)

3GPP TS 37.471 [4] specifies the physical layer technologies that may be used to support the W1 interface.

## 8.2	E-UTRAN and NG-RAN; W1 signalling transport (3GPP TS 37.472)

3GPP TS 37.472 [5] specifies the signalling bearers for the W1AP for the W1-C interface.

## 8.3	E-UTRAN and NG-RAN; W1 signalling transport (3GPP TS 37.473)

3GPP TS 37.473 [6] specifies the F1AP protocol for radio network control plane signalling over the W1 interface.

## 8.4	NG-RAN; NR user plane protocol (3GPP TS 38.425)

3GPP TS 38.425 [2] specifies the user plane protocol being used over the W1-U interface.

--------------------------------------------------------Change End--------------------------------------------------------