3GPP TSG-RAN3 Meeting #107-e	R3-200714

E-Meeting, 24 February – 6 March, 2020

Title: 	(TP for NPN BL CR for TS 38.413): Mobility restriction list

Source: 	Huawei

Agenda item:	16.2.2.2

Document for:	Discussion

1. Introduction

This paper intends to further discussion issue on Mobility Restriction List in the BL CR for NGAP and XnAP [1, 2].

2. Discussion

Currently the Allowed PNI-NPN List is formatted as follows for NGAP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Allowed PNI-NPN List |  | 1 |  |  |
| >Allowed PNI-NPN Item |  | 1..<maxnoofEPLMNs+1> |  |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  |
| >>PNI-NPN restricted | M |  | ENUMERATED(restricted, not-restricted, …) | If set to “restricted”, indicates that the UE may not access public (non-CAG) cells for this PLMN. |
| >>Allowed CAG list per PLMN |  | 1..<maxnoofCAGsperPLMN> |  |  |
| >>>>CAG ID | M |  | 9.3.3.Y2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs+1 | Maximum no. of equivalent PLMNs plus one serving PLMN. Value is 16. |
| maxnoofAllowedCAGsperPLMN | Maximum number of CAGs per PLMN in UE’s Allowed PNI-NPN list. Value is FFS. |

2. 1 The maximum number of CAGs

The per PLMN in UE’s Allowed PNI-NPN list is FFS. The maximum number of PLMNs included in Mobility Restriction List is 16 as follow,

| maxnoofEPLMNs+1 | Maximum no. of equivalent PLMNs plus one serving PLMN. Value is 16. |
| --- | --- |

It implies the maximum number of public networks included in Mobility Restriction List is 16. In RAN2, it is already agreed that:

- The total number of PLMNs in the PLMN-IdentityInfoList does not exceed 12.

For the latest the email discussion [108#37][PRN] RRC Running CR (Nokia) for the information broadcasted in SIB1,

- The total number of PLMNs (identified by a PLMN identity in plmn -IdentityList), PNI-NPNs (identified by a PLMN identity and a CAG-ID), and SNPNs (identified by a PLMN identity and a NID) together in the PLMN-IdentityInfoList and NPN-IdentityInfoList does not exceed 12.

Hence it is reasonable to restrict the maximum number of CAGs per PLMN in UE’s Allowed PNI-NPN list to 16, following the principle of legacy spec TS 38.413. And the total number of public networks and PNI-NPNs does not exceed 16 (Note that CHOICE structure was introduced in Mobility Restriction List and there is no SNPN included in Mobility Restriction List when there is allowed PNI-NPN List).

The maximum number of CAGs per PLMN in UE’s Allowed PNI-NPN list is 16.

2. 2 PNI-NPN restricted

In BL CR for NGAP, the allowed PNI-NPN restricted IE is enumered as restricted, or not-restricted.  If set to “restricted”, indicates that the UE may not access public (non-CAG) cells for this PLMN..

But in fact, in the agreed TPs, the PNI-NPN restricted is formatted as follows [1], [2].

| >>PNI-NPN restricted | O |  | ENUMERATED(CAG-only, …) | Indicates that the UE may only access cells that support PNI-NPNs for this PLMN. |
| --- | --- | --- | --- | --- |

And this format is quite aligned with stage 2 in TS 23. 501 as follows.

To support CAG, the UE may be pre-configured or (re)configured with the following CAG information, included in the subscription as part of the Mobility Restrictions:

-	an Allowed CAG list i.e. a list of CAG Identifiers the UE is allowed to access; and

-	optionally, a CAG-only indication whether the UE is only allowed to access 5GS via CAG cells (see TS 38.304 [50] for how the UE identifies whether a cell is a CAG cell);

Therefore, it might be helpful to clearly indicate PNI-NPN restricted IE as ENUMERATED(CAG-only, …) to align with the definition in TS 23.501 [6].

PNI-NPN restricted IE is suggested to be formatted as ENUMERATED(CAG-only, …).

3. Conclusion

In this paper, we have discussed issues on Mobility Restriction List, and have the following proposals,

- The maximum number of CAGs per PLMN in UE’s Allowed PNI-NPN list is 16.

PNI-NPN restricted IE is suggested to be formatted as ENUMERATED(CAG-only, …).

The corresponding TP is provided in Annex.

4. Reference

- R3-197592, (TP for NPN BL CR for TS 38.413): Mobility restriction list, Huawei

- R3-197593, Nokia, (TP for NPN BL CR for 38.423) Mobility Restriction List of NPN, Nokia Shanghai Bell

- draft v2 R3-19xxxx was R3-197824 was R3-197584 NPN 38413 BL.docx, Qualcomm Incorporated

- R3-197596, TP on paging aspects for baseline CR on NPN/NGAP (38.413 CR#0290), Qualcomm Incorporated

- R3-197595, [TP for XnAP BL CR] TP for Xn Paging, Ericsson

- 3GPP TS 23.501 V16.3.0 (2019-12)

Annex –TP for TS 38.413 (on the top of BL R3-200051)

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

9.3	Information Element Definitions

<Unchanged Text Omitted>

9.3.3.Y4	Allowed PNI-NPN List

This IE contains information on allowed UE mobility in PNI-NPN including allowed PNI-NPNs and whether the UE is allowed to access non-CAG cells cells for each PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Allowed PNI-NPN List |  | 1 |  |  |
| >Allowed PNI-NPN Item |  | 1..<maxnoofEPLMNs+1> |  |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  |
| >>PNI-NPN restricted | O |  | ENUMERATED( CAG-only, …) | Indicates that the UE may only access cells that support PNI-NPNs for this PLMN as in TS 23.501 [9]. |
| >>Allowed CAG list per PLMN |  | 1..<maxnoofCAGsperPLMN> |  |  |
| >>>>CAG ID | M |  | 9.3.3.Y2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs+1 | Maximum no. of equivalent PLMNs plus one serving PLMN. Value is 16. |
| maxnoofAllowedCAGsperPLMN | Maximum number of CAGs per PLMN in UE’s Allowed PNI-NPN list. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>