3GPP TSG-RAN3 Meeting #107-e	R3-200720

E-Meeting, 24 February – 6 March, 2020

Title: 	(TP for NPN BL CR for TS 38.473): Further support of NPN over F1

Source: 	Huawei, China Telecom

Agenda item:	16.2.6

Document for:	Discussion

1. Introduction

RAN#105 meeting made the following agreements on F1 interface for stand-alone NPN (SNPN) and public network integrated NPN (PNI-NPN) in [1].

SNPN

Over F1, exchange list of supported (PLMN ID, NID)s between DU and CU: DU configured [PLMN ID, NID] to CU, CU matching [PLMN ID, NID] received from 5GC and sending result to DU.

PNI-NPN

All CAG information configured in DU;

Over F1, need to signal cell supported list of CAG IDs from DU to CU

In this document we intend to include the agreements into the TP and discuss the following remaining issues on F1 interface.

- 7.1 PNI NPN

  - Need to signal HRN to CU?

  - Need to signal cell reserved for xx use from DU to CU?

  - Need of any signaling from CU to DU?

  - UE dedicated impact?

- 7.2 SNPN

  - UE dedicated impact? E.g. need to add selected (PLMN, NID) in F1 UE context setup?

2. Discussion

  - General aspects

Last RAN3#106 meeting agreed the following working assumption, which was captured into the Xn/NG BL CR.

- WA: introduce a CHOICE structure for CAG and SNPN (where applicable); further details to be refined

For F1 interface, the same structure can be used as well to align with NGAP and XnAP. For example, the DU can provide the {PLMN ID, NID} or CAG ID information to the CU over the F1 interface setup message in a structure format.

The CHOICE structure is introduced over F1 interface for CAG and SNPN.

  - SNPN

2.1.1 Interface Management Messages

As agreed above, the supported (PLMN, NID)s can be exchanged between the gNB-CU and the gNB-DU. There may exist abnormal cases. For example, the configured PLMN or NID of the gNB-DU is different from the PLMN or NID of the gNB-CU. Then during the F1 setup procedure, the gNB-CU shall respond a F1 SETUP FAILURE and appropriate cause value, where the cause value can be set as “SNPN(s) not supported”. Note that the cause value “PLMN not served by the gNB-CU” is already specified.

A new cause value “SNPN(s) not supported” should be introduced over F1, similar to the cause value “PLMN not served by the gNB-CU”.

For unified access control (UAC), the gNB-CU could send UAC assistance information for the gNB-DU to set the parameters which is broadcasted in SIB1. The UAC assistance information is configured per PLMN by the gNB-CU and then transmitted from the gNB-CU to the gNB-DU. Therefore, in order to configure the UAC parameters for an SNPN, the NID needs to be provided to gNB-DU together with PLMN ID to identify the SNPN when the combination of the serving PLMN ID and the NID identifies the SNPN.

The NID is provided from the gNB-CU to the gNB-DU for UAC assistance information in the Network Access Rate Reduction message.

In addition, a human-readable network name (HRNN) can be configured per NID in the scope of a PLMN ID. The agreement in RAN2#107 meeting on the HRNN of NPN was given in [3].

If HRNN are broadcast then the HRNN should a be broadcasted in a separate SIB (i.e. different from SIB1).

That is, the HRNN is not included in SIB1 but in Other SIB.  Therefore, when HRNN is configured in gNB-DU, it should be provided to gNB-CU on F1 interface to encode Other SIB.

The available human-readable network name per NID should be provided to the gNB-CU, e.g. in F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE message.

2.1.2 UE Context Management Messages

When the UE access is granted, the gNB-CU could send the selected NID to the gNB-DU, e.g. in the UE CONTEXT SETUP REQUEST message, which might be helpful since different NID may require different scheduling mechanism. Note that currently the Serving PLMN IE is already specified from the gNB-CU to the gNB-DU.

For the handover cases, the selected PLMN ID and NID of the source SNPN cell (i.e., SNPN SpCell) should be also notified to the target gNB-DU by gNB-CU, e.g., via UE CONTEXT SETUP REQUEST message. Note that for intra-gNB-DU handover, the handover between cells identified by different NIDs is not supported (i.e., handover between SNPNs). Even for the handover between cells identified by the same PLMN and NID (i.e., handover within SNPN), it is not necessary for the gNB-CU to inform the gNB-DU of the NID either, because the gNB-DU has the knowledge already.

The serving PLMN ID and the NID should be informed to the gNB-DU by the gNB-CU in UE CONTEXT SETUP REQUEST message.

2.2 PNI-NPN

2.2.1 Interface Management Messages

For PNI-NPN, the CU can provide its available CAG cell information to the DU due to the following reasons.

  - Following the same logic of SNPN above, the CU can provide its available CAG information to the DU, e.g., due to the OAM configuration, or other specific situations.

  - Currently the supported slices of gNB-CU may be updated to the gNB-DU via the gNB-CU configuration updated procedure. Hence, the CAG IDs of the gNB-CU can be updated as well. Otherwise, if the pre-configured PNI-NPN(s) of gNB-DU are not aligned with the gNB-CU so as to CAG ID(s), UE will access to the wrong network and result in rejection.

The available CAG cell information supported by the gNB-CU should be provided to the gNB-DU, e.g. via the following messages,

- F1 SETUP RESPONSE

- GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message

- GNB-CU CONFIGURATION UPDATE message.

Similar to the proposal 2 for SNPN, an appropriate cause value, e.g, “PNI-NPN(s) not supported”, could be provided for F1 SETUP FAILURE message.

A new cause value “PNI-NPN(s) not supported” shall be introduced over F1.

Also the human-readable network name (HRNN) can be configured per CAG ID in the scope of a PLMN ID.  Following the Proposal 4, the available HRNN per CAG ID should be also provided to gNB-CU.

The available human-readable network name per CAG ID should be provided to gNB-CU, e.g. in F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE message.

2.2.2 UE Context Management Messages

For the state transition of the UE from CM-IDLE to CM-CONNECTED, it is described in [2] as follows,

-	During transition from CM-IDLE to CM-CONNECTED, if the UE is accessing the 5GS via a CAG cell, the UE shall provide the selected CAG Identifier to NG-RAN and the NG-RAN shall provide the CAG Identifier to the AMF:

-	The AMF shall verify whether UE access is allowed by Mobility Restrictions:

-	If the CAG Identifier received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

-	If the CAG Identifier received from the NG-RAN is not part of the UE's Allowed CAG list, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE removes that CAG Identifier, if it exists, from its Allowed CAG list, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure; and

-	If the UE is accessing the network via a non-CAG cell and the UE's subscription contains an indication that the UE is only allowed to access CAG cells, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE updates its local configuration, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure.

It can be observed that, if the CAG only UE is accessing the network via a non-CAG cell, the AMF shall reject the NAS request. In case of the disaggregated gNB, the gNB-CU shall send the ERROR INDICATION message to the gNB-DU with a cause equals to “Release due to only CAG cells is allowed”

A new cause value “Release due to only CAG cells is allowed” is introduced for the rejection of the UE access over F1-C.

Regarding whether the gNB-CU needs to inform the selected CAG ID to the gNB-DU during the UE context procedure, as the gNB-CU knows the supported CAG IDs of the gNB-DU, as well as the allowed CAG ID list of the UE, the gNB-CU knows whether the gNB-DU can service the UE. On the other hand, the CAG ID is used only for access control, but not for e.g., RRM policy. Hence, there is no need to inform the selected CAG ID to the gNB-DU during the UE context procedure. Then we have the following proposals:

There is no need to provide the selected CAG ID to the gNB-DU during the UE context setup/modification procedure.

In addition, during UE initial access, the selected PLMN is reported by UE in RRC Setup Complete message. As the following process in [4], in RAN sharing, the gNB-DU may connect to an unsuitable gNB-CU before receiving the RRC Setup Complete message. The gNB-DUA sends the F1AP UE CONTEXT RELEASE REQUEST message to the gNB-CUA, including a Cause set to "PLMN not served by the CU".

Figure 8.11.2-1: UE Initial Access procedure and network sharing with multiple cell-ID broadcast

Similarly, as agreed in RAN2#108 meeting, the selected NID together with the selected PLMN to identify the selected SNPN are also reported by UE in RRC Setup Complete message.

1.	From RAN2 point of view there is no requirement for CAG ID to be included in RRC signalling at RRC connection establishment.

2.	For SNPN, include the SNPN ID in the RRCSetupComplete message. Stage 3 detalls are FFS

3.	For SNPN, there is no need to include SNPN ID in the RRCResumeComplete message since the UE context is known to the network.

4.	Send a LS to SA3 with Agreement#1 with SA2 and RAN3 in To.

Hence the gNB-DU connects to an unsuitable gNB-CU who does not support the selected SNPN (does not support either NID or PLMN, or both) before the gNB-DU receives the RRC Setup Complete message. Then the gNB-DU needs to send the F1AP UE CONTEXT RELEASE REQUEST message to the unsuitable gNB-CU, including a Cause set to "SNPN not served by the CU". Note that there has been the Cause "PLMN not served by the CU" in current spec. We have the following proposal,

Add Cause "NID not served by the CU" for F1AP UE CONTEXT RELEASE REQUEST message.

3. Conclusion

In this paper, we have discussed whether the general support of NPN over F1, and have the following proposals,

- The CHOICE structure is introduced over F1 interface for CAG and SNPN.

- A new cause value “SNPN(s) not supported” should be introduced over F1, similar to the cause value “PLMN not served by the gNB-CU”.

The NID is provided from the gNB-CU to the gNB-DU for UAC assistance information in the Network Access Rate Reduction message.

The available human-readable network name per NID should be provided to the gNB-CU, e.g. in F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE message.

The serving PLMN ID and the NID should be informed to the gNB-DU by the gNB-CU in UE CONTEXT SETUP REQUEST message.

The available CAG cell information supported by the gNB-CU should be provided to the gNB-DU, e.g. via the following messages,

- F1 SETUP RESPONSE

- GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message

- GNB-CU CONFIGURATION UPDATE message.

A new cause value “PNI-NPN(s) not supported” shall be introduced over F1.

The available human-readable network name per CAG ID should be provided to gNB-CU, e.g. in F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE message.

A new cause value “Release due to only CAG cells is allowed” is introduced for the rejection of the UE access over F1-C.

There is no need to provide the selected CAG ID to the gNB-DU during the UE context setup/modification procedure.

Add Cause "NID not served by the CU" for F1AP UE CONTEXT RELEASE REQUEST message.

The corresponding TP is provided in the following Annex.

4. Reference

- R3-194686, Way Forward with private networks, Nokia, Nokia Shanghai Bell

- 3GPP TS 23.501 V16.1.0 (2019-06)

- RAN2-107-Prague-chair-notes-2019-08-28-1830.docx

- 3GPP TS 38.401 V16.0.0

- R3-195772	Remaining issues on support of NPN over F1, Huawei, China Telecom

Annex – TP for TS 38.473 (on the top of agreed BL R3-197585)

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

CAG                    Closed Access Group

CN	Core Network

CG	Cell Group

CGI	Cell Global Identifier

CP	Control Plane

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

IMEISV 	International Mobile station Equipment Identity and Software Version number

NID	Network identifier

NPN	Non-Public Network

NSSAI	Network Slice Selection Assistance Information

PNI-NPN             Public Network Integrated NPN

RANAC	RAN Area Code

RRC	Radio Resource Control

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

HRNN                 Human-readable network name

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.2.3	F1 Setup

8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<Unchanged Text Omitted>

If the F1 SETUP RESPONSE message contains the Transport Layer Addresses Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

For NG-RAN, the gNB-DU may include the NID IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

For NG-RAN, the gNB-DU may include the CAG Id IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

For NG-RAN, the gNB-CU may include the NID IE in the F1 SETUP RESPONSE message, if the NID(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, the gNB-DU shall take this into account and only broadcast the received NID(s) included in the F1 SETUP RESPONSE message.

For NG-RAN, the gNB-CU may include CAG Id IE in the F1 SETUP RESPONSE message, if the CAG Id(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, the gNB-DU shall take this into account and only broadcast the CAG Id(s) included in the F1 SETUP RESPONSE message.

For NG-RAN, the gNB-DU may include the the HRNN IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

<Unchanged Text Omitted>

8.2.4	gNB-DU Configuration Update

8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<Unchanged Text Omitted>

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Addresses Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If NID IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall store and replace any previously provided NID IE by the received NID IE.

If CAG Id(s) are included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall store and replace any previously provided CAG Id(s) by the received CAG Id(s).

If HRNN(s) are included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall store and replace any previously provided HRNN(s) by the received HRNN(s).

<Unchanged Text Omitted>

8.2.5	gNB-CU Configuration Update

8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<Unchanged Text Omitted>

If the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Addresses Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If NID IE is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the NID(s) and update the corresponding system information.

If CAG Id(s) are contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole supported CAG Id(s) and update the corresponding system information.

<Unchanged Text Omitted>

8.2.9	Network Access Rate Reduction

8.2.9.1	General

The purpose of the Network Access Rate Reduction procedure is to indicate to the gNB-DU that the rate at which UEs are accessing the network need to be reduced from its current level.

The procedure uses non-UE associated signalling.

8.2.9.2	Successful operation

gNB

-

DU

NETWORK ACCESS RATE REDUCTION

gNB

-

CU

Figure 8.2.9.2-1: Network Access Rate Reduction, Successful operation

The gNB-CU initiates the procedure by sending a NETWORK ACCESS RATE REDUCTION message to the gNB-DU. When receiving the NETWORK ACCESS RATE REDUCTION message the gNB-DU should take into account the information contained in the UAC assistance information to set the parameters for Unified Access Barring.

If the NID IE is contained in the Network Access Rate Reduction message, the gNB-DU should take it into account and combine the NID IE with the PLMN Identity IE to identify the SNPN.

<Unchanged Text Omitted>

8.3	UE Context Management procedures

8.3.1	UE Context Setup

8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, and DRB configuration. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

<Unchanged Text Omitted>

If the Trace Activation IE is included in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, initiate the requested trace function as described in TS 32.422 [29].

If the Serving NID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall combine the Serving NID IE with the Serving PLMN IE to identify the serving NPN, and take it into account.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2	Message Functional Definition and Content

<Unchanged Text Omitted>

9.2.2	UE Context Management messages

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. | - |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >> UL Configuration | O |  | UL Configuraiton 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | - |  |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN ID 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| Trace Activation | O |  | 9.3.1.88 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Serving NID | O |  | 9.3.1.x1 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |

<Unchanged Text Omitted>

9.3	Information Element Definitions

<Unchanged Text Omitted>

9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the F1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, RL failure-RLC, Unknown or already allocated gNB-CU UE F1AP ID, Unknown or already allocated gNB-DU UE F1AP ID, Unknown or inconsistent pair of UE F1AP ID, Interaction with other procedure, Not supported QCI Value, Action Desirable for Radio Reasons, No Radio Resources Available, Procedure cancelled, Normal Release, ..., Cell not available, RL failure-others, UE rejection, Resources not available for the slice, AMF initiated abnormal release, Release due to Pre-Emption, PLMN not served by the gNB-CU, Multiple DRB ID Instances, Unknown DRB ID, only CAG cells allowed, NID not supported, CAG(s) not supported, NID not served by the CU) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, Transport Resource Unavailable, ...) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, ...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| RL Failure-RLC | The action is due to an RL failure caused by exceeding the maximum number of ARQ retransmissions. |
| Unknown or already allocated gNB-CU UE F1AP ID | The action failed because the gNB-CU UE F1AP ID is either unknown, or (for a first message received at the gNB-CU) is known and already allocated to an existing context. |
| Unknown or already allocated gNB-DU UE F1AP ID | The action failed because the gNB-DU UE F1AP ID is either unknown, or (for a first message received at the gNB-DU) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of UE F1AP ID | The action failed because both UE F1AP IDs are unknown, or are known but do not define a single UE context. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| Not supported QCI Value | The action failed because the requested QCI is not supported. |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. |
| Normal Release | The action is due to a normal release of the UE (e.g. because of mobility) and does not indicate an error. |
| Cell Not Available | The action failed due to no cell available in the requested node. |
| RL Failure-others | The action is due to an RL failure caused by other radio link failures than exceeding the maximum number of ARQ retransmissions. |
| UE rejection | The action is due to gNB-CU’s rejection of a UE access request. |
| Resources not available for the slice | The requested resources are not available for the slice. |
| AMF initiated abnormal release | The release is triggered by an error in the AMF or in the NAS layer. |
| Release due to Pre-Emption | Release is initiated due to pre-emption. |
| PLMN not served by the gNB-CU | The PLMN indicated by the UE is not served by the gNB-CU. |
| Multiple DRB ID Instances | The action failed because multiple instances of the same DRB had been provided. |
| Unknown DRB ID | The action failed because the DRB ID is unknow. |
| Only CAG cells are allowed | The action fails because the UE is only allowed to access CAG cells. |
| NID not supported | The action fails because no SNPN(s) is supported in the request node. |
| CAG(s) not supported | The action fails because no PNI-NPN(s) is supported in the request node. |
| NID not served by the CU | DU requests CU to release UE context because NID is not served by the CU. |

<Unchanged Text Omitted>

9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >NPN Support Information | O |  | 9.3.1.x5 | Supported NPNs per PLMN. | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per TA. | YES | ignore |
| CHOICE NR-Mode-Info | M |  |  |  | - |  |
| >FDD |  |  |  |  | - |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >TDD |  |  |  |  | - |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>> NR FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>> Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >>>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8]. | - |  |
| RANAC | O |  | RAN Area Code 9.3.1.57 |  | YES | ignore |
| Extended Served PLMNs List |  | 0..1 |  | This is included if more than 6 Served PLMNs is to be signalled. | YES | ignore |
| >Extended Served PLMNs Item |  | 1 ..<maxnoofExtendedBPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >>NPN Support Information | O |  | 9.3.1.x5 | Supported NPNs per PLMN. | - |  |
| >>TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per TA. | - |  |
| Cell Direction | O |  | 9.3.1.78 |  | YES | ignore |
| Cell Type | O |  | 9.3.1.87 |  | YES | ignore |
| Broadcast PLMN Identity Info List |  | 0..<maxnoofBPLMNsNR-1> |  | This IE corresponds to the PLMN-IdentityInfoList IE in SIB1 as specified in TS 38.331 [8]. The PLMN Identities and associated information contained in this IE is provided in the same order as broadcast in SIB1. | YES | ignore |
| >PLMN Identity List | M |  | Available PLMN List 9.3.1.65 |  | - |  |
| >Extended PLMN Identity List | O |  | Extended Available PLMN List 9.3.1.76 |  | - |  |
| >5GS-TAC | O |  | OCTET STRING (3) |  | - |  |
| >NR Cell Identity | M |  | BIT STRING (36) |  | - |  |
| >RANAC | O |  | RAN Area Code 9.3.1.57 |  | - |  |
| Aggressor gNB Set ID | O |  | 9.3.1.93 | This IE indicates the associated aggressor gNB Set ID of the cell | YES | ignore |
| Victim gNB Set ID | O |  | 9.3.1.93 | This IE indicates the associated Victim gNB Set ID of the cell | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR-1 | Maximum no. of PLMN Ids.broadcast in an NR cell minus 1. Value is 11. |

<Unchanged Text Omitted>

9.3.1.65	Available PLMN List

This IE indicates the list of available PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available PLMN Item IEs |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.3.1.14 |  |
| >Available NPN List | O |  | 9.3.1.x5 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |

<Unchanged Text Omitted>

9.3.1.76	Extended Available PLMN List

This IE indicates the list of available PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Available PLMN Item IEs |  | 1..< maxnoofExtendedBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.3.1.14 |  |
| >Available NPN List | O |  | 9.3.1.x5 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |

<Unchanged Text Omitted>

9.3.1.83	UAC Assistance Information

This information element contains assistance information helping the gNB-DU to set parameters for Unified Access Class barring.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UAC PLMN List |  | 1 |  |  |
| >UAC PLMN Item |  | 1..<maxnoofUACPLMNs> |  |  |
| >>PLMN Identity | M |  | 9.3.1.14 |  |
| >>UAC Type List |  | 1 |  |  |
| >>>UAC Type Item |  | 1..<maxnoofUACperPLMN> |  |  |
| >>>>UAC Reduction Indication | M |  | 9.3.1.85 |  |
| >>>>CHOICE UAC Category Type | M |  |  |  |
| >>>>>UAC Standardized |  |  |  |  |
| >>>>>> UAC Action | M |  | 9.3.1.84 |  |
| >>>>>UAC Operator Defined |  |  |  |  |
| >>>>>>Access Category | M |  | INTEGER (32..63, …) | Indicates the operator defined Access Category as defined in subclause 6.3.2 in TS 38.331 [8]. |
| >>>>>>Access Identity | M |  | BIT STRING (SIZE(7)) | Indicates whether access attempt is allowed for each Access Identity as defined in subclause 6.3.2 in TS 38.331 [8]. |
| >>NID | O |  | 9.3.1.x1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUACPLMNs | Maximum no. of UAC PLMN Ids. Value is 12. |
| maxnoofUACperPLMN | Maximum no. of signalled categories per PLMN. Value is 64. |

<Unchanged Text Omitted>

9.3.1.x1	NID

This IE is used to identify (together with a PLMN identifier) a Stand-alone Non-Public Network. The NID is specified in TS 23.003 [22].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NID | M |  | OCTET STRING (SIZE(7)) | Editor’s Note: Coding and semantics are FFS. |

9.3.1.x2	CAG-Identifier

This IE is used to identify (together with a PLMN identifier) a Public Network Integrated Non-Public Network. The CAG-Identifier is specified in TS 23.003 [22].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CAG-Identifier | M |  | BIT STRING (SIZE(32)) | Editor’s Note: Coding is FFS. |

9.3.1.x3	SNPN ID List

This IE contains a list of SNPN IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SNPN ID Item IEs |  | 1..<maxnoofSNPNIDs> |  |  |
| >NID | M |  | 9.3.1.x1 |  |
| >HRNN | O |  | OCTET STRING | Corresponds to the HRNN as defined in TS 38.331 [8]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSNPNIDs | Maximum no. of SNPN IDs broadcast in a cell. Value is 12. |

9.3.1.x4	PNI-NPN ID List

This IE contains a list of PNI-NPN IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PNI-NPN ID Item IEs |  | 1..<maxnoofCAGs> |  | Broadcast PLMNs |
| >CAG-Identifier | M |  | 9.3.1.x2 |  |
| >HRNN | O |  | OCTET STRING | Corresponds to the HRNN defined in TS 38.331 [8]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCAGs | Maximum no. of CAG-Identifiers broadcast in a cell. Value is 12. |

9.3.1.x5	NPN Support Information

This IE contains NPN related broadcast information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NPN Support Information | M |  |  |  |
| >SNPN Support Information |  |  |  |  |
| >> SNPN ID List | M |  | 9.3.1.x3 |  |
| >PNI-NPN Support Information |  |  |  |  |
| >> PNI-NPN ID List | M |  | 9.3.1.x4 |  |

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>