3GPP TSG-RAN WG3 Meeting #107e	R3-200842

# Online, 24th – 28th February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1756 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNB node identification in case it is associated with more than one PLMN ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There have been discussions in RAN3 revealing that there is no common understanding on how eNBs are identified in case it serves cells with more than one PLMN ID associated to them. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the abnormal conditions of the S1 Setup procedure, it is specified, that in case the PLMN Identity in the Global eNB ID does not match with any of the PLMNs supported by the MME, but at least one PLMN is supported by both, the MME and the eNB, a subsequent eNB Configuration Update procedure is triggered - The definition of the Global eNB IE is amended by the following statement: Global identification of an eNB node that serves cells associated with more than one PLMN Identity is specified in TS 36.300 [14] - An equivalent statement is provided for the Global en-gNB ID. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The definition of the Global eNB ID has no impact with the previous version of the specification for implementations following the statement introduced by this CR. The modification of the abnormal condition of the S1 Setup procedure has isolated impact with the previous version of the specification (same release) as it concerns the S1 Setup procedure only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would be still no common understanding how to identify eNBs if more than one PLMN ID is associated with it. It would be unclear how to deal with an eNB ID during S1 Setup indicating a PLMN not supported by MME. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.3.4, 9.2.1.37, 9.2.1.37a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.300 CR ... Rel-15 TS 36.300 CR ... Rel-16 TS 36.413 CR1757 Rel-16 TS 36.423 CR1457 Rel-15 TS 36.423 CR1458 Rel-16. |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.7.3	S1 Setup

#### 8.7.3.1	General

The purpose of the S1 Setup procedure is to exchange application level data needed for the eNB and the MME to correctly interoperate on the S1 interface. This procedure shall be the first S1AP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received and clears MME overload state information at the eNB. If the eNB and MME do not agree on retaining the UE Contexts this procedure also re-initialises the E-UTRAN S1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do. If the eNB initiating the S1 Setup procedure supports a CSG cell, the procedure shall report the CSG ID(s) of the supported CSGs.

#### 8.7.3.2	Successful Operation

Figure 8.7.3.2-1: S1 Setup procedure: Successful Operation.

The eNB initiates the procedure by sending a S1 SETUP REQUEST message including the appropriate data to the MME. The MME responds with a S1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used for the duration of the TNL association. When this procedure is finished, the S1 interface is operational and other S1 messages can be exchanged.

If the eNB initiating the S1 SETUP procedure supports one (or more) CSG cell(s), the S1 SETUP REQUEST message shall contain the CSG ID(s) of the supported CSG(s).

If the S1 SETUP REQUEST message contains the eNB Name IE the MME may use this IE as a human readable name of the eNB.

If the S1 SETUP RESPONSE message contains the MME Name IE the eNB may use this IE as a human readable name of the MME.

If the MME Relay Support Indicator IE is included in the S1 SETUP RESPONSE message, the eNB shall consider this information when selecting an appropriate MME for the Relay Node.

If the UE Retention Information IE set to “ues-retained“ was included in the S1 SETUP REQUEST message, then the MME may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the S1 SETUP RESPONSE message.

If the NB-IoT Default Paging DRX IE is included in the S1 SETUP REQUEST message, the MME will take it into account as specified in TS36.300 [14].

If the Connected en-gNB List IE is included in the S1 SETUP REQUEST message, the MME shall take it into account as specified in TS 36.300 [14].

If the S1 SETUP RESPONSE message contains the ServedDCNs IE then the eNB shall, if supported, use it as defined in TS 23.401 [11].

#### 8.7.3.3	Unsuccessful Operation

Figure 8.7.3.3-1: S1 Setup procedure: Unsuccessful Operation.

If the MME cannot accept the setup, it should respond with a S1 SETUP FAILURE and appropriate cause value.

If the S1 SETUP FAILURE message includes the Time To Wait IE, the eNB shall wait at least for the indicated time before reinitiating the S1 setup towards the same MME.

#### 8.7.3.4	Abnormal Conditions

If the eNB initiates the procedure by sending a S1 SETUP REQUEST message including the PLMN Identity IEs and none of the PLMNs provided by the eNB is identified by the MME, then the MME shall reject the eNB S1 Setup Request procedure with the appropriate cause value, e.g., “Unknown PLMN”.

Interaction with the eNB Configuration Update procedure

If the S1 SETUP REQUEST message includes a Global eNB ID containing a PLMN Identity not supported by the MME, but the eNB and the MME support at least one common PLMN Identity, the S1 Setup procedure shall be terminated successfully and the eNB shall  trigger the eNB Configuration Update procedure including the Global eNB ID IE containing a PLMN Identity supported by the MME.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.37	Global eNB ID

This information element is used to globally identify an eNB (see TS 36.401 [2]). Global identification of an eNB that serves cells associated with more than one PLMN Identity is specified in TS 36.300 [14].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.3.8 |  |
| CHOICE eNB ID | M |  |  |  |
| >Macro eNB ID |  |  |  |  |
| >>Macro eNB ID | M |  | BIT STRING (SIZE(20)) | Equal to the 20 leftmost bits of the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of each cell served by the eNB. |
| >Home eNB ID |  |  |  |  |
| >>Home eNB ID | M |  | BIT STRING (SIZE(28)) | Equal to the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of the cell served by the eNB. |
| >Short Macro eNB ID |  |  |  |  |
| >> Short Macro eNB ID | M |  | BIT STRING (SIZE(18)) | Equal to the 18 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB. |
| >Long Macro eNB ID |  |  |  |  |
| >> Long Macro eNB ID | M |  | BIT STRING (SIZE(21)) | Equal to the 21 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB. |

#### 9.2.1.37a	Global en-gNB ID

This information element is used to globally identify an en-gNB (see TS 36.401 [2]).  Global identification of an en-gNB that serves cells associated with more than one PLMN Identity follows the same principles as specified for the global identification of an eNB as specified in TS 36.300 [14].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.3.8 |  |
| en-gNB ID | M |  | BIT STRING (SIZE(22..32)) |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>