3GPP TSG-RAN3 #107-e	R3-200949

24th February – 6th March 2020

Online

Agenda Item:	10.2.1.2

Source:	Ericsson

Title:	Examination of FFSs in SON BL CR for TS 36.413

Document for:	Approval

# Introduction

At the last meeting a SON BL CR for TS 36.413 was agreed in R3-197672. In the following we will look into the FFS in the agreed document and provide our answers.

# Discussion

The first FFS concerns the nrofSS-BlocksToAverage IE. Looking into the semantics description and also as stated in TS 36.331, we see that it is the same as maxRS-IndexCellQual IE. Based on that we propose to remove nrofSS-BlocksToAverage IE.

- It is proposed that nrofSS-BlocksToAverage IE is removed

The next FFS concerns SMTC. SMTC shall be Optional to be in line with the TS36.331 description.

- It is proposed that SMTC shall be Optional

The next FFS is about ThresholdNR. Our view is that ThresholdNR shall be optional because if maxRS-IndexCellQual has only one SSB, the threshold is not used (threshold is used to select further SSBs to be included in the average, if maxRS-IndexCellQual has more than 1 SSB – see TS38.331). Also we note that the name should be corrected according to TS 36.331 and thus be changed to threshRS-Index-r15

- It is proposed that ThresholdNR shall be optional and also that its name is changed into threshRS-Index-r15

The next FFS concerns SS-RSSI-Measurement. SS-RSSI-Measurement is necessary to deduce RSRQ. The IE SS-RSSI-Measurement specifies the configuration of NR SSB based RSSI measurements as described in TS 38.331.

- It is proposed that SS-RSSI-Measurement is included

Next we look into quantityConfigIndex. This IE is needed to filter P-RSRQ SINR measurements (L3 filtering), so it should be included. Also, we note that the name should be corrected according to TS 36.331 and thus be changed to quantityConfigSet

- It is proposed that quantityConfigIndex is included and also that the name is changed to quantityConfigSet.

Following we discuss blackCellsToAddList. This IE, included also in TS. 36.331, is needed because otherwise unnecessary HO will be deduced from wrong measurements associated to cells that are not available to the UE but that are measurable. Also based on TS. 36.331, it should be renamed to blackCellsToAddModList.

- It is proposed that blackCellsToAddList is included and is renamed to blackCellsToAddModList.

# Conclusions

In this paper the FFS in the SON BL CR for TS 36.413 were considered and the following proposals were derived:

Proposal 1.	It is proposed that nrofSS-BlocksToAverage IE is removed

Proposal 2.	It is proposed that SMTC shall be optional

Proposal 3.	It is proposed that ThresholdNR shall be optional and also that its name is changed into threshRS-Index-r15

Proposal 4.	It is proposed that SS-RSSI-Measurement is included

Proposal 5.	It is proposed that quantityConfigIndex is included and also that its name is changed to quantityConfigSet.

Proposal 6.	It is proposed that blackCellsToAddList is included and is renamed to blackCellsToAddModList.

TP capturing the proposals above is provided below.

# TP for SON BL CR for TS 36.413

### ============ Start of 1st change ==============

#### 9.2.1.X1	Inter System Measurement Configuration

The Inter-System Measurement Configuration IE contains information for instructing the incoming UE to continue measuring the cells of the NR RAT after a successful inter-system handover to LTE network.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RSRP | O |  | INTEGER (0.. 127) | Threshold of RSRP. | - | - |
| RSRQ | O |  | INTEGER (0.. 127) | Threshold of RSRQ. | - | - |
| SINR | O |  | INTEGER (0..127) | Threshold of SINR | - | - |
| Inter-System Measurement Parameters | M |  |  |  | - | - |
| >Measurement Duration | M |  | INTEGER (1..100) | The period of time following the successful IRAT handover, during which the target RAT instructs the UE to measure cells of the source RAT. Unit: [second]. | - | - |
| >Inter-System Measurement List |  | 0..1 |  |  | EACH | reject |
| >>Inter-System Measurement Item |  | 1.. <maxnooffrequencies> |  | List of inter-system measurements configured | YES | reject |
| >>>FreqBandIndicatorNR | M |  | INTEGER (1..1024) | The frequency band in which the ssbFrequency is located and according to which the UE shall perform the RRC measurements. |  |  |
| >>>SSB frequencies | M |  | INTEGER (0..maxNARFCN) | Designates the specific SSB frequencies i.e., ARFCN-ValueNR which the target RAT may instruct the UE to measure. | - | - |
| >>>SubcarrierSpacingSSB | M |  | ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1} | Subcarrier spacing of SSB according to TS 36.331 [16]. |  |  |
| >>>maxRS-IndexCellQual | O |  | INTEGER (1..maxRS-IndexCellQual) | Indicates the maximum number of RS indices to be considered/ averaged to derive the cell quality for RRM.Also defined in TS 36.331 [16] |  |  |
| >>>SMTC | O |  | OCTET STRING | Contains the MTC-SSB-NR-15 as defined in TS 36.331 [16]. |  |  |
| >>> threshRS-Index-r15 | O |  | OCTET STRING | threshRS-Index-r15 as defined in TS 36.331 [16]，List of thresholds for consolidation of L1 measurements per RS index |  |  |
| >>>SSB-ToMeasure | O |  | OCTET STRING | Contains the IE SSB-ToMeasure as defined in TS 36.331 [16]. |  |  |
| >>>SS-RSSI-Measurements | O |  | OCTET STRING | Contains the IE SS-RSSI-Measurements as defined in TS 36.331 [16]. |  |  |
| >>>quantityConfigSet | O |  | INTEGER (1..maxNrofQuantityConfig) | Indicates the n-th element of quantityConfigNR-List provided in MeasConfig. As defined in TS 36.331 [16]. |  |  |
| >>>blackCellsToAddModList | O |  |  | List of cells to add/modify in the black list of cells. It applies only to SSB resources. As defined in TS 36.331 [16]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnooffrequencies | Maximum no. of frequencies. Value is FFS. |
| maxNARFCN | Maximum value of NR carrier frequency, defined in TS 38.331 [r1]. Value is 3279165 |
| maxNrofSS-BlocksToAverage | Max number for the number of SS blocks to average to determine cell measurement, defined in TS 38.331 [r1]. Value is 16. |
| maxRS-IndexCellQual | Maximum number of RS indices averaged to derive cell quality for RRM, defined in TS 36.331 [16]. Value is 16 |
| maxNrofQuantityConfig | Maximum number of quantity configurations, defined in TS 38.331 [r1]. Value is 2. |