3GPP TSG-RAN WG3 #107-e	R3-201019

# Online, 24th February - 06th March 2020

Agenda Item:	19.3

Source: 	Ericsson

Title:  	Reporting of UL AoA beam info to LMF

Document for:	Discussion and approval

# Introduction

RAN2 has recently agreed to include spatial direction information of the DL-PRS Resources in the position calculation assistance data (e.g., azimuth, elevation). In TS 38.331, the IE DL-PRS-Beam-Info includes the azimuth and elevation angles of each DL-PRS Resource (beam) defined in the same way as for AoA.

For UE-based positioning, a split representation in an integer representation part and a differential deci-degree part of the beam direction info is beneficial for AoA representation. This will benefit UE-based Assistance Data, since the assistance data can be subject to broadcast and it can be necessary that the ASN.1 info is properly encoded.

In the RAN2 e-mail discussion about UE-based downlink positioning assistance data [1], the following representation was proposed:

-- ASN1START

# NR-PRS-Beam-Info-r16 ::= SEQUENCE (SIZE(0..255)) OF NR-PRS-BeamInfoPerTRP-r16

NR-PRS-BeamInfoPerTRP-r16 ::= SEQUENCE (SIZE(0..1)) OF NR-PRS-Beam-Info-ResourceSet-r16

NR-PRS-Beam-Info-ResourceSet-r16 ::= SEQUENCE {

nr-PRS-BeamInfoList-r16			SEQUENCE (SIZE(0..63)) OF NR-PRS-BeamInfoElement-r16

lcs-To-GCS-Translation           SEQUENCE {

alpha                            INTEGER (0..359),

alpha-fine                       INTEGER (0..9),      OPTIONAL,  -- Need OP

beta                             INTEGER (0..359)     OPTIONAL,  -- Need OP

beta-fine                        INTEGER (0..9)       OPTIONAL,  -- Need OP

gamma                            INTEGER (0..359)     OPTIONAL   -- Need OP

gamma-fine                       INTEGER (0..9)       OPTIONAL   -- Need OP

},                                                       OPTIONAL,  -- Need OP

...

}

NR-PRS-BeamInfoElement-r16 ::= SEQUENCE {

nr-PRS-Azimuth-r16				INTEGER (0..359),

nr-PRS-Azimuth-fine-r16				INTEGER (0..9),				OPTIONAL,	-- Cond FineAngles

nr-PRS-Elevation-r16			INTEGER (0..180)				OPTIONAL,	-- Need ON

nr-PRS-Elevation-fine-r16			INTEGER (0..9)				OPTIONAL,	-- Cond FineAngles

...

}

-- ASN1STOP

In this paper, we propose to use the same angle reporting via NRPPa for AoA estimates (highlighted above) with the split representation mentioned above. The corresponding information elements will be added to the Positioning Information Response message sent from gNB to LMF.

# Reference

Summary of [108#89][NR/Pos] UE-based downlink positioning assistance data, Qualcomm Inc.

# TP to NRPPa BL CR: Addition of AoD reporting to LMF

START OF CHANGE

### 8.2.x	Positioning Information Exchange

#### 8.2.x.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN NODE positioning information for the UE.

Editor’s Note: It is FFS whether a single procedure is used for UE associated measurements and non-UE associated information exchange, or whether these are separated into two procedures. As currently the procedure describes UE measurements, and is UE-associated,  if it stays as such, then it should be renamed to Positioning Measurement procedure.

#### 8.2.x.2	Successful Operation

Figure 8.2.x.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE in the POSITIONING INFORMATION RESPONSE message.

If the NR-PRS Beam Information IE is included in the POSITIONING INFORMATION RESPONSE message, the NG-RAN node may take this information into account when configuring AoA assistance data for the UE-based positioning,

[Editor’s Note: further details on the IEs are FFS / pending to RAN2]

NEXT CHANGE

#### 9.1.1.b	POSITIONING INFORMATION RESPONSE

This message is sent by NG-RAN node to provide positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| NR-PRS Beam Information | O |  | 9.2.xx |  | YES | ignore |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

NEXT CHANGE

### 9.2.xx	NR-PRS Beam Information

This IE contains spatial direction information of the DL-PRS Resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR-PRS Beam Information |  | 1 .. <maxnoNRPRSBeamtypes> |  |  |
| >NR PRS Azimuth | O | I | INTEGER (0,..,359) |  |
| >NR PRS Azimuth fine | O |  | INTEGER (0,..,9) | Fine angles |
| >NR PRS Elevation | O |  | INTEGER (0,..,180) |  |
| >NR PRS Elevation fine | O |  | INTEGER (0,..,9) | Fine angles |

| Range bound | Explanation |
| --- | --- |
| maxnoNRPRSBeamtypes | Maximum no. of NR PRS beam information types that can reported with one message. Value is 64. |

NEXT CHANGE

### 9.3.4 PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for NRPPa.

--

-- **************************************************************

NRPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

E-CID-MeasurementResult,

OTDOACells,

OTDOA-Information-Item,

Measurement-ID,

MeasurementPeriodicity,

MeasurementQuantities,

ReportCharacteristics,

RequestedSRSTransmissionCharacteristics,

Cell-Portion-ID,

OtherRATMeasurementQuantities,

OtherRATMeasurementResult,

WLANMeasurementQuantities,

WLANMeasurementResult,

Assistance-Information,

Broadcast,

AssistanceInformationFailureList,

SRSConfiguration,

UL-RTOAMeasurement,

NRPRSBeamInfo

FROM NRPPA-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-Single-Container{},

NRPPA-PRIVATE-IES,

NRPPA-PROTOCOL-EXTENSION,

NRPPA-PROTOCOL-IES

FROM NRPPA-Containers

maxnoOTDOAtypes,

id-Cause,

id-CriticalityDiagnostics,

id-LMF-UE-Measurement-ID,

id-OTDOACells,

id-OTDOA-Information-Type-Group,

id-OTDOA-Information-Type-Item,

id-ReportCharacteristics,

id-MeasurementPeriodicity,

id-MeasurementQuantities,

id-RAN-UE-Measurement-ID,

id-E-CID-MeasurementResult,

id-RequestedSRSTransmissionCharacteristics,

id-Cell-Portion-ID,

id-OtherRATMeasurementQuantities,

id-OtherRATMeasurementResult,

id-WLANMeasurementQuantities,

id-WLANMeasurementResult,

id-Assistance-Information,

id-Broadcast,

id-AssistanceInformationFailureList,

id-SRSConfiguration,

id-UL-RTOAMeasurement,

id-NRPRSBeamInfo

FROM NRPPA-Constants;

UNCHANGED PART OMITTED

-- **************************************************************

--

#### -- POSITIONING INFORMATION RESPONSE

--

-- **************************************************************

PositioningInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationResponse-IEs}},

...

}

PositioningInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ id-NRPRSBeamInfo					CRITICALITY ignore	TYPE NRPRSBeamInfo				PRESENCE optional},

...

}

NEXT CHANGE

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

maxCellinRANnode,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-OtherRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxWLANchannels,

maxnoFreqHoppingBandsMinusOne,

id-TDD-Config-EUTRA-Item,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs,

maxNrOfNRPRSBeamtypes

UNCHANGED PART OMITTED

#### -- N

NRPRSBeamInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NRPRSBeamInfo ::= SEQUENCE (SIZE (1..maxNrOfNRPRSBeamtypes)) OF NRPRSBeamInfoItem

NRPRSBeamInfoItem ::= CHOICE {

nr-PRS-Azimuth				INTEGER (0..359),			OPTIONAL,

nr-PRS-Azimuth-fine			INTEGER (0..9),				OPTIONAL,

nr-PRS-Elevation-			INTEGER (0..180)			OPTIONAL,

nr-PRS-Elevation-fine		INTEGER (0..9)				OPTIONAL,

...

...

}

UNCHANGED PART OMITTED

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellinRANnode							INTEGER ::= 3840

maxNoMeas									INTEGER ::= 63

maxCellReport								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxGERANMeas								INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNrOfNRPRSBeamtypes						INTEGER ::= 64

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-LMF-UE-Measurement-ID										ProtocolIE-ID ::= 2

id-ReportCharacteristics										ProtocolIE-ID ::= 3

id-MeasurementPeriodicity										ProtocolIE-ID ::= 4

id-MeasurementQuantities										ProtocolIE-ID ::= 5

id-RAN-UE-Measurement-ID										ProtocolIE-ID ::= 6

id-E-CID-MeasurementResult										ProtocolIE-ID ::= 7

id-OTDOACells													ProtocolIE-ID ::= 8

id-OTDOA-Information-Type-Group									ProtocolIE-ID ::= 9

id-OTDOA-Information-Type-Item									ProtocolIE-ID ::= 10

id-MeasurementQuantities-Item									ProtocolIE-ID ::= 11

id-RequestedSRSTransmissionCharacteristics						ProtocolIE-ID ::= 12

id-Cell-Portion-ID												ProtocolIE-ID ::= 14

id-OtherRATMeasurementQuantities								ProtocolIE-ID ::= 15

id-OtherRATMeasurementQuantities-Item							ProtocolIE-ID ::= 16

id-OtherRATMeasurementResult									ProtocolIE-ID ::= 17

id-WLANMeasurementQuantities									ProtocolIE-ID ::= 19

id-WLANMeasurementQuantities-Item								ProtocolIE-ID ::= 20

id-WLANMeasurementResult										ProtocolIE-ID ::= 21

id-TDD-Config-EUTRA-Item										ProtocolIE-ID ::= 22

id-Assistance-Information										ProtocolIE-ID ::= xx

id-Broadcast													ProtocolIE-ID ::= xy

id-AssistanceInformationFailureList								ProtocolIE-ID ::= xz

id-SRSConfiguration												ProtocolIE-ID ::= yx

id-UL-RTOAMeasurement											ProtocolIE-ID ::= yy

id-NRPRSBeamInfo												ProtocolIE-ID ::= aa

END

-- ASN1STOP

END OF CHANGES