3GPP TSG-RAN WG3 #107-e	R3-201044

# Online, 24th February - 06th March 2020

Agenda Item:	14.3.1

Source:	Ericsson

Title: 	Finalizing discussion on LTE-M indicator in NGAP

Document for:	Discussion and Approval

# 1	Introduction

In Last meeting, the addition of the LTE-M indicator IE to NGAP BL CR #0172 on eMTC UE connected to 5GS was approved [1].

A FFS mark remained concerning whether the new LTE-M IE needs to be defined in a dedicated IE tabular description, or only in the INITIAL UE MESSAGE tabular description with its codepoints, since it is only called for there. By wandering through the NGAP specification, we remark that there are several cases of an IE being defined only in one procedure tabular and having its own dedicated tabular description afterwards (for instance, the Assistance Data for Paging in the NG Paging message, defined in 9.3.1.69 as well as other paging related IEs, etc.)

Anyways, the vital aspect for the specification is to have a proper ASN.1 definition of the IE. The FFS mark for this issue does not really constitute an issue.

Proposal 1: remove the FFS mark on the LTE-M indicator IE.

As a reminder, this LTE-M indicator was requested from SA2 in order to enable LTE-M identification from Initial UE message when sent to the AMF, before the UE context and the user plane is set up. During the discussion, other views were proposed, among them:

- add the LTE-M indicator in the Handover Request procedure [2];

- use the same mechanism for LTE-M identification for EPC [3].

The first point for support during Handover request is not deemed necessary, because the only new aspect asked by SA2 was that LTE-M indicator is added in Msg5, which was approved by RAN2. Therefore, the only RAN3 impact is to enable the control during the initial access by adding the new LTE-M indicator IE in the Initial UE Message. Any other case is supposed already handled as in legacy by some existing mechanism. No more over-interpretation is needed here.

For the second point, such discussion should have occurred in RAN2 and/or SA2 first, and since there was little traction on this topic and it was not discussed in SA2, then no further action is required from us that would allow supporting the same functionality for EPC as well.

Besides, considering that no more action was requested from RAN3 after two SA2 meetings, that after our reply RAN3 have not received further LS on this topic and that RAN3 are on the verge of finishing Rel-16 work, it is proposed to close the discussion on LTE-M indicator in 5GS.

Proposal 2: RAN3 to conclude the discussion on LTE-M indicator for 5GS; no more action is further required from us.

# 3	Conclusion

Proposal 1: remove the FFS mark on the LTE-M indicator IE.

Proposal 2: RAN3 to conclude the discussion on LTE-M indicator for 5GS; no more action is further required from us.

# 4	References

R3-200038, BL CR#1072, “Introduction of eMTC connected to 5GC”

R3-196562, Support of LTE-M Identification in 5GC (Nokia, Nokia Shanghai Bell)

R3-197308, (TP for BL CR 0172 for TS 38.413): LTE-M Indication in NGAP Initial UE Message (Ericsson)

# 5	References

START OF CHANGES

#### 9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | reject |
| RRC Establishment Cause | M |  | 9.3.1.111 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | reject |
| AMF Set ID | O |  | 9.3.3.12 |  | YES | ignore |
| UE Context Request | O |  | ENUMERATED (requested, ...) |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 |  | YES | reject |
| Source to Target AMF Information Reroute | O |  | 9.3.3.27 |  | YES | ignore |
| CE-mode-B Support Indicator | O |  | 9.3.1.xxz |  | YES | reject |
| LTE-M Indication | O |  | 9.3.1.xx |  | YES | ignore |

NEXT CHANGE

#### 9.3.1.xx	LTE-M Indication

This element is provided by the NG-RAN node to inform that the UE indicates category M1 or M2 in its UE Radio Capability.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTE-M Indication | M |  | ENUMERATED (LTE-M, ...) |  |

END OF CHANGES