3GPP TSG-RAN3 Meeting #107-e	R3-201065

E-Meeting, 24 February – 6 March, 2020

Title: 	Normal inter-RAT HO from SA to EN-DC

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

1. Introduction

According to the LS [1] from RAN2, RAN2 has supported the NR SA to EN-DC handover in Rel-16.

RAN2 discussed the support of inter-RAT HO from NR SA to EN-DC in Rel-16 and agreed that: - The NR SA to EN-DC handover shall be supported in Rel-16 - Assume a new UE capability is needed RAN2 kindly asks all the involved groups to take above RAN2 decisions into account and update specifications accordingly if any.

In this contribution, we will discuss whether there are any RAN3 impacts to support the basic procedure of handover from NR SA to EN-DC.

2. Discussion

RAN2 discussed the basic procedure of handover from NR SA to EN-DC, where the procedure is shown in Figure 1.

Figure 1  Procedure of handover from NR to EN-DC

The procedure of handover from NR to EN-DC uses the procedure of handover from NR to E-UTRAN. Currently all signalling procedures are already supported by RAN3 specifications.

During this procedure, the eNB can configure the NR SCG in the LTE RRCConnectionReconfiguration. From RAN3’views, the LTE RRCConnectionReconfiguration is included within the Target to Source Transparent Container of the handover request ACK and the Handover command.

In addition RAN2 may introduce a new UE capability for it. From RAN3’views, the UE capability is included within the Source to Target Transparent Container of the handover required and the handover request.

Therefore there is no RAN3 impact to support the basic procedure of handover from NR to EN-DC.

Observation 1: In order to support the NR SA to EN-DC handover, the NR SCG configuration in LTE RRCConnectionReconfiguration and new UE capability are within RAN2’s scope.

Proposal 1:  There is no RAN3 impact to support the basic handover procedure from NR SA to EN-DC.

4. Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: In order to support the NR SA to EN-DC handover, the NR SCG configuration in LTE RRCConnectionReconfiguration and new UE capability are within RAN2’s scope.

Proposal 1:  There is no RAN3 impact to support the basic handover procedure from NR SA to EN-DC.

5. Reference

- R2-1916600, LS on inter-RAT HO from SA to EN-DC, RAN2