3GPP TSG-RAN WG3 Meeting #107-e	R3-201079

24 February-6 March 2020

E-meeting

Agenda Item:	15.3.1.3

Source:	Ericsson

Title:	CHO - Modification from target node

Document for:	Discussion and Decision

# Introduction

During RAN3#106, the target-initiated modification of an already prepared CHO was discussed. In this contribution the different cases and solutions are discussed.

# Discussion

Target-initiated modification of an already prepared CHO was discussed during many meetings, with a limited set of use-cases identified. However, a majority of companies were in favour of supporting at least some kind of modification, such as CFRA resources redistribution. Therefore, and in order to close this topic, it is proposed to support this possibility. However, the use-cases being limited (the CHO can always be cancelled by the target in case the resources are not available anymore), the impact on the standard and on the feature should be limited. A new procedure does not fulfill this target. But another option, discussed during RAN3#106, would be to reuse the existing cancellation procedure (from target to source) to inform the source that new resources can be reattributed to the UE by the target. This information could be contained in a new cause value, which seems to be the best way forward.

Proposal 1: Add a new cause value for the candidate target node to inform the source node that new resources are available for that UE

# Conclusion

### In this contribution the target-initiated modification of an already prepared CHO was discussed and the following proposal was made:

Proposal 1: Add a new cause value for the candidate target node to inform the source node that new resources are available for that UE

It is therefore proposed to agree the associated X2AP and XnAP TPs