3GPP TSG-RAN WG3 Meeting #107-e	R3-201103

Electronic Meeting, February 24th – March 6th, 2020

Agenda item:		15.2.1

Source:	Intel Corporation

Title:	(TP for LTE_feMob-Core BL CR for TS 36.300): Support for per DRB DAPS

Document for:		Other

# 1.	Introduction

In this contribution, we address necessary changes to support per DRB DAPS that RAN2 finally agreed.

# 2		Discussion

Given per DRB DAPS, the first question is whether to encode the indicator in X2/Xn or by RRC. Yes, both works and the source can read the transparent container, but we still prefer to go with an explicit indicator in X2/Xn. It is true that in Rel-14 MBB (make-before-break) indication was done by RRC, but the primary reason was because the MBB does not change the HO behavior between the source and the target. On the other hand, DAPS HO directly impacts the behavior between them – one example is, for a DRB configured with DAPS, a downlink forwarding tunnel is a must. A DAPS cannot be supported if a DRB is admitted by the target without accepting the DRB forwarding proposal from the source.

Proposal 1: Per DRB DAPS indicator is encoded in X2/XnAP.

Needless to say, the source proposes DAPS per each DRB. Of course, the source also needs to know whether the requested DAPS HO is accepted for a DRB. Otherwise the source does not know whether to prepare early data forwarding or not.

Proposal 2: HO REQ indicates whether DAPS is requested for a DRB. HO REQ ACK indicates whether DAPS is accepted for a DRB.

In the last RAN3-106, [1] proposed to consider a fallback mechanism which can be used in case the target rejects the DAPS HO.

Now considering together with per DRB DAPS, this proposal seems promising because the source is able to tell the target, per each DAPS requested DRB, whether the DRB should be rejected or it is OK to be fallback to the legacy HO (OK to be admitted as an legacy HO) in case the target cannot support DAPS HO for this DRB. A service that requires DAPS HO may be useless if treated by a normal HO (e.g. due to strict latency requirement) or may be OK depending on its service characteristics. The source can indicate such possibility by sending the desired fallback method of “reject” in the first case and “legacy HO” in the second case, which can be considered by the target as a guideline.

Proposal 3: For a DAPS requesting DRB, the source also proposes desired fallback handling (legacy HO or reject) to the target, for the case when the requested DAPS cannot be accepted by the target.

Regarding new cause, when the target rejects DAPS HO for a DRB, there seems no special reason other than rejecting normal HO (e.g. resource shortage, etc.). We thus propose not to introduce new cause for DAPS rejection, unless identified further.

Proposal 4: Do not introduce new cause for DAPS rejection, unless some special reason other than rejecting normal HO is identified.

In the past, the SN STATUS TRANSFER message had been abused to support DAPS HO forwarding, i.e. downlink PDCP SDUs forwarded with SNs assigned by the source. In the last meeting, another approach was discussed which is to introduce a new class-2 message exclusively to support this DAPS forwarding while keeping the legacy message intact. The new “Early Forwarding Transfer” message was incorporated into BL CRs with FFS.

Both works, but we believe that “Early Forwarding Transfer” is focused and easier to describe the DAPS HO behaivor in the existihg HO C-plane handling and also for CHO, which we agreed to follow DAPS-like forwarding when the early data forwarding is applied. We thus propose to adopt this new message and remove FFS.

Proposal 5: Adopt a new class-2 “Early Forwarding Transfer” message. Remove FFS.

In CP-UP separation, it is the target CU-UP who will be hosting upper-layer protocols. Whether DAPS HO is requested for a DRB to be setup, and its desired fallback (if any), is better to be considred as part of the target CU-UP’s admission process. We thus propose to propagate DAPS information to the target CU-UP and let it decide whether to accept or not.

Proposal 6: In CP-UP separation, the target CU-CP decides whether the requested DAPS HO can be accepted for a DRB.

The corresponding TP for stage-3 X2AP, XnAP, and E1AP can be found in [2], [4], and [6], respectively.

Together with the above proposals, it is better to align the current stage-2 texts as much as possible to the per DRB DAPS agreement, with the relevant U-plane handling and data forwarding details.

Proposal 7: Align the current stage-2 texts to the per DRB DAPS agreement, with the relevant U-plane handling and data forwarding details.

Such alignments are tried. The corresponding TP for 36.300 is provided in Section 5 below. The TP for 38.300 and 38.401 are tried in [3] and [5], respectively.

# 3		Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: Per DRB DAPS is indicated explicitly in X2/Xn.

Proposal 2: HO REQ indicates whether DAPS is requested for a DRB. HO REQ ACK indicates whether DAPS is accepted for a DRB.

Proposal 3: For a DAPS requesting DRB, the source also proposes desired fallback handling (legacy HO or reject) to the target, for the case when the requested DAPS cannot be accepted by the target.

Proposal 4: Do not introduce new cause for DAPS rejection, unless some special reason other than rejecting normal HO is identified.

Proposal 5: Adopt a new class-2 “Early Forwarding Transfer” message. Remove FFS.

Proposal 6: In CP-UP separation, the target CU-CP decides whether the requested DAPS HO can be accepted for a DRB.

The corresponding TP for stage-3 X2AP, XnAP, and E1AP can be found in [2], [4], and [6], respectively.

Together with the above proposals, it is better to align the current stage-2 texts as much as possible to the per DRB DAPS agreement, with the relevant U-plane handling and data forwarding details.

Proposal 7: Align the current stage-2 texts to the per DRB DAPS agreement, with the relevant U-plane handling and data forwarding details.

Such alignments are tried. The corresponding TP for 36.300 is provided in Section 5 below. The TP for 38.300 and 38.401 are tried in [3] and [5], respectively.

# 4		Reference

[1] R3-197434, “DAPS HO - Fallback mechanism”, Ericsson

[2] R3-201104, “(TP for LTE_feMob-Core BL CR for TS 36.423): Support for per DRB DAPS”, Intel Corporation

[3] R3-201105, “(TP for NR_Mob_enh-Core BL CR for TS 38.300): Support for per DRB DAPS”, Intel Corporation

[4] R3-201106, “(TP for NR_Mob_enh-Core BL CR for TS 38.423): Support for per DRB DAPS”, Intel Corporation

[5] R3-201107, “(TP for NR_Mob_enh-Core BL CR for TS 38.401): Support for per DRB DAPS”, Intel Corporation

[6] R3-201108, “(TP for NR_Mob_enh-Core BL CR for TS 38.463): Support for per DRB DAPS”, Intel Corporation

# 5		TP for BL CR for TS 36.300

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

##### 10.1.2.1.1	C-plane handling

The preparation and execution phase of the HO procedure is performed without EPC involvement, i.e. preparation messages are directly exchanged between the eNBs. The release of the resources at the source side during the HO completion phase is triggered by the eNB. In case an RN is involved, its DeNB relays the appropriate S1 messages between the RN and the MME (S1-based handover) and X2 messages between the RN and target eNB (X2-based handover); the DeNB is explicitly aware of a UE attached to the RN due to the S1 proxy and X2 proxy functionality (see clause 4.7.6.6). The figure below depicts the basic handover scenario where neither MME nor Serving Gateway changes:

# Figure 10.1.2.1.1-1: Intra-MME/Serving Gateway HO

Below is a more detailed description of the intra-MME/Serving Gateway HO procedure:

0	The UE context within the source eNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1	The source eNB configures the UE measurement procedures according to the roaming and access restriction information and e.g. the available multiple frequency band information. Measurements provided by the source eNB may assist the function controlling the UE's connection mobility.

2	A MEASUREMENT REPORT is triggered and sent to the eNB.

3	The source eNB makes decision based on MEASUREMENT REPORT and RRM information to hand off the UE.

4	The source eNB issues a HANDOVER REQUEST message to the target eNB passing necessary information to prepare the HO at the target side (UE X2 signalling context reference at source eNB, UE S1 EPC signalling context reference, target cell ID, KeNB*, RRC context including the C-RNTI of the UE in the source eNB, AS-configuration, E-RAB context and physical layer ID of the source cell + short MAC-I for possible RLF recovery). The source eNB may request a DAPS Handover for some E-RABs and may also indicate desired fallback (either legacy HO or reject) in case the requested DAPS Handover cannot be accepted for an E-RAB. UE X2 / UE S1 signalling references enable the target eNB to address the source eNB and the EPC. The E-RAB context includes necessary RNL and TNL addressing information, and QoS profiles of the E-RABs.

5	Admission Control may be performed by the target eNB dependent on the received E-RAB QoS information to increase the likelihood of a successful HO, if the resources can be granted by target eNB. The target eNB configures the required resources according to the received E-RAB QoS information and reserves a C-RNTI and optionally a RACH preamble. The AS-configuration to be used in the target cell can either be specified independently (i.e. an "establishment") or as a delta compared to the AS-configuration used in the source cell (i.e. a "reconfiguration").

6	The target eNB prepares HO with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source eNB. The HANDOVER REQUEST ACKNOWLEDGE message includes a transparent container to be sent to the UE as an RRC message to perform the handover. The container includes a new C-RNTI, target eNB security algorithm identifiers for the selected security algorithms, may include a dedicated RACH preamble, and possibly some other parameters i.e. access parameters, SIBs, etc. If RACH-less HO is configured, the container includes timing adjustment indication and optionally a preallocated uplink grant. The HANDOVER REQUEST ACKNOWLEDGE message may also include RNL/TNL information for the forwarding tunnels, if necessary. The target eNB also indicates if a DAPS Handover is accepted for an E-RAB.

NOTE:	As soon as the source eNB receives the HANDOVER REQUEST ACKNOWLEDGE, or as soon as the transmission of the handover command is initiated in the downlink, data forwarding may be initiated.

NOTE:	For E-RABs configured with DAPS, downlink PDCP SDUs are forwarded with SN assigned by the source eNB, until SN assignment is handed over to the target eNB in step 11b, for which the normal data forwarding follows as defined in 10.1.2.3.

Steps 7 to 16 provide means to avoid data loss during HO and are further detailed in 10.1.2.1.2 and 10.1.2.3.

7	The target eNB generates the RRC message to perform the handover, i.e. RRCConnectionReconfiguration message including the mobilityControlInformation, to be sent by the source eNB towards the UE. The source eNB performs the necessary integrity protection and ciphering of the message.

The UE receives the RRCConnectionReconfiguration message with necessary parameters (i.e. new C-RNTI, target eNB security algorithm identifiers, and optionally dedicated RACH preamble, target eNB SIBs, etc.) and is commanded by the source eNB to perform the HO. If RACH-less HO is configured, the RRCConnectionReconfiguration includes timing adjustment indication and optionally preallocated uplink grant for accessing the target eNB. If preallocated uplink grant is not included, the UE should monitor PDCCH of the target eNB to receive an uplink grant. The UE does not need to delay the handover execution for delivering the HARQ/ARQ responses to source eNB.

If Make-Before-Break HO or DAPS Handover is configured, the connection to the source cell is maintained after the reception of RRCConnectionReconfiguration message with mobilityControlInformation before the UE executes initial uplink transmission to the target cell.

NOTE:	If Make-Before-Break HO is configured, the source eNB decides when to stop transmitting to the UE.

NOTE:	The UE can be configured with Make-Before-Break HO and RACH-less HO simultaneously.

NOTE:	In case of DAPS HO, the UE releases the source SRB resources, security configuration of the source cell and stops DL/UL reception/transmission with source upon receiving explicit release from target node.

NOTE:	For E-RABs configured with DAPS, the source eNB does not stop transmitting downlink packets until it receives the HO SUCCESS message from the target eNB in step 11a.

8a	For E-RABs configured with DAPS, the source eNB sends the EARLY FORWARDING TRANSFER message. The DL COUNT value conveyed in the EARLY FORWARDING TRANSFER message indicates PDCP SN and HFN of the first PDCP SDU that the source eNB forwards to the target eNB. The source eNB does not stop assigning PDCP SNs to downlink packets until it sends the SN STATUS TRANSFER message to the target eNB in step 11b.

8	For E-RABs not configured with DAPS, the source eNB sends the SN STATUS TRANSFER message to the target eNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of E-RABs for which PDCP status preservation applies (i.e. for RLC AM). The uplink PDCP SN receiver status includes at least the PDCP SN of the first missing UL SDU and may include a bit map of the receive status of the out of sequence UL SDUs that the UE needs to retransmit in the target cell, if there are any such SDUs. The downlink PDCP SN transmitter status indicates the next PDCP SN that the target eNB shall assign to new SDUs, not having a PDCP SN yet. The source eNB may omit sending this message if none of the E-RABs of the UE shall be treated with PDCP status preservation.

NOTE:	In case of DAPS Handover, the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status for an E-RAB with RLC-AM and not configured with DAPS may be transferred by the SN STATUS TRANSFER message in step 11b instead of step 8.

NOTE:	For E-RABs configured with DAPS, the source eNB may additionally send the EARLY FORWARDING TRANSFER message(s) between step 8 and step 11b, to inform discarding of already forwarded PDCP SDUs. The target eNB does not transmit forwarded downlink PDCP SDUs to the UE whose COUNT is less than the conveyed DL COUNT value and discards them if transmission has not been attempted already.

9	If RACH-less HO is not configured, after receiving the RRCConnectionReconfiguration message including the mobilityControlInformation , UE performs synchronisation to target eNB and accesses the target cell via RACH, following a contention-free procedure if a dedicated RACH preamble was indicated in the mobilityControlInformation, or following a contention-based procedure if no dedicated preamble was indicated. UE derives target eNB specific keys and configures the selected security algorithms to be used in the target cell. 

If RACH-less HO is configured, UE performs synchronisation to target eNB. UE derives target eNB specific keys and configures the selected security algorithms to be used in the target cell.

10	If RACH-less HO is not configured, the target eNB responds with UL allocation and timing advance.

10a If RACH-less HO is configured and the UE did not get the periodic pre-allocated uplink grant in the RRCConnectionReconfiguration message including the mobilityControlInfo, the UE receives uplink grant via the PDCCH of the target cell. The UE uses the first available uplink grant after synchronization to the target cell.

11	When the RACH-less HO is not configured and the UE has successfully accessed the target cell, the UE sends the RRCConnectionReconfigurationComplete message (C-RNTI) to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB, which indicates that the handover procedure is completed for the UE. The target eNB verifies the C-RNTI sent in the RRCConnectionReconfigurationComplete message. The target eNB can now begin sending data to the UE.

When the RACH-less HO is configured, after the UE has received uplink grant, the UE sends the RRCConnectionReconfigurationComplete message (C-RNTI) to confirm the handover, along with an uplink Buffer Status Report, and/or UL data, whenever possible, to the target eNB. The target eNB verifies the C-RNTI sent in the RRCConnectionReconfigurationComplete message. The target eNB can now begin sending data to the UE. The handover procedure is completed for the UE when the UE receives the UE contention resolution identity MAC control element from the target eNB.

11a/b In case of DAPS Handover, the target eNB sends the HO SUCCESS message to the source eNB to inform that the UE has successfully accessed the target cell. In return, the source eNB sends the SN STATUS TRANSFER message for E-RABs configured with DAPS for which the description in step 8 applies, and the normal data forwarding follows as defined in 10.1.2.3.

NOTE	The uplink PDCP SN receiver status and the downlink PDCP SN transmitter status are also conveyed for E-RABs with RLC-UM in the SN STATUS TRANSFER message in step 11b, if configured with DAPS.

NOTE:	For E-RABs configured with DAPS for which duplication avoidance is required (i.e. RLC-AM), the source eNB does not stop deliverying uplink packets to the S-GW until it sends the SN STATUS TRANSFER message in step 11b. The target eNB does not forward the uplink PDCP SDUs successfully received in-sequence to the S-GW until it receives the SN STATUS TRANSFER message, in which UL HFN and the first missing SN in the uplink PDCP SN receiver status indicates the start of uplink PDCP SDUs to be delivered to the S-GW. The target eNB does not deliver any uplink packet which has an UL COUNT lower than the provided.

NOTE	The source eNB may omit sending the SN STATUS TRANSFER message if none of the E-RABs of the UE is configured with DAPS or shall be treated with PDCP status preservation.

12	The target eNB sends a PATH SWITCH REQUEST message to MME to inform that the UE has changed cell.

13	The MME sends a MODIFY BEARER REQUEST message to the Serving Gateway.

14	The Serving Gateway switches the downlink data path to the target side. The Serving gateway sends one or more "end marker" packets on the old path to the source eNB and then can release any U-plane/TNL resources towards the source eNB.

15	The Serving Gateway sends a MODIFY BEARER RESPONSE message to MME.

16	The MME confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

17	By sending the UE CONTEXT RELEASE message, the target eNB informs success of HO to source eNB and triggers the release of resources by the source eNB. The target eNB sends this message after the PATH SWITCH REQUEST ACKNOWLEDGE message is received from the MME.

18	Upon reception of the UE CONTEXT RELEASE message, the source eNB can release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

When an X2 handover is used involving HeNBs and when the source HeNB is connected to a HeNB GW, a UE CONTEXT RELEASE REQUEST message including an explicit GW Context Release Indication is sent by the source HeNB, in order to indicate that the HeNB GW may release of all the resources related to the UE context.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

##### 10.1.2.1.2	U-plane handling

The U-plane handling during the Intra-E-UTRAN-Access mobility activity for UEs in ECM-CONNECTED takes the following principles into account to avoid data loss during HO:

-	During HO preparation U-plane tunnels can be established between the source eNB and the target eNB. There is one tunnel established for uplink data forwarding and another one for downlink data forwarding for each E-RAB for which data forwarding is applied. In the case of a UE under an RN performing handover, forwarding tunnels can be established between the RN and the target eNB via the DeNB.

-	During HO execution, user data can be forwarded from the source eNB to the target eNB. The forwarding may take place in a service and deployment dependent and implementation specific way.

-	Forwarding of downlink user data from the source to the target eNB should take place in order as long as packets are received at the source eNB from the EPC or the source eNB buffer has not been emptied.

-	During HO completion:

-	The target eNB sends a PATH SWITCH message to MME to inform that the UE has gained access and MME sends a MODIFY BEARER REQUEST message to the Serving Gateway, the U-plane path is switched by the Serving Gateway from the source eNB to the target eNB.

-	The source eNB should continue forwarding of U-plane data as long as packets are received at the source eNB from the Serving Gateway or the source eNB buffer has not been emptied.

# For RLC-AM bearers:

-	During normal HO not involving Full Configuration:

-	For in-sequence delivery and duplication avoidance, PDCP SN is maintained on a bearer basis and the source eNB informs the target eNB about the next DL PDCP SN to allocate to a packet which does not have a PDCP sequence number yet (either from source eNB or from the Serving Gateway).

-	For security synchronisation, HFN is also maintained and the source eNB provides to the target one reference HFN for the UL and one for the DL i.e. HFN and corresponding SN.

-	In both the UE and the target eNB, a window-based mechanism is needed for duplication detection.

-	The occurrence of duplicates over the air interface in the target eNB is minimised by means of PDCP SN based reporting at the target eNB by the UE. In uplink, the reporting is optionally configured on a bearer basis by the eNB and the UE should first start by transmitting those reports when granted resources in the target eNB. In downlink, the eNB is free to decide when and for which bearers a report is sent and the UE does not wait for the report to resume uplink transmission.

-	The target eNB re-transmits and prioritizes all downlink PDCP SDUs forwarded by the source eNB (i.e. the target eNB should send data with PDCP SNs from X2 before sending data from S1), with the exception of PDCP SDUs of which the reception was acknowledged through PDCP SN based reporting by the UE.

-	The UE re-transmits in the target eNB all uplink PDCP SDUs starting from the first PDCP SDU following the last consecutively confirmed PDCP SDU i.e. the oldest PDCP SDU that has not been acknowledged at RLC in the source, excluding the PDCP SDUs of which the reception was acknowledged through PDCP SN based reporting by the target.

-	During HO involving Full Configuration:

-	The following description below for RLC-UM bearers also applies for RLC-AM bearers. Data loss may happen.

# For RLC-UM bearers:

-	The PDCP SN and HFN are reset in the target eNB, unless DL and UL status are received from the source eNB in the SN STATUS TRANSFER message.

-	No PDCP SDUs are retransmitted in the target eNB.

-	The target eNB prioritizes all downlink PDCP SDUs forwarded by the source eNB if any (i.e. the target eNB should send data with PDCP SNs from X2 before sending data from S1).

-	The UE PDCP entity does not attempt to retransmit any PDCP SDU in the target cell for which transmission had been completed in the source cell. Instead UE PDCP entity starts the transmission with other PDCP SDUs.

# For DAPS handover:

A DAPS Handover can be used for an RLC-AM or RLC-UM bearer. For a DRB configured with DAPS, the following principles will additionally apply:

Downlink:

- During HO preparation, a forwarding tunnel is always established.

- The source eNB is responsible for allocating donwlink PDCP SNs until the SN assignment is handed over to the target eNB and data forwarding in 10.1.2.3.1 (RLC-AM) or in 10.1.2.3.2 (RLC-UM) takes place. That is, the source eNB does not stop assigning PDCP SNs to downlink packets until it receives the HO SUCCESS message and sends the SN STATUS TRANSFER message to the target eNB.

- Upon allocation of downlink PDCP SNs by the source eNB, it starts scheduling downlink data on the source radio link and also starts forwarding downlink PDCP SDUs along with assigned PDCP SNs to the target eNB.

- For security synchronisation, HFN is maintained for the forwarded downlink SDUs with PDCP SNs assigned by the source eNB. The source eNB sends the EARLY FORWARDING TRANSFER message to convey the DL COUNT value, indicating PDCP SN and HFN of the first PDCP SDU that the source eNB forwards to the target eNB.

- HFN and also PDCP SN are maintained after the SN assignment is handed over to the target eNB. The SN STATUS TRANSFER message indicates the next DL PDCP SN to allocate to a packet which does not have a PDCP sequence number yet, even for RLC-UM.

- During handover execution period, the

- source and target eNBs will separately perform ROHC header compression, ciphering, and adding PDCP header.

- During handover execution period, the UE will continue to receive downlink data from both source eNB and target eNBs until source eNB connection is released by an explicit release command from target eNB.

- During handover execution period, the UE DAPS PDCP will maintain separate security and ROHC header decompression associated with each eNB, while maintaining common reordering function, duplicate detection, discard function, and PDCP SDUs in-sequence delivery to upper layers. PDCP SN continuity will be supported for both RLC AM and UM DRBs configured wih DAPS.

Uplink:

- The UE will transmit UL data to the source eNB until the random access procedure toward the target eNB has been successfully completed. Afterwards the UE switches its UL data transmission to the target eNB.

- After switching its UL data transmissions, the UE will continue to send UL layer 1 CSI feedback, HARQ feedback, layer 2 RLC feedback, ROHC feedback, HARQ data re-transmissions, and RLC data re-transmission to the source eNB.

- During handover execution period, the UE maintains separate security context and ROHC header compressor context for uplink transmissions towards the source and target eNBs. The UE maintains common UL PDCP SN allocation. PDCP SN continuity will be supported for both RLC AM and UM DRBs configured wih DAPS.

- During handover execution period, the source and target eNBs will maintain their own security and ROHC header decompressor contexts to process UL data received from the UE.

- The establishment of a forwarding tunnel is optional.

- HFN and PDCP SN are maintained in the target eNB. The SN STATUS TRANSFER message indicates the first missing UL COUNT that the target should start delivering to the S-GW, even for RLC-UM.

Upon receiving DAPS hadover command message, the UE suspends source cell SRBs, stops sending and receiving any RRC control plane signalling toward the source cell, and establishes SRBs for the target cell. The UE releases the source cell SRBs configuration upon receving source cell release indication from the target cell after successful DAPS handover execution. When DAPS handover to the target cell fails and if source cell link is available, then the UE will fallback to source cell configuration and activate source cell SRBs for control plane signaling.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 10.1.2.3	Data forwarding

##### 10.1.2.3.1	For RLC-AM DRBs

Upon handover, the source eNB may forward in order to the target eNB all downlink PDCP SDUs with their SN that have not been acknowledged by the UE. In addition, the source eNB may also forward without a PDCP SN fresh data arriving over S1 to the target eNB.

NOTE:	The target eNB does not have to wait for the completion of forwarding from the source eNB before it begins transmitting packets to the UE.

The source eNB discards any remaining downlink RLC PDUs. Correspondingly, the source eNB does not forward the downlink RLC context to the target eNB.

NOTE:	The source eNB does not need to abort ongoing RLC transmissions with the UE as it starts data forwarding to the target eNB.

Upon handover, the source eNB forwards to the Serving Gateway the uplink PDCP SDUs successfully received in-sequence until the sending of the Status Transfer message to the target eNB. Then at that point of time the source eNB stops delivering uplink PDCP SDUs to the S-GW and shall discard any remaining uplink RLC PDUs. Correspondingly, the source eNB does not forward the uplink RLC context to the target eNB.

Then the source eNB shall either:

-	discard the uplink PDCP SDUs received out of sequence if the source eNB has not accepted the request from the target eNB for uplink forwarding or if the target eNB has not requested uplink forwarding for the bearer during the Handover Preparation procedure,

-	forward to the target eNB the uplink PDCP SDUs received out of sequence if the source eNB has accepted the request from the target eNB for uplink forwarding for the bearer during the Handover Preparation procedure.

The PDCP SN of forwarded SDUs is carried in the "PDCP PDU number" field of the GTP-U extension header. The target eNB shall use the PDCP SN if it is available in the forwarded GTP-U packet.

For normal HO in-sequence delivery of upper layer PDUs during handover is based on a continuous PDCP SN and is provided by the "in-order delivery and duplicate elimination" function at the PDCP layer:

-	in the downlink, the "in-order delivery and duplicate elimination" function at the UE PDCP layer guarantees in-sequence delivery of downlink PDCP SDUs;

-	in the uplink, the "in-order delivery and duplicate elimination" function at the target eNB PDCP layer guarantees in-sequence delivery of uplink PDCP SDUs.

After a normal handover, when the UE receives a PDCP SDU from the target eNB, it can deliver it to higher layer together with all PDCP SDUs with lower SNs regardless of possible gaps.

For handovers involving Full Configuration, the source eNB behaviour is unchanged from the description above. The target eNB may not send PDCP SDUs for which delivery was attempted by the source eNB. The target eNB identifies these by the presence of the PDCP SN in the forwarded GTP-U packet and discards them.

After a Full Configuration handover, the UE delivers received PDCP SDU from the source cell to the higher layer regardless of possible gaps. UE discards uplink PDCP SDUs for which transmission was attempted and retransmission of these over the target cell is not possible.

##### 10.1.2.3.2	For RLC-UM DRBs

Upon handover, the source eNB does not forward to the target eNB downlink PDCP SDUs for which transmission had been completed in the source cell. PDCP SDUs that have not been transmitted may be forwarded. In addition, the source eNB may forward fresh downlink data arriving over S1 to the target eNB. The source eNB discards any remaining downlink RLC PDUs. Correspondingly, the source eNB does not forward the downlink RLC context to the target eNB.

Upon handover, the source eNB forwards all uplink PDCP SDUs successfully received to the Serving Gateway (i.e. including the ones received out of sequence) and discards any remaining uplink RLC PDUs. Correspondingly, the source eNB does not forward the uplink RLC context to the target eNB.

##### 10.1.2.3.3	SRB handling

With respect to SRBs, the following principles apply at HO:

-	No forwarding or retransmissions of RRC messages in the target;

-	The PDCP SN and HFN are reset in the target.

##### 10.1.2.3.4	User data forwarding for Dual Connectivity

Upon DC specific activities user data forwarding may be performed for E-RABs configured with the SCG bearer option　or with the split bearer option. The behaviour of the eNB from which data is forwarded is the same as specified for the "source eNB" for handover, the behaviour of the eNB to which data is forwarded is the same as specified for the "target eNB" for handover. If data forwarding for split bearer option is applied, the PDCP PDUs which are not acknowledged by the UE are forwarded from the SeNB to the MeNB in the course of procedures involving the release of the SCG part of the split bearer (e.g., SeNB Modification, SeNB Release, Change of SeNB).

##### 10.1.2.3.5	For DRBs configured with DAPS Handover

Data forwarding after the source eNB receives the HO SUCCESS message from the target eNB follows the same behaviors as defined in 10.1.2.3.1 if with RLC-AM and in 10.1.2.3.2 if with RLC-UM.

Before the source eNB receives the HO SUCCESS message:

-	The source eNB may forward to the target eNB downlink PDCP SDUs with SNs assigned by the source eNB. No downlink PDCP SDU without a SN assigned can be forwarded. No uplink PDCP SDU can be forwarded.

-	The source eNB sends the EARLY FORWARDING TRANSFER message(s) for discarding of already forwarded downlink PDCP SDUs in the target eNB. The first message is reserved to maintain HFN continuity by indicating PDCP SN and HFN of the first PDCP SDU that the source eNB forwards to the target eNB. The subsequent messages can be used for discarding.

-	The source eNB does not stop transmitting downlink packets to the UE. The source eNB keeps forwarding to the Serving Gateway the uplink PDCP SDUs successfully received in-sequence from the UE.