3GPP TSG-RAN WG3 Meeting #107-e	R3-201107

Electronic Meeting, February 24th – March 6th, 2020

Agenda item:		15.2.1

Source:	Intel Corporation

Title:	(TP for NR_Mob_enh-Core BL CR for TS 38.401): Support for per DRB DAPS

Document for:		Other

# 1		Introduction

This contribution proposes TP for TS 38.401 based on enhancements proposed for support for per DRB DAPS as discussed in [1].

# 2		Reference

[1] R3-201103, “(TP for LTE_feMob-Core BL CR for TS 36.300): Support for per DRB DAPS”, Intel Corporation

# 3		TP for BL CR for TS 38.401

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.9.2	Bearer context setup over F1-U

Figure 8.9.2-1 shows the procedure used to setup the bearer context in the gNB-CU-UP.

Figure 8.9.2-1: Bearer context setup over F1-U

0.	Bearer context setup (e.g., following an SGNB ADDITION REQUEST message from the MeNB) is triggered in gNB-CU-CP.

1.	The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for S1-U or NG-U, and if required, DL TNL address information for X2-U or Xn-U to setup the bearer context in the gNB-CU-UP. For NG-RAN, the gNB-CU-CP decides flow-to-DRB mapping and sends the generated SDAP and PDCP configuration to the gNB-CU-UP.

NOTE:	In case of DAPS Handover, the BEARER CONTEXT SETUP REQUEST message may include a list of DRBs that a DAPS Handover is requested, optionally an indication of desired fallback (either legacy HO or reject) in case the requested DAPS Handover cannot be accepted for a DRB.

2.	The gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, and DL TNL address information for S1-U or NG-U, and if required, UL TNL address information for X2-U or Xn-U.

NOTE:	The indirect data transmission for split bearer through the gNB-CU-UP is not precluded.

NOTE:	In case of DAPS Handover, the BEARER CONTEXT SETUP RESPONSE message indicates if a DAPS Handover is accepted for a DRB.

3.	F1 UE context setup procedure is performed to setup one or more bearers in the gNB-DU.

4.	The gNB-CU-CP sends a BEARER CONTEXT MODIFICATION REQUEST message containing the DL TNL address information for F1-U and PDCP status.

5.	The gNB-CU-UP responds with a BEARER CONTEXT MODIFICATION RESPONSE message.