3GPP TSG-RAN WG3 #107-e		R3-201132

E-meeting, 24th February – 6th March 2020

Source:	CATT

Title:	Summary on CB: # 29_Email029-RACHopt_enhs

Agenda Item:	10.2.3.1

Document for:	Discussion and decision

Introduction

In last RAN WG1 meeting, a reply LS was sent toward RAN WG3, listing all the high-layer parameters which affecting the RACH configuration, albeit leave the flexibility that RAN WG3 could take the final decision on what parameter to be transferred over the interfaces, and how to transfer them [1]. Based on this LS, many discussion paper were raised in this RAN WG3 meeting, discussing what parameters and how to include into RAN WG3 specifications for PRACH coordination [2–6]. In this document, we briefly summarise all these discussions, and try to collect different companies’ understanding on several key issues.

Since this is the first time to discuss the PRACH configuration parameters, it may be controversial.

Companies are invited to include their comments in the present document if possible by Tuesday, Feb. 26, 6 PM CET,

Based on the comments collected, we could discuss on way forward before Wednesday, Feb. 26, 6 PM CET.         After that, draft TPs could be prepared until the deadline of email discussion.

Summarise of TDocs raised before meeting

Where to include

There seems to be a consensus that the parameters provided in [1] should be included at least into the Served Cell Information NR IE in TS 38.423, the Served NR Cell Information IE in TS 36.423, and the Served Cell Information IE (delivered from the gNB-DU toward the gNB-CU) in TS 38.473.

Proposal 1: Include a new IE, namely NR Cell PRACH Configuration, into the Served Cell Information NR IE in TS 38.423 and the Served Cell Information IE in TS 38.473 (FFS for TS 36.423).

In addition, One company also proposes that these parameters should also be included into the Neighbour Information NR IE in TS 38.423 and the NR Neighbour Information IE in TS 36.423 (see in Proposal 6 of [2]).

It was widely mentioned how a gNB-CU should deliver neighbour cells’ PRACH configuration toward the gNB-DU, but their opinions are very split:

- One company proposes to introduce a new F1AP procedure (see in Proposal 7 of [2]).

- One company proposes to include it into the current Neighbour Cell Information List Item IE with in the GNB-CU CONFIGURATION UPDATE message (see in §9.2.1.10 of [7]).

- One company proposes to include it into the Cells to be Activated List Item IE within the F1 SETUP RESPONSE message and the GNB-CU CONFIGURATION UPDATE message (see in Proposal 2 of [4]).

- One company proposes to include it into the F1 SETUP RESPONSE message and the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message (see in Proposal 1 of [5]).

In addition, one company also propose to include a NR PRACH configuration IE into the Cells to be Activated List Item IE within the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message (see in Proposal 1 of [5]), while the usage of this IE needs more clarification.

How many to include

Different companies have different views on exchanging how many “PRACH configuration items” per cell (here a “PRACH configuration items” refers to a set of parameters which can determine a set of RACH occasions all of which can be used by one UE):

- One company thinks that we only need to exchange one “PRACH configuration item” per UL/SUL (see in Proposal 4 of [2]).

- One company thinks that the amount of “PRACH configuration item” can be multiple per UL/SUL (see in Proposal 1 of [3]).

- One company think that we need to exchange one “PRACH configuration item” per RACH type per UL/SUL (see in Proposal 3 and 5 of [4]).

- One company thinks that the amount of “PRACH configuration item” can be multiple per cell (see in §9.2.2.x of [8]).

- One company thinks that we only need to exchange one “PRACH configuration item” per cell (see in [6]).

What to include

All companies agree that most of the IEs listed in [1] should be included, but some companies propose some adaption:

Two companies separate the IEs into two parts. One part of them (e.g. scs-SpecificCarrierList) are relatively “common” ones and should be delivered per UL/SUL, while the other part of them (e.g. prach-ConfigurationIndex) are relatively “dedicated” ones and should be delivered per “PRACH configuration item” [3, 4].

Two companies figure out that the information of absoluteFrequencyPointA and freqBandIndicatorNR is already included within the “Served Cell Information” IEs, and thus no need to duplicate them (see in Proposal 1 of [2] and [3]).

One company thinks that the information of tdd-UL-DL-ConfigurationCommon should be included (see in Proposal 3 of [2]), but another company thinks that the existing Intended TDD DL-UL Configuration NR field can be reused directly [3].

One company thinks that the IEs related to BWP (i.e. locationAndBandwidth and msg1-FrequencyStart) are not suitable to include, and thus proposes a method to “bypass” the two IEs (see in Proposal 3 and 4 of [3]).

One company thinks that the IEs related to SSB do not cover RACH type of BFR, and propose not to include them in Rel-16 and to discuss them in later release (see in Proposal 5 of [3]).

One company thinks that the timing offset between cells should also be deliverd over interfaces (see in Proposal 7 of [4]), even if it is not mentioned in [1].

Questions

Based on abovementioned summaries, we raised following questions. Companies are welcome to provide any feedback.

Question 1-1: A gNB-CU should be possible to inform a gNB-DU about the PRACH configuration of the latter’s neighbour cells. Within what IE (or message) to include it?

| Company | Place to include | Reason (if any) |
| --- | --- | --- |
| CATT | Neighbour Cell Information List Item within the GNB-CU CONFIGURATION UPDATE message | This IE is already used for inter-gNB-DU low layer coordination, i.e. deliver the Intended TDD DL-UL Configuration NR of the neighbour cell. |
| China Telecom | To introduce a new F1AP procedure | - In TS38.473 clauses 8.2.5.2, the purpose of gNB-CU Configuration Update is to transfer the updated information to DU. From the CU side, it just forward the PRACH Configuration from neighbour node to DU rather than inform DU of the configuration update for neighbour node and/or CU - Moreover, the gNB-DU use The Neighbour Cell Information List IE only for Cross Link Interference management rather than SON purpose.(see in TS38.473 clauses 8.2.5.2) |
| QC | No Strong View | OK with either a new or existing F1AP procedure |
| Huawei | Existing procedures | Existing F1AP procedures like F1 SETUP RESPOSE, gNB-CU configuration update are sufficient. Introducing new procedures for cell level parameters/configurations is not a good way. |
| Samsung | Neighbour Cell Information List Item within the GNB-CU CONFIGURATION UPDATE message | Same view as CATT |
| ZTE | Reuse message | Depends on discussion progress of Email030, message like F1 SETUP RESPONSE can be used to update RACH configuration |
| Ericsson | New F1 procedure | This discussion overlaps with email discussion #30. Adding the list of PRACH information to an existing message that carries already other lists of cell information incurs the risk of very large messages that will not be decodable. A dedicated procedure carrying a limited number of PRACH configurations is preferred, as proposed in R3-200961. Note that the list of PRACH configurations should only be sent for RACH conflict resolution, namely there is no need to signal this list a priori if RACH conflicts are not detected. |
| Nokia | Existing procedures | Existing procedures should be sufficient, e.g. as proposed by CATT. |

Rapporteur’s summary: Views are split on this question. It should be left as an FFS.

Question 1-2: Is it necessary to include PRACH configuration IE into the Neighbour Information NR IE in TS 38.423 and the NR Neighbour Information IE in TS 36.423?

The major motivation of this proposal is to facilitate PRACH coordination between RAN nodes serving as SNs in DC.

| Company | Opinion (e.g. yes or no) | Reason (if any) |
| --- | --- | --- |
| CATT | Yes for EN-DC | It could be used to make the two neighbour gNB which does not have Xn interface know the PRACH configuration of each other |
| China Telecom | Yes for EN-DC | For the RACH configuration, a set of physical layer parameters, there is no obvious difference between SA mode and NSA mode. And the appropriate RACH configuration in both SA and NSA network can achieve the same targets |
| QC | Yes |  |
| Huawei | No strong view |  |
| Samsung |  | To make clear what information should be included in the Served Cell firstly. |
| Ericsson | NO | In the unlikely case of a RACH configuration conflict a change of PRACH configuration in the affected cell is likely to resolve the conflict issue. No need to over engineer the RACH conflict resolution solution. |
| Nokia | Yes | This information can help filter out neighbours where the PRACH configuration indicates that there is no conflict possible. |

Rapporteur’s summary: Views are split on this question. For convenience we prefer not adding any IE at present.

Question 2-1: For a given cell, should the PRACH configuration be delivered separately per UL/SUL?

NOTE: PRACH is entirely a uplink physical channel used to send RACH preamble (i.e. MSG1). Any other information exchanging over Uu during a RACH procedure does not use PRACH.

| Company | Opinion (e.g. yes or no) | Reason (if any) |
| --- | --- | --- |
| CATT | Yes |  |
| China Telecom | Yes | In NR system, one cell can support two uplink cells, one is NUL and another is SUL. Thus the PRACH configuration shall be delivered separately per NUL/SUL. |
| QC | Yes | PRACH configuration can be configured differently for NUL and SUL, so it is useful to exchange PRACH config separately per UL and SUL. |
| Samsung | Yes | UL/SUL represents different UL carriers belonging to the same cell |
| Huawei | Yes |  |
| Ericsson | Yes |  |
| Nokia | Yes |  |

Rapporteur’s summary: All companies agree that the PRACH configuration should be delivered separately for UL and SUL.

Proposal 2: The NR Cell PRACH Configuration IE further contain two sub-fields: UL PRACH Configuration List and SUL PRACH Configuration List, both encoded as an NR PRACH Configuration List structure.

Question 2-2: Is it necessary to exchange multiple “PRACH configuration items” per UL/SUL or per cell?

Here a “PRACH configuration item” refers to a set of parameters which can determine a set of RACH occasions Comparing to LTE, one main difference in NR is the introduction of multiple BWPs, and thus PRACH resources could be configured in both initial BWP and other active BWPs. According to the LS from RAN1, PRACH resources used for BFR should also be coordinated between neighbour node which would mostly be configured in active BWPs. Therefore, it seems necessary to exchange multiple PRACH configuration items to aviod PRACH configuration confliction in NR.

| Company | Opinion (e.g. yes or no) | Reason (if any) |
| --- | --- | --- |
| CATT | Yes | For one cell, different UEs can be configured with different active BWPs with corresponding PRACH configuration parameters. |
| China Telecom | No [CATT]：After further clarification,it is also OK for China Telecom to introduce multiple PRACH configruations | As we know, the PRACH resource are usually used in initial access procedure. Thus, the PRACH configuration is mandatory for initial BWP but not for other dedicated BWP. And all UEs within different BWP can share the same PRACH resource of initial BWP. Moreover, we don’t see the need to configure multiple PRACH resources in one cell. Because it will markedly increase the overhead for uplink direction. |
| QC | Yes | Similar comment as CATT |
| Samsung | Yes | For one cell, multiple PRACH resources may be configured, especially for different BWPs |
| Huawei | Yes | We propose to differentiate the PRACH configuration of cells based on the usage type, like PRACH config for BFR, PRACH config for SI, etc. Because the network may config different rout sequence parameters for different purpose. and we donot know how many PRACH items we should define. |
| ZTE | Yes |  |
| Ericsson | Yes |  |
| Nokia | No | It is not exactly clear what a “PRACH Configuration Item” is. We should try not to create an explosion of signalling through the interfaces to optimize RACH for all possible (UE specific) scenarios. |

Rapporteur’s summary: Most companies think that it is necessary to exchange multiple PRACH configuration items per UL/SUL. (Each item defines a group of RACH occasions continuous in the frequency domain and consistent on encoding parameters.)

Proposal 3: One NR PRACH Configuration List structure may contain multiple NR PRACH Configuration Item structures.

For the parameters that should be introduced, we list all the parameters listed in RAN1 LS as below.

|  | Parameter | Granularity | Presence in current spec | Note |
| --- | --- | --- | --- | --- |
| PRACH preamble sequences and formats | prach-ConfigurationIndex | Per PRACH configuration item | no |  |
|  | prach-RootSequenceIndex | Per PRACH configuration item | no |  |
|  | restrictedSetConfig | Per PRACH configuration item | no |  |
|  | zeroCorrelationZoneConfig | Per PRACH configuration item | no |  |
|  | msg1-SubcarrierSpacing | Per PRACH configuration item | no |  |
|  | rootSequenceIndex-BFR | Per PRACH configuration item | no |  |
| Time domain parameters | prach-ConfigurationIndex | Per PRACH configuration item | no |  |
|  | msg1-SubcarrierSpacing | Per PRACH configuration item | no |  |
| Frequency domain | absoluteFrequencyPointA | Per UL/SUL | NR ARFCN |  |
|  | scs-SpecificCarrierList | Per UL/SUL | Transmission Bandwidth | Only one SCS and bandwidth is provided in the current RAN3 specs, which should be updated as a list Ericsson: alternatively the whole list scs-SpecificCarrierList can be encoded as an octet string |
|  | freqBandIndicatorNR | Per UL/SUL | NR Frequency Band List |  |
|  | frequencyShift7p5khz | Per UL/SUL | no | Should be introduced in Serving cell information IE Ericsson: If this is not part of the PRACH configuration but part of Served Cell Info, then the CU to DU assistance information would need to consist of full served cell information, instead of only the PRACH configuration. Therefore this parameter should remain in the PRACH configuration |
|  | msg1-FDM | Per PRACH configuration item | no |  |
|  | msg1-FrequencyStart | Per PRACH configuration item (but related to BWP) | no |  |
|  | msg1-SubcarrierSpacing | Per PRACH configuration item | no |  |
|  | locationAndBandwidth | Per BWP | no |  |
|  | subcarrierSpacing | Per PRACH configuration item (Used to find the SCS specific offset to carrier ) | no |  |
| SSB | ssb-PositionsInBurst | Per cell | no |  |
|  | ssb-perRACH-OccasionAndCB-PreamblesPerSSB | Per cell | no |  |
|  | tdd-UL-DL-ConfigurationCommon | Per cell | Intended TDD DL-UL Configuration NR |  |

According to above table, there are three types of the parameters i.e. per-cell/UL/SUL level, per-BWP level and per-PRACH-configuration-item level.

For per-cell/UL/SUL level parameters, as noted in above table, absoluteFrequencyPointA, freqBandIndicatorNR and tdd-UL-DL-ConfigurationCommon are already included in the serving cell information IE. So, for the cell/UL/SUL specific parameters, it is only proposed to introduce NR Carrier List and frequencyShift7p5khz in serving cell information.

Question 3-1: Do you agree with the proposal above, i.e. for the cell/UL/SUL specific parameters listed in RAN1’s LS, only the NR Carrier List and the frequencyShift7p5khz should be included?

| Company | Opinion | Reason (if any) |
| --- | --- | --- |
| CATT | Yes |  |
| China Telecom | Yes for frequencyShift7p5khz | frequencyShift7p5khz shall be contained in Dynamic Spectrum Sharing and SUL |
| QC | Yes |  |
| Samsung |  | For granularity of each IE, we may have different understandings. We think the above IEs with the granularity of “Per PRACH configuration item” should be BWP specific item (except rootSequenceIndex-BFR, which we think it is UE specific). Thus, the above IEs should be divided into per-BWP/per-UL-SUL-carrier/per-Cell/per-UE. The per-BWP IEs can be included a “PRACH configuration item”; For the per-UL-SUL-carrier IEs, scs-SpecificCarrierList (corresponding to NR Carrier List in Question 3-1) and frequencyShift7p5khz are missing in current specification, which can be included in NR Frequency Info IE and SUL information IE, respectively. For the per-Cell IEs, ssb-PositionsInBurst and ssb-perRACH-OccasionAndCB-PreamblesPerSSB are missing, which can be added to Served Cell Info IE The per-UE IEs need not to be exchanged over Xn and F1 |
| Huawei | Yes |  |
| Nokia | Yes |  |

Rapporteur’s summary: Most companies agree that for the UL/SUL specific parameters listed in RAN1’s LS (except for ssb-PositionsInBurst which should be an FFS), only the NR Carrier List and the frequencyShift7p5khz should be included. For other parameters listed by RAN1, existing IEs can be reused.

Proposal 4: The NR Carrier List IE and the frequencyShift7p5khz IE should be added per-UL/SUL.

Question 3-2: Is the timing offset between cells necessary to exchange?

| Company | Opinion (e.g. yes or no) | Reason (if any) |
| --- | --- | --- |
| CATT | No strong opinion |  |
| China Telecom | No | In 4G era, timing offset is not exchanged between eNBs. For NR system, we do not see the need to introduce this information for Xn interface as well. |
| QC | No | We don’t see the need to exchange timing offset between NG-RANs |
| Samsung | No | Whether a time offset is needed or not should be decided by RAN1 first. So, this discussion is not in scope at this stage. |
| Huawei | Yes | If the two neighbour cells are not time synchronized, how could a cell know it’s PRACH channel is interfered by neighbour cell’s random access? |
| Ericsson | NO | This was discussed in the past with respect to SFTD measurements. At that time there were proposals to signal time offset that were rejected because RAN2 has already specified a UE based solution relying on SFTD measurements. |
| Nokia | No | We do not see the need to exchange this information. It is even not obvious how it this can help when networks are asynchronous. |

Rapporteur’s summary: Most companies think that it is not needed to exchange the timing offset between cells.

For the BWP related parameters(i.e. locationAndBandwidth and msg1-FrequencyStart),in Uu interface, it is configured to UE via dedicated signalling and is not suitable to be copied and exchanged over Xn interfaces. A “decoupled” IE, namely e.g. MSG1 Frequency Start from Carrier, shall be used instead of the two IEs related with BWP. Three reasons are raised:

One reason is that the BWP is usually allocated dynamically in a per-UE manner, and thus parameters related to the BWP are not suitable to be included in non-UE-associated signalling.

Another reason is that from the perspective of network, PRACH resources are naturally decoupled with BWP, which means that multiple BWPs of different UEs may share the same PRACH resources.

The last reason is that “decoupled” solution costs only 9 bits, but the locationAndBandwidth and msg1-FrequencyStart cost 25 bits in combination—16 bits are wasted for each PRACH configuration item. (Such waste does not exist over Uu, since every UE are configured with a BWP either by dedicated signalling or by broadcast, which means that the locationAndBandwidth IE is anyhow necessary to be deliverd.)

Figure 1: Different BWPs sharing the same set of PRACH resources, and the way to bypass them.

Question 3-3: Based on above analysis, shall we include MSG1 Frequency Start from Carrier instead of locationAndBandwidth and msg1-FrequencyStart?

| Company | Opinion (e.g. yes or no) | Reason (if any) |
| --- | --- | --- |
| CATT | Yes |  |
| China Telecom | No | We agree to remove LocationAndBandwidth but suggest to keep msg1-FrequencyStart |
| QC | Yes | OK to include this new parameter “MSG1 Frequency Start from Carrier” (the IE needs to be defined clearly though to not confuse with the ASN.1 IE). Also in regards to China Telecom’s comment, we are not sure how including only msg1-FrequencyStart without including LocationAndBandwidth will help determine RACH resource in frequency domain |
| Samsung | No | locationAndBandwidth is in BWP-UplinkCommon, which is for an BWP and cell specific. Thus, if a cell has multiple BWPs, each BWP will be configured with this parameter msg1-FrequencyStart is in RACH-ConfigCommon, which is BWP specific as well. So these two parameters can be included in “PRACH configuration item”. |
| Huawei |  | This may be related to how we transfer the other BWP related PRACH parameters over Xn? |
| Ericsson |  | We would like to keep the suggestions in the RAN1 LS |
| Nokia | No special preference | We could go with both options, either a) using locationAndBandwidth together with msg1-FrequencyStart or b) introduce the new parameter MSG1 Frequency Start from Carrier. We still think that the latter could be postponed to Release 17 since it is not a critical point for the current release. |

Rapporteur’s summary: Views are split on this question. It should be left as an FFS.

Proposal 5: Following IEs should be included into the NR PRACH Configuration Item structure: an NR SCS (SCS of the carrier), MSG1-FDM, the PRACH Configuration Index, the MSG1 SCS (only when L=139) the Root Sequence Index, the Zero Correlation Zone Config, and the Restricted Set Config (only when L=839).

Proposal 6: It is FFS over the following two options: a) to include a locationAndBandwidth and a msg1-FrequencyStart into the NR PRACH Configuration Item structure; or b) to include one MSG1 Frequency Start from Carrier IE instead.

Among per PRACH configuration related parameters, rootSequenceIndex-BFR is just the root sequence index for the Prach configuration set used in BFR procedure, so it is not needed to be introduced since prach-RootSequenceIndex would be included. Other per-PRACH-configuration-item parameter proposed by RAN1, it should be included in PRACH Configuration.

Question 3-4: Do you agree with the proposal above, i.e. for the per-PRACH-configuration-item parameters listed in RAN1’s LS, all of them except the rootSequenceIndex-BFR should be included?

| Company | Opinion | Reason (if any) |
| --- | --- | --- |
| CATT | Yes |  |
| China Telecom | No | In our understanding, the NW can adjust this parameter to optimize the access performance in BFR |
| QC | No | NW might configure different values for rootSequenceIndex-BFR and prach-RootSequenceIndex. So it would be useful to include rootSequenceIndex-BFR as well along with other parameters |
| Samsung | Yes | As analysed above, rootSequenceIndex-BFR is UE-specific IE, which needs not to be exchanged over network interface. |
| Huawei | No | Agree to QC. That’s why we propose to differentiate the PRACH parameters based on the usage. Or we may have a list. |
| ZTE | No | The parameter is used for RO in BFR case. |
| Ericsson | NO | Either we take the suggestion from RAN1 or we generate anLS back to RAN1 explaining why we do nto agree. We suggest to follow the RAN1 suggestinos |
| Nokia | No | In our view the mandatory parameters conveying information that needs to be exchanged for PRACH optimization are the following: prach-ConfigurationIndex, absoluteFrequencyPointA, msg1-FDM, msg1-FrequencyStart, locationAndBandwidth, ssb-PositionsInBurst,ssb-perRACH-OccasionAndCB-PreamblesPerSSB, and tdd-UL-DL-ConfigurationCommon. (or the MSG1FrequencyStartFromCarrier if it is used to replace msg1-FrequencyStart and locationAndBandwidth). The list by RAN1 was only a guidance and it was mentioned that “It would be up to RAN3 to decide which of the RACH configuration parameters are used to resolve the potential RACH configuration conflicts”. |

Rapporteur’s summary: This question seemingly incurs some misunderstanding. In fact it is only talking about aligning the IE name delivering the root sequence index, not its value…An FFS should be added here anyhow.

Proposal 7: It is FFS on whether the rootSequenceIndex-BFR IE should be encoded encoded as a Root Sequence Index IE or as another IE.

Question 3-5: Considering the limited time and we are not clear on the benefit of exchanging beam-related parameters, for the issue on whether and how to include the beam-related parameters, shall we delay it to later release, e.g. Rel-17?

| Company | Opinion (e.g. yes or no) | Reason (if any) |
| --- | --- | --- |
| CATT | Yes |  |
| China Telecom | No | In order to minimize access delays for the UEs under the coverage of popular SSBs, per beam level optimization need to be considered in NR network. Therefore, ssb-perRACH-OccasionAndCB-PreamblesPerSSB IE shall be exchanged between nodes. Based on this parameter, the neighbour node can derive the association pattern period which indicates PRACH occasions and SS/PBCH blocks repeats at most every 160 msec. |
| QC | No | Including beam level RACH parameters might be useful for the NW to optimize RACH performance specially for the SA mmWave networks. So we propose to include the beam-related parameters in Rel-16. |
| Samsung | Yes |  |
| Huawei | No |  |
| Ericsson | Yes | We do not think this is needed today |
| Nokia | No | Beam level RACH parameters can be very useful for RACH optimization in NR. We propose to exchange all the beam-related parameters specified in the RAN1 LS response, namely ssb-PositionsInBurst,ssb-perRACH-OccasionAndCB-PreamblesPerSSB, and tdd-UL-DL-ConfigurationCommon. |

Rapporteur’s summary: Views are split on this question. It should be left as an FFS. Since we need to draft a TP, there is another issue: what IE to add. We think copying the entire ssb-perRACH-OccasionAndCB-PreamblesPerSSB seems too much than we need. Let’s see the field description of for this IE:

ssb-perRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion. Value oneEight corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicates the number of Contention Based preambles per SSB. Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1, SSB-per-rach-occasion). See TS 38.213 [13].

What we need is only the number of SSBs per RACH occasion—whether a preamble is used for Contention Based Random Access (CBRA) or Contention Free Random Access (CFRA) is entirely a MAC layer issue and no need to coordinate between cells. (Note that there is another IE in RACH-ConfigCommon, namely totalNumberOfRA-Preambles, defining the total number of CBRA preambles and CFRA preambles for common use.)

Hence another IE, namely ssb-perRACH-Occasion in TS 38.331, is more proper to be introduced. Nevertheless an FFS should be tagged here.

Proposal 8: It is FFS over the following two options: a) to postpone the discussion over SSB-related paraters to Rel-17; or b) to include one SSB Positions In Burst IE per cell, and one SSB per RACH Occasion IE per NR PRACH Configuration Item structure.

Conclusion

According to the discussion, the following could be agreed:

Include a new IE, namely NR Cell PRACH Configuration, into the Served Cell Information NR IE in TS 38.423 and the Served Cell Information IE in TS 38.473 (FFS for TS 36.423).

The NR Cell PRACH Configuration IE further contain two sub-fields: UL PRACH Configuration List and SUL PRACH Configuration List, both encoded as an NR PRACH Configuration List structure.

The NR Carrier List IE and the frequencyShift7p5khz IE should be added per-UL/SUL.

Following IEs should be included into the NR PRACH Configuration Item structure: an NR SCS (SCS of the carrier), MSG1-FDM, the PRACH Configuration Index, the MSG1 SCS (only when L=139) the Root Sequence Index, the Zero Correlation Zone Config, and the Restricted Set Config (only when L=839).

For the following bullet, it is supported by 7 companies and 1 company objects，to  make progress, we proposed to decide base on the views of majority.

One NR PRACH Configuration List structure may contain multiple NR PRACH Configuration Item structures.

The following is still FFS:

It is FFS over the following two options: a) to include a locationAndBandwidth and a msg1-FrequencyStart into the NR PRACH Configuration Item structure; or b) to include one MSG1 Frequency Start from Carrier IE instead.

It is FFS on whether the rootSequenceIndex-BFR IE should be encoded as a Root Sequence Index IE or as another IE.

It is FFS over the following two options: a) to postpone the discussion over SSB-related paraters to Rel-17; or b) to include one SSB Positions In Burst IE per cell, and one SSB per RACH Occasion IE per NR PRACH Configuration Item structure.

Proposal :It is proposed to agree  the corresponding TP for XnAP CR and F1AP CR is in R3-201329 and R3-201347 which reflect the above conclusion and FFs.

Reference

[1] R1-1913578; Reply LS on PRACH configuration conflict detection.

[2] R3-200164; RACH Optimization Enhancement for NR; China Telecom.

[3] R3-200437; Discussion on PRACH coordination; CATT.

[4] R3-200494; (TP for SON BL CR for TS 38.423): PRACH configuration exchange; Huawei, CMCC.

[5] R3-200625; Left issue for NR RACH OPTIMIZATION; ZTE.

[6] R3-200959; Solution for RACH Optimisation over the Xn; Ericsson.

[7] R3-200440; (TP for SON BL CR for TS 38.473) Addition of PRACH Coordination; CATT.

[8] R3-200627; (TP for [NR_SON_MDT] BL CR for TS 38.423)Addition of RACH OPTIMIZATION; ZTE.