3GPP TSG-RAN3 Meeting #107-e	R3-201160

E-Meeting, 24 February – 6 March, 2020

Title: 	CB: # 89_Email088-MobEnh_CHO_common_CHOmod

Source: 	Huawei

Agenda item:	15.3.1.3

Document for:	Discussion and decision

1. Introduction

This document contains the summary of email discussion for the following CB:

CB: # 89_Email088-MobEnh_CHO_common_CHOmod

- Source-initiated CHO modification:

- Remove “CHO-replace” code and target node UE AP ID in HO REQ? (ZTE)

- standardize how to inform the source about parts of the UE configuration that are kept or released at the target, so that the source knows which reconfigurations may be executed without re-initiating the CHO preparation;  add a bitmap (e.g. 16 bits) where each bit corresponds to a configurable option that may or may not be kept at the target for the UE. Details of the usage of the bitmap may be clarified once RAN2 is consulted? (Nok)

- Target-initiated CHO modification:

- no need to use an explicit HO cancellation to cancel previous configurations; Reuse existing CHO Cancel and HO Request for target-initiated CHO modification? (CT)

- By an explicit indicator in CHO Cancel? (HW), (Nok)

- Add a new cause value for the candidate target node to inform the source node that new resources are available for that UE? (E///)

- anything broken with “conservative” approach (i.e. not to introduce new, explicit procedure)? If not, propose to go for such approach?

- if agreeable, revise/merge as needed; go for agreement

(HW)

Summary of offline disc R3-201160

2. Discussion

2.1 Source-initiated CHO modification

It is proposed in [1] that:

During last RAN3#106 meeting, for source initial CHO modification，RAN3 agreed to reuse HO Request / Ack to replace an already prepared CHO procedure. And FFS whether explicit HO cancellation is needed

From our point of view, the newest Handover Request message can be considered as an implicit indication to cancel the previous configuration at the target cell. There is no need to use an explicit HO cancellation to cancel previous configurations. And the target node has the ability to identify about which is the newest Handover Request message, there is no need to add an indication in the Handover Request message to flag whether the message is the newest one.

Observation 1: The newest Handover Request message can be considered as an implicit indication to cancel the previous configuration at the target cell.

Proposal 1: There is no need to use an explicit HO cancellation to cancel previous configurations.

Companies’ comments and views to above proposal 1:

| Company | Yes/NO | Comments |
| --- | --- | --- |
| Intel | Yes | Thought we already agreed in this way. |
| ZTE | Yes |  |
| Samsung | Yes |  |
| Nokia | Yes | As Intel commented, this is a status quo, already agreed. |
| CATT | Yes |  |
| QC | Yes |  |
| NEC |  | No, no need. Yes, no need. |
| CT | Yes |  |
| LGE | Yes |  |
| Google | Yes |  |
| Ericsson | yes |  |
| Huawei | yes |  |

Summary:

All companies agree that no need explicit HO cancel message for source initial CHO modification.

Proposal 1a: Conclusion: There is no need to use an explicit HO cancellation to cancel previous configurations for source initial CHO modification.And  it seem already an agreement of last meeting.

For the source node initiated CHO modification case, there is such FFS:

“Editor’s note: Details of the signalling (if the “CHO-replace” is needed) are FFS.”

In [5], it is observed that:

- Observation 1: Both target and source node shall know which candidate target cell is associated to which UE signaling connection during CHO setup/initiate phase.

- Observation 2: Once the candidate target cell is created via certain UE signaling connection, then the subsequent “modify” or “cancel” operation with that cell must be done via the same associated UE signaling connection for its setup.

- Observation 3: The “CHO-replace” code and even the “ target node UE APID” is not needed for HANDOVER REQUEST message, as the source node shall always follow the co-relation between existing candidate target cell and UE signaling connection.

Therefore, companies’ comments and views are collected here to remove the CHO-replace code and the ‘target node UE APID’ in Handover request message as per observations in [5]:

| Company | Yes/NO | Comments |
| --- | --- | --- |
| Intel | Yes | Agree with ZTE’s point. A single cell ID cannot be associated with two or more target UE AP IDs, but the opposite is possible. There seems no need to have “CHO-replace” code and the target UE AP ID when overwriting. |
| ZTE | Yes | If we follow above principles, then “CHO-replace” code and the “target UE AP ID” should be redundant. |
| Samsung |  | We agree with ZTE’s observations, but “CHO-replace” indicator may be helpful. We don’t have strong opinion. |
| Nokia | NO | “CHO-replace” was added (and agreed!) in order to make usage of the target’s UE IE clear: so that it can be conditional, not simply optional. This makes ASN.1 clearer (in the past, in other solutions, we had such codepoints, too, e.g. in the configuration for EN-DC). Therefore, even though one can imagine a solution without it, having the “CHO-replace” will help a lot! |
| CATT |  | NO strong opinion. |
| QC |  | Agree with removing “CHO-replace”. But the target UE AP ID should be necessary. |
| NEC |  | If it can identify from HANDOVER REQUEST message is for the same UE Context, then no need to have “CHO-replace” nor “target UE AP ID”. |
| CT |  | NO strong opinion. |
| LGE | Yes | Agree with ZTE and Intel |
| Google |  | Agree with NEC |
| Ericsson | No | Once known, it is always beneficial to have the pair of UE AP IDs to identify the UE-associated logical connection. For the new codepoint, I agree with Nokia that it helps having the new IE conditional (if this is a modification the target UE AP ID is known, and shall be added) |
| Huawei | No | We think the explicit CHO-replace value could simplify the implementation in the target side. |

Summary:

5 Companies think that there is no need to have the CHO-replace indication and the target UEAP ID in case of source Initiated CHO modification. 1 company think there is no need to have the CHO-replace, but the target UE AP ID should be necessary. 1 companies share the same view, but has no strong option.

2 companies have no strong option.

3 companies think the CHO-replace code is necessary and beneficial for the target node.

Conclusion: There is no agreement to remove the CHO-replace code and the target UE AP ID from HO Request message in case of source initiated CHO modification.

In [2], in order to avoid unnecessary CHO modification signalling over network interface it is proposed that:

Proposal 1: RAN3 should enable standardised method to inform the source about parts of the UE configuration that are kept or released at the target, so that the source knows which reconfigurations may be executed without re-initiating the CHO preparation.

Proposal 2: RAN3 to add a bitmap (e.g. 16 bits) where each bit corresponds to a configurable option that may or may not be kept at the target for the UE. Details of the usage of the bitmap may be clarified once RAN2 is consulted.

Therefore, companies’ comments and views are collected here for above two proposals:

| Company | Yes/NO | Comments |
| --- | --- | --- |
| Intel | No | Seems a bit complicated (bitmap?), which requires RAN2 consultation. Seems not suitable as we are toward the end of the WI. |
| ZTE | No | More like optimization, network side is not really keen on message size. |
| Samsung | No | Seems bit complicated and target configuration should be transparent to source node. |
| Nokia | Yes | Well, sure, this is optimization, but it can save a lot of effort on the target’s side: currently, every RRC reconfiguration at the source, even a small one, requires cancelling CHO at the UE, sending a new CHO request, full admission control at the target and then finally setting up CHO again in the UE. The bitmap allows us to complete the ASN.1 on time and add meaning to the bits in backward-compatible way in future, once RAN2 responds to our request. |
| CATT | No | Agree with Intel |
| QC | No | This can be quite complicated because the target configuration changes compared to the source may be extensive. RAN2 needs to be consulted to check feasibility. |
| NEC |  | Not sure how does it work. |
| LGE |  | As mentioned, RAN2 should be consulted first. |
| Google |  | Agree that RAN2 should be consulted first |
| Ericsson | No | Not convinced, but seems to be a RAN2 topic anyway |
| Huawei |  | Agree to consult RAN2 firstly. |

6 companies say no due to complexity reason or an optimization issue. 5 companies think this is a RAN2 issue and should consult RAN2.

Conclusion: There is no agreement to add a bitmap from the target to source to indicate about parts of the UE configuration that are kept or released at the target,

2.2 Target-initiated CHO modification

Since there are 3 companies propose to support target-initiated CHO modification in [1], [7] and [10] and there is no objection proposals, the CB master would like to draw the following proposal directly without the quiz step:

Proposal 2a: The candidate target node initiated CHO modification is supported in rel-16.

Different views, if any, please provide here:

| Company | Yes/NO | Comments |
| --- | --- | --- |
| Intel | No | But open for Yes. |
| ZTE | Yes |  |
| Samsung | No | It might be rare case and seems to make the implementation complicated, but open for this proposal. |
| Nokia | ? | Well, it is a fact, not proposal, that the target may have to release resources allocated for the UE and then it may have other resources available. And such release is already supported, by target-initiated cancellation. So I am not sure what the question is about? |
| CATT | No | Just rare case for the target CHO modification. But open for Yes |
| QC | Yes |  |
| NEC | No. | If the CHO Cancelation from the target is considered as the Target-initiated CHO Modification, then no need further procedure, and no need further new IE. The rest will be up to the source node to decide e.g. whether to initiate new CHO preparation. |
| CT | Yes | We think it is beneficial to enable the candidate node modify the prepared CHO configurations in the UE during CHO procedure. The modification procedure is different with the cancel procedure, for source node receiving the CHO cancel message, it will just cancel the ongoing CHO procedure, but for source node receiving the CHO cancel message contains a modification indicator(or cause value), it will re-triggered another CHO preparation procedure to the same candidate target node. |
| LGE | Yes |  |
| Google | Yes |  |
| Ericsson | Yes/No | Regarding our proposal in [10] this is not a target-initiated modification per-se, because it is up to the source to send a new Handover Request. So we do not support target-initiated modification, but we support the possibility for the target to cancel CHO to free the allocated resources, but with the possibility to signal to the source that it might accept a new CHO for the same UE |
| Huawei | Yes |  |

For the procedure for target initiated CHO modification, the majority companies propose in [1], [7] and [10] that:

- Reuse existing CHO Cancel and HO Request for target-initiated CHO modification.

Companies’ comments and views to above proposal:

| Company | Yes/NO | Comments |
| --- | --- | --- |
| Intel | Yes | If P2a is aggregable, then prefer in this way. |
| ZTE | Yes | Reuse conditional HO Cancel. |
| Samsung | Yes | If P2a is agreeable, we also prefer it. |
| Nokia | ? | Again, cancelling prepared CHO from the target side is already now possible, as well as sending a new CHO request from the source. So what new functionality the question addresses? |
| CATT | Yes |  |
| QC | Yes |  |
| NEC | No | If the CHO Cancelation from the target is considered as the Target-initiated CHO Modification, then no need further procedure, and no need further new IE. The rest will be up to the source node to decide e.g. whether to initiate new CHO preparation. |
| CT | Yes |  |
| LGE | Yes |  |
| Google | Yes |  |
| Ericsson | Yes | But as said above, this is not a target initiated modification, but an indication that the target might accept a new CHO for the same UE. Final decision is up to source node |
| Huawei | yes |  |

Summary:

4 companies say no, 8 companies say yes.

Therefore, the majority is to OK or support to the proposal.

Proposal: Reuse existing CHO Cancel and HO Request to enable target node to inform source node the change of reserved CHO resources for the UE.

There are two options on how to support the target initiated CHO modification:

- Option 1: Add a new CHO modification required indicator in CHO Cancel message and the source node may issue a new handover preparation for this, as proposed in [7].

- Option 2: Add a new cause value for the candidate target node to inform the source node that new resources are available for that UE, as proposed in [10].

Companies’ comments and views to each option:

| Company | Option 1 or 2? | Comments |
| --- | --- | --- |
| Intel | Option 2 | Prefer Option 2. In anyway, we believe that the cell-level indication should be also in place. Currently, Candidate Cells To Be Cancelled List IE does not have cause as a sub-IE. Also, the indicator proposed by [7] (also proposed by Nokia’s R3-200216) is only per message-level. |
| ZTE | Option 2 | Per cell level cause is better. |
| Samsung | Option 2 | If P2a is agreeable, we also prefer option 2. |
| Nokia | Option 1 | By RAN3 principles, the Cause is not supposed to trigger any action in the receiver – it is for statistics only. That’s why we never specify using particular cause values. Therefore, if the solution is to be correct, we should have a separate indicator which then will be described that if used, the source may consider starting a CHO again. Regarding per-UE or per-cell, we are open to both, though we believe the per-UE is enough. |
| CATT | Option1 | If P2a is agreeable, we also prefer option 1. Similar understanding as Nok |
| QC | Option 1 | If P2a is agreeable, we prefer Option 1. |
| NEC | Option 2 | Think no need to include anything new in the CHO Cancel. If need to tell the reason then cause value is enough. |
| CT | Option1 | Prefer Option1, but open for Option2. |
| LGE | Option 2 | Cause value is workable |
| Google | Option 2 | Cause value is fine |
| Ericsson | Option 2 | As in our paper. Rare case, minimum impact is preferable |
| Huawei | Option 1 | Agree to Nokia’s comment. |

Summary:

7 companies select option 2, and 5 companies support option 1.

Proposal 2c: It is FFS whether an explicit indicator is needed in conditional HO cancel message or a new cause value is sufficient.

3. Summary

- In this document, after discussion, the following proposals are made for agreement:

Proposal 1: Reuse existing CHO Cancel and HO Request to enable target node to inform source node the change of reserved CHO resources for the UE.

Proposal 2: It is FFS whether an explicit indicator is needed in conditional HO cancel message or a new cause value is sufficient.

4. Reference

|  | 15.3.1.3. CHO Modification |  |  |
| --- | --- | --- | --- |
| [1] | R3-200161 | Discussion on Conditional Handover Modification procedure (China Telecommunication) | discussion |
| [2] | R3-200213 | Avoiding unnecessary CHO modification (Nokia, Nokia Shanghai Bell) | discussion |
| [3] | R3-200214 | (TP for NR_Mob_enh BL CR for TS 38.423): Enabling information on the scope of changes allowed for the source (Nokia, Nokia Shanghai Bell) | other |
| [4] | R3-200215 | (TP for LTE_feMob BL CR for TS 36.423): Enabling information on the scope of changes allowed for the source (Nokia, Nokia Shanghai Bell) | other |
| [5] | R3-200322 | TP for LTE_Mob_enh BL CR for TS 36.423 “CHO-replace” code and target node UE APID are not needed (ZTE) | other |
| [6] | R3-200323 | TP for NR_Mob_enh BL CR for TS 38.423 “CHO-replace” code and target node UE APID are not needed (ZTE) | other |
| [7] | R3-200507 | Target Node Initiated CHO Modification (Huawei) | discussion |
| [8] | R3-200508 | (TP for NR_Mob_enh BL CR for TS 38.423): Target Node Initiated CHO Modification (Huawei) | other |
| [9] | R3-200509 | (TP for LTE_feMob BL CR for TS 36.423): Target Node Initiated CHO Modification (Huawei) | other |
| [10] | R3-201079 | CHO - Modification from target node (Ericsson) | discussion |
| [11] | R3-201080 | (TP for NR Mob BL CR for TS 38.423): CHO - Modification from target node (Ericsson) | other |
| [12] | R3-201081 | (TP for LTE_feMob BL CR for TS 36.423): CHO - Modification from target node (Ericsson) | other |