3GPP TSG-RAN WG3 #107-e	R3-201182

24 February-6 March 2020

Online

Title: 	Summary of offline discussion for CB: # 100_Email100-IIoT_HLmulticonn_sol1

Source: 				CATT

Agenda item:		17.2.4.2

Document for:		Discussion and Decision

1	Introduction

In RAN3#107e meeting, the email discussion is assigned as below for the NRIIOT Hi-layer multi-connectivity solution #1.

CB: # 100_Email100-IIoT_HLmulticonn_sol1

-  note LS (0109); take into account SA2 agreements

- RSN should be carried in related procedures in E1AP and NGAP and XnAP when redundant PDU session setup/modify procedure initialed in NGAP; Both Xn and NG handover should be supported for redundant PDU session; RSN shall be carried in SN Addition procedure and Xn handover ; NG-RAN should inform SMF about the redundant PDU session setup failure in the PDU Session Resource Setup Response Transfer when keep the PDU sessions; NG-RAN should carried the cause value about  the redundant PDU session setup failure in the PDU Session Resource Setup Unsuccessful Transfer  when release the PDU sessions; In Xn handover case, the target node notifies the 5GCN in path switch request with cause value; In Xn handover case, the target node notifies the source node in HO request ack or Ho Preparation Failure with cause value; introduce new cause value? (CATT), (Nok), (SS), (ZTE), (LG), (E///), (HW)

- converge around a minimum agreeable set; if agreeable, split work, revise/merge; go for agreement

(CATT)

Summary of offline disc

This contribution will initial the email discussion and summarize the status of this discussion during the meeting.

The email discussion owner would like to trigger email discussions as below steps:

- Summarize the contributions which are submitted in section 17.2.4.2 in the meeting agenda.

- Group the topic and analysis the solutions

- For the issues on which we have same view from all the contributions, directly give out the agreement proposal

- For the issues on which we have different view, list all the solutions and questions for discussion.

- Converge the different the solutions during the email discussion, if get agreement, convert to agreement proposal.

- If we cannot get the convergence for the difference, we will leave them open for next meeting.

In this email discussion, we try to get the agreement for the solutions for all topics. Companies are welcome to provide answer for the questions by Feb. 26, 18:00 CET. Then I can make the agreement proposal for the TP generation and work split base on the discussion. So we may have one day to modify the TPs base on the discussion summary before the CB deadline Feb. 27, 18:00 CET.

2	Discussions

2.1 Summarize the contributions

2.1.1 RSN

We get the LS from SA2 about the RSN and PDU session pair information in R3-200109. SA2 concluded that for Rel-16 SA2 will not provide any solution to gain more flexibility and rely upon RSN as currently specified.  SA2 may consider this issue in future release and provide a complete solution, if there is a need to do so.

In the redundancy information carrying related contributions for the solution#1, all the companies agree use   RSN in R16 for the redundancy information transfer.

But different company use the different name and construct for the RSN. List all the RSN naming as blow.  You may select preference.

Proposal 1： Only use the RSN information for redundant PDU session handling in R16

2.1.2 Topics

After summarize and analyse the contributions, we get the below topic which are discussed in the contributions

- In which procedure, the RSN information is needed

- How to handle the failure setup for the Redundancy but successfully setup as normal PDU session.

- Cause value added for kinds of failure case

- How to exchange the redundant UP path supporting information between the nodes

- The indications of the Secondary RAN to SMF

We will discuss the 1st topics in different interface and discuss the topic 4th and 5th in separate section.

In the contributions, all the companies discuss the 1st topic; CATT, LG and E/// discuss the 2rd topic; ZTE,

CATT, LG, E/// and HW discuss the 3rd about cause value; ZTE/CATT discusses the 4th topic; SS discusses the 5th topic.

2.1.3 RSN information in procedures

2.1.3.1 NG interface

In NG interface, the summary of the contribution on 1st topic is in the below table.

Table 2.1.3.1-1 RSN in Procedure contents

| Message | Company | Comments |
| --- | --- | --- |
| 8.2.1 PDU Session Resource Setup | ZTE, CATT, SS, LG, E///, HW |  |
| 8.2.3 PDU Session Resource Modify | CATT |  |
| 8.4.2 Handover Resource Allocation | CATT, SS, LG, HW |  |

Table 2.1.3.1-2 RSN in Message/IE

| Message/IE | Company | Comments |
| --- | --- | --- |
| 9.3.4.1 PDU Session Resource Setup Request Transfer | ZTE, CATT, SS, LG, E///, HW |  |
| 9.3.4.4 PDU Session Resource Modify Request Transfer | CATT |  |

From the above table, all the companies agree to carry the RSN in PDU Session Resource Setup procedure and PDU Session Resource Setup Request Transfer. Only CATT propose the PDU Session Resource Modify Request Transfer in PDU Session Resource Modify procedure also need carry the RSN information.

Beside the agreement, we still have some different view on this topic. We would like to ask the questions as below.

Question 2-1: Should RSN information be carried in Handover Resource Allocation?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | For NG based HO, the AMF/SMF needs to send RSN to the target node for redundant PDU session configuration. |
| Nokia | Yes | This is needed for N2 handover. |
| Huawei | Yes |  |
| ZTE | Yes | For NG based HO, it is needed. |
| Ericsson | Yes | The HO request reuse the same SMF IE as the Setup. We think it is included already in Proposal 1. |
| LG | Yes | For NG based HO, it is also important to keep two redundant PDU sessions for that UE in URLLC. |
| CATT | Yes |  |

Question 2-2: Should RSN information be carried in PDU Session Resource Modify?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | Seems there is no scenario to modify the RSN. |
| Nokia | No | Agree with Samsung. |
| Huawei | No | Agree that there is no scenario for the modification of RSN described in stage 2 specification. |
| ZTE | No | SA2 did not send this scenario to RAN. |
| Ericsson | No |  |
| LG | No | Agree with Samsung |

Observation 1: All the company agree that RSN information is carried in Handover Resource Allocation, but not agree that RSN information is carried in PDU Session Resource Modify

Proposal 2： Carry the RSN information in the NG procedure: PDU Session Resource Setup, Handover Resource Allocation and the IE:PDU Session Resource Setup Request Transfer

2.1.3.2 Xn interface

In Xn interface, the summary of the contribution on 1st topic is in the below table.

Table 2.1.3.2-1 RSN in Procedure contents

| Message | Company | Comments |
| --- | --- | --- |
| 8.3.1 S-NG-RAN node Addition Preparation | ZTE, CATT, SS, LG, E///, HW, NOK |  |
| 8.3.3 M-NG-RAN node initiated S-NG-RAN node Modification Preparation | ZTE, CATT, SS, LG, E///, NOK |  |
| 8.2.1 Handover Preparation | CATT, SS, LG, E///, HW, NOK |  |

Table 2.1.3.2-2 RSN in Message/IE

| Message | Company | Comments |
| --- | --- | --- |
| 9.2.1.5 PDU Session Resource Setup Info – SN terminated | ZTE, CATT, SS, LG, E///, HW, NOK |  |
| 9.2.1.1 PDU Session Resources To Be Setup List | CATT, SS, LG, E///, HW, NOK |  |

From the above table, all the company agree to carry the RSN in S-NG-RAN node Addition Preparation procedure and PDU Session Resource Setup Info – SN terminated.

Beside the agreement, we still have some different view on this topic. We would like to ask the questions as below to the specific company

| Questions | Company | Yes/No | Comments |
| --- | --- | --- | --- |
| Question 2-3: Should RSN information be carried in M-NG-RAN node initiated S-NG-RAN node Modification Preparation? | Samsung | Yes | The MN may use M-NG-RAN node initiated S-NG-RAN node Modification Preparation to add a PDU session in the SN configured as SN terminated bearer. |
|  | HW | Yes |  |
| Question 2-4: Should RSN information be carried in Handover Preparation procedure? | Samsung | Yes | Agreed by SA2. Needed for redundant PDU session configuration in target side during handover |
|  | ZTE | Yes |  |
| Question 2-5: Should RSN information be carried in PDU Session Resources To Be Setup List | Samsung | Yes | PDU Session Resources To Be Setup List is in Handover Request message. |
|  | ZTE | Yes |  |

Observation 2: All the company agree that RSN information is carried in M-NG-RAN node initiated S-NG-RAN node Modification Preparation, Handover Preparation procedure and the IE PDU Session Resources To Be Setup List

Proposal 3： Carry the RSN in in the Xn procedure: S-NG-RAN node Addition Preparation, M-NG-RAN node initiated S-NG-RAN node Modification Preparation, Handover Preparation and the IE: PDU Session Resource Setup Info – SN terminated, PDU Session Resources To Be Setup List

2.1.3.3 E1 interface

In E1 interface, the summary of the contribution on 1st topic is in the below table.

Table 2.1.3.3-1 RSN in Procedure contents

| Message | Company | Comments |
| --- | --- | --- |
| 8.3.1 Bearer Context Setup | ZTE, CATT, LG, E///, HW | Nokia: NOK |
| 8.3.2 Bearer Context Modification (gNB-CU-CP initiated) | ZTE, CATT, LG, E/// | Nokia: NOK |

Table 2.1.3.3-2 RSN in Message/IE

| Message | Company | Comments |
| --- | --- | --- |
| 9.3.3.2 PDU Session Resource To Setup List | ZTE, CATT, LG, E///, HW | Nokia: NOK |
| 9.3.3.10 PDU Session Resource To Setup Modification List | ZTE, CATT, LG, E/// | Nokia: NOK |
| 9.3.3.5 PDU Session Resource Setup List | ZTE | Nokia: NOK |
| 9.3.3.11 PDU Session Resource To Modify List | ZTE | Nokia: NOK |
| 9.3.3.17 PDU Session Resource Setup Modification List | ZTE | Nokia: NOK |
| 9.3.3.19 PDU Session Resource Modified List | ZTE | Nokia: NOK |
| 9.3.3.23 PDU Session Resource Required To Modify List | ZTE | Nokia: NOK |

From the above table, all the companies agree to carry the Bearer Context Setup procedure and 9.3.3.2 PDU Session Resource To Setup List.

[removed] Proposal 4： Agree to carry the RSN in the Bearer Context Setup procedure and 9.3.3.2 PDU Session Resource To Setup List

Nokia : NOK. We have no agreement for an E1 CR.

Beside the agreement, we still have some different view on this topic. We would like to ask the questions as below.

Question 2-6:  Should RSN information be carried Bearer Context Modification (gNB-CU-CP initiated) and PDU Session Resource To Setup Modification List?

| Company | Yes/No | Comments |
| --- | --- | --- |
| HW | Yes |  |
| Samsung | Yes |  |
| Nokia | No | No agreement to have an E1 CR. |
| ZTE | No | If disjoint UP number exchange between CU-CP and CU-UP is not agreed then E1AP is not needed |
| Ericsson | Yes |  |
| LG | Yes |  |
| CATT | Yes |  |

Question 2-7:  Should RSN information be carried in the message/IE which only ZTE proposed in Table 2.1.3.3-2

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | Don’t need RSN from UP to CP. |
| Nokia | No | No agreement to have an E1 CR. |
| Huawei | No |  |
| ZTE | Yes | For example of MR-DC and SN with split CU/DU. In SN, CU-CP should know which one or more CU-UPs can configure RSN1 and which one or more CU-UPs can configure RSN2, then SN can transfer disjoint UP path number to MN for good RAN configuration. |
| Ericsson | No |  |
| LG | No |  |
| CATT | No |  |

Nokia does not agree to introduce the RSN information in E1. Other companies think it is necessary for the E1 CR.  And think the RSN information should be carried in Bearer Context Setup Bearer Context Modification (gNB-CU-CP initiated). So we cannot get the consensus on the E1 CR.

Observation 3: No consensus on whether the E1 CR is needed

2.1.3.4 F1 interface

In F1 interface, the summary of the contribution on 1st topic is in the below table.

Table 2.1.3.4-1 RSN in Procedure contents/IEs

| Message | Company | Comments |
| --- | --- | --- |
| 8.3.1 UE Context Setup | E/// | Nokia: NOK |
| 8.3.4 UE Context Modification (gNB-CU initiated) | E/// | Nokia : NOK |
| 9.3.1.45 QoS Flow Level QoS Parameters | E/// | Nokia: NOK |

From the above table, only E/// proposed the F1 modification for the RSN information transfer. We would like to ask the questions as below.

Question 2-8:  Should RSN information be carried in F1 interface?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung |  | DU has no PDU session information. Need more clarification why DU need RSN ? |
| Nokia | No | No agreement to have an F1 CR. |
| Huawei | No |  |
| ZTE | No |  |
| Ericsson | Yes | We are open to this point, but PDU Session ID is known by F1AP. |
| LG | No |  |
| CATT | No |  |

Observation 4: No consensus on whether the F1 CR is needed

2.1.4 Failure for Redundant setup

In the contributions, CATT, LG and E/// discuss the issue that the redundant PDU session setup failure but successfully setup the normal PDU session.

The proposal is below:

| Company | Proposal | Comments |
| --- | --- | --- |
| CATT | The NG-RAN should inform SMF about the redundant PDU session setup failure in the PDU Session Resource Setup Response Transfer when keep the PDU sessions | Include Redundant PDU Failure to Setup in the successfully setup PDU session response |
| E/// | include the Used Redundancy Sequence Number in the positive PDU Session resource response | include the real Used Redundancy Sequence Number in response message |
| LG | The updated RSN information should be sent to the SMF | include the updated Redundancy Sequence Number in response message indicate not use the requested RSN |

From the above table, all the solution based on successful setup PDU session use the different UP path which the initial node requested. The E/// and LG solution is same, the accepted node inform the initial node the result real used RSN in the successfully setup PDU session.  CATT proposed that the accepted node just informs the initial node it is failure to setup for the redundant. So we have two solutions:

Sol1: The node informs the initial node the used RSN

Sol2: The node informs the initial node failure to setup redundant

We would like to ask the questions as below.

Question 2-9:  Should the accepted node inform the initial node the failure setup for redundant? If yes, which sol is selected?

| Company | Yes/No | Sol1/Sol2 | Comments |
| --- | --- | --- | --- |
| Samsung | Yes | Sol2 | The RSN is decided in SMF, not in RAN. Based on 23.501, RAN just notify SMF the failure of redundant configuration and it is the SMF decision whether to keep the PDU session or release it. |
| Nokia | Yes | Sol1 | The NG-RAN decides based on local configuration. Then it can inform SMF of the result. |
| Huawei | No |  | It was agreed that the NG-RAN can determine based on its local configuration to keep or reject if the RAN cannot satisfy the disjoint user plane requirements. We don’t see the need for SMF to be aware of RAN’s local configuration. And the cause value of the failure case is enough. |
| ZTE | No |  | Agree with HW, in SA2 LS, it is agreed that NG-RAN can decide failure or successful, we suggest to send a new Cause value only. |
| Ericsson | Yes | Sol1 | It is beneficial to feedback this information. |
| LG | Yes | Sol1 | In order to send the updated RSN to the target NG-RAN during NG Handover procedure, the SMF also needs to store this information. |
| CATT | Yes | Sol2 | Sol2 is enough to meet the description In 23.501, The SMF doesn’t need to know which RSN used In 23.501, the statement is : The RAN shall determine whether to notify the SMF in case the RAN resources indicated by the RSN parameter can no longer be maintained and SMF can use that to determine if the PDU Session should be released . The RAN just inform the SMF failure to setup in the requested RSN |

From the companies’ feedback, all the company agree to inform the initial node the failure setup for redundant. For the answer is NO, also mentioned cause value may be used to inform. The solution is different. We can further discuss this issue in the next meeting.

Observation 5: No consensus on whether and how to inform the redundant setup result in the response message

2.1.5 Cause value

In the contributions, some companies discuss the cause value the kinds failure case in each interface. But each company has different understanding and view. List all the proposed value as below.

| Company | Cause value | Explanation |
| --- | --- | --- |
| ZTE | RSN requirement not fulfilled | Release of the Redundant PDU Session is initiated because Redundant Sequence Number (RSN) requirement cannot be fulfilled. |
| CATT | DC for redundant transmission not support | Failure to establish dual connectivity for the redundant PDU session setup |
| LG | Resources not available for the RSN | The RAN resources indicated by RSN are not available |
| E/// | RSN not available for the UP | The redundant user plane resources indicated by RSN is not available. |
| HW | Disjoint user plane resources for DC operation not available | The requirement of disjoint user plane resources for DC operation indicated by the Redundancy Indication cannot be met. |

We would like to ask the questions for the cause value as below.

Question 2-10:  Should the cause value be introduced for the redundant setup failure case? If yes, which one do you prefer?

| Company | Yes/No | Which one prefer | Comments |
| --- | --- | --- | --- |
| Nokia | Yes | LG or E// seems ok. |  |
| Huawei | Yes | Huawei (slightly preferred) | “Disjoint user plane resources not available” seems ok since it is described in stage 2 “…to ensure the user plane paths disjoint according to redundancy information received from 5GC” |
| ZTE | Yes | all fine |  |
| Ericsson | Yes | Ericsson proposal |  |
| LG | Yes | LG or E// |  |
| CATT | Yes | All fine |  |

Observation 6: All company agree that the cause value is introduced for the redundant setup failure case

Proposal 4： Introduce the cause value in impacted interface: RSN not available for the UP

2.1.6 Other topic

2.1.6.1 Exchange UP path information between the nodes

ZTE co-sign with CATT contribution discussed the disjoint UP path quantity issues and give the proposal:

Proposal 1: It is proposed to transfer the disjoint UP path quantity at SN from SN to MN, then the MN can establish redundant PDU Sessions with one or two disjoint UP paths at SN side.

Proposal 2: It is proposed to transfer the disjoint UP path quantity at SN through the Xn Setup procedure and NG-RAN node Configuration Update procedure.

We would like to ask the questions for the cause value as below.

Question 2-11:  Should the SN node transfer the disjoint UP path information to MN have the benefit for the redundant setup? If yes, the above solution is acceptable?

| Company | Yes/No | solution is acceptable( Yes/No) | Comments |
| --- | --- | --- | --- |
| Nokia | No | No | We are not convinced at this stage but we can further discuss. |
| Huawei | No | No |  |
| ZTE | Yes | Yes | In our view, without this information, the MN will have to blindly configure redundant PDU session with either RSN1, or RSN2 or both. |
| Ericsson | No |  | Do not see the clear benefit. |
| LG | No | No |  |
| CATT | Yes |  | If one node support more than 2 disjoint UP path, the information exchange is benefit for the hosting node. |

Observation 6: No consensus on whether SN node transfers the disjoint UP path information to MN

2.1.6.2 The indications of the Secondary RAN to SMF

SS contribution discussed the issue why and how the Secondary RAN node information is sent to SMF and gives the proposal:

Add the identity of the Secondary RAN node into N2 SM information in the above NG procedures in case the Secondary RAN is modified (or added/released).

We would like to ask the questions as below.

Question 2-12:  Should the identity of the Secondary RAN node be informed to SMF?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Agreed by SA2 and captured in 23.501. |
| Nokia | Yes | This seems aligned SA2. |
| Huawei | No (so far) | This SA2 specification only mentions the “Ethernet PDU Sessions”. So far it is not clear to us why not apply to other PDU session types. More check is needed. |
| ZTE | No | It is not very clear now. |
| Ericsson | No | We need to understand why SMF needs to know about the Secondary RAN node. Think this can be kept in RAN. |
| LG | No | Agree with Huawei. According to SA2 specification, in the case of Ethernet PDU Sessions, the SMF has the possibility to change the UPF (acting as the PSA) and select a new UPF based on the identity of the Secondary NG-RAN for the second PDU Session. It is not clear whether to apply to other PDU session types. |
| CATT | No | Agree with HW |

Observation 7: No consensus on whether the identity of the Secondary RAN node is informed to SMF

Proposal 5： List the open issue for further discussion in the future

Whether the E1 CR is needed

Whether the F1 CR is needed

Whether and how to inform the redundant setup result in the response message

Whether SN node transfers the disjoint UP path information to MN is needed

Whether the identity of the Secondary RAN node is informed to SMF

Proposal 6： Revise the TPs base on the agreements

3	Conclusions

In this contribution, we discussed the NRIIOT Hi-layer multi-connectivity solution #1.  Based on the discussion, we make the summary and give proposal:

Agreements:

only use the RSN information for redundant PDU session handling in R16

Carry the RSN information in the NG procedure: PDU Session Resource Setup, Handover Resource Allocation and the IE: PDU Session Resource Setup Request Transfer

Carry the RSN in in the Xn procedure: S-NG-RAN node Addition Preparation, M-NG-RAN node initiated S-NG-RAN node Modification Preparation, Handover Preparation and the IE: PDU Session Resource Setup Info – SN terminated, PDU Session Resources To Be Setup List

Introduce the cause value in impacted interface: RSN not available for the UP

Remaining issues for further discussion:

List the open issue for further discussion in the future

Whether the E1 CR is needed

Whether the F1 CR is needed

Whether and how to inform the redundant setup result in the response message

Whether SN node transfers the disjoint UP path information to MN is needed

Whether the identity of the Secondary RAN node is informed to SMF

Propose to agree the following TPs

Revise the TPs for NRIIOT Hi-layer Sol#1 base on the agreements

Revise R3-200539 in R3-201264 for 38.413 NG, CATT

Revise R3-200249 in R3-201265 for 38.423 Xn, ZTE

4	Reference

- R3-200204	(TP for BL CR NR IIoT for 38.423) Redundant PDU sessions over Xn for solution 1	Nokia, Nokia Shanghai Bell

- R3-200248	(TP for Introduction of NR_IIOT support to TS38.413): Redundant PDU session establishment	ZTE

- R3-200249	(TP for Introduction of NR_IIOT support to TS38.423): Redundant PDU session establishment	ZTE

- R3-200250	(TP for Introduction of NR_IIOT support to TS38.463): Redundant PDU session establishment	ZTE

- R3-200538	Discussion on Higher Layer Multi-Connectivity Solution#1	CATT

- R3-200539	(TP for [NR_IIOT] BL CR for TS 38.413)NRIIOT Higher Layer Multi-Connectivity Solution#1	CATT

- R3-200540	(TP for [NR_IIOT] BL CR for TS 38.423)NRIIOT Higher Layer Multi-Connectivity Solution#1	CATT

- R3-200541	(TP for [NR_IIOT] BL CR for TS 38.463)NRIIOT Higher Layer Multi-Connectivity Solution#1	CATT

- R3-200581	(TP for NR_IIOT BL CR for TS 38.413): Redundant PDU session setup	Samsung

- R3-200582	(TP for NR_IIOT BL CR for TS 38.423): Redundant PDU session setup	Samsung

- R3-200786	Discussion on remaining issues for Solution #1	LG Electronics

- R3-200787	Introduction of Redundant Transmission Information for Solution #1 (TS 38.413)	LG Electronics

- R3-200788	Introduction of Redundant Transmission Information for Solution #1 (TS 38.423)	LG Electronics

- R3-200789	Introduction of Redundant Transmission Information for Solution #1 (TS 38.463)	LG Electronics

- R3-200981	Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity	Ericsson

- R3-200982	Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity	Ericsson

- R3-200983	Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity	Ericsson

- R3-200984	Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity	Ericsson

- R3-201054	(TP for NR_IIOT BL CR for TS 38.413): Higher layer duplication for solution 1	Huawei

- R3-201055	(TP for NR_IIOT BL CR for TS 38.423): Higher layer duplication for solution 1	Huawei

- R3-201056	(TP for NR_IIOT BL CR for TS 38.463): Higher layer duplication for solution 1	Huawei

- R3-200251	(TP for Introduction of NR_IIOT support to TS38.423): Disjoint UP path quantity at SN transfer	ZTE, CATT

- R3-200252	(TP for Introduction of NR_IIOT support to TS38.463): Disjoint UP path quantity at SN transfer	ZTE, CATT

- R3-200583	(TP for NR_IIOT BL CR for TS 38.413): The indications of the Secondary RAN to SMF	Samsung