3GPP TSG RAN WG3#107-e	R3-201210

24 February-6 March 2020

Online

Title: 	Alternative to Port Assignment Request for W1 Interface

Source: 	Orange

Agenda item:	7.2

Document for:	Approval

# Introduction

IANA is about to reject the request for a new Port Number for the new W1 Application used over SCTP. 3GPP needs to define an alternative mechanism.

# Discussion

An alternative based on DNS could allow ng-eNB-DUs to dynamically discover the port number used by ng-eNB-CUs to set up an SCTP association.

A DNS-SD based procedure is specified to allow ng-eNB-DUs to dynamically discover the port number used by ng-eNB-CUs to set up an SCTP association.

With this new mechanism, the ng-eNB-DUs perform can either:

-	contact a DNS server in the NG-RAN to discover and select the ng-eNB-CUs, IP address(es) and port sued for W1AP, or

-	contact directly the target ng-eNB-CUs, using unicast or multicast DNS queries, in the absence of DNS server in the NG-RAN

The DNS-based mechanism will be described in the TS 29.303, specification describing the DNS related procedures used in 3GPP

If this DNS-based mechanism is not agreed (with or without DNS server), there will be no available solutions defined to dynamically discover the port number used by ng-eNB-CUs to set up an SCTP association, forcing operators to rely on static configuration in the ng-eNB-DUs of IP address(es) and port number used by the remote ng-eNB-CUs.

# Conclusion

RAN 3 is kindly asked to review the proposed DNS-based mechanism described in the attached CR against the TS 29.303, endorse it and, if endorsed, send an official LS to CT4 to ask CT4 to update the TS 29.303 to include in the specification the proposed DNS-based mechanism to discover the port used by ng-eNB-CU to receive W1AP signaling.