3GPP TSG-RAN3 Meeting #107-e	R3-201304

24 February - 6 March 2020 E-Meeting

Agenda item:	15.3.1.4

Source:	ZTE

Title:	TP for LTE_Mob_enh BL CR for TS 36.423 Correction of Conditional HO Cancel

Document for:	Agreement

# 1	Introduction

In the agreed R3-197615 at RAN3#106, some confusions regarding (Conditional) HO Cancel procedure were still left.

For the “ Conditional Handover Cancel”, the following abnormal condition has been captured  in section 8.2.Y.4:

“If the candidate target cells contained in the Candidate Cells To Be Cancelled List IE included in CONDITIONAL HANDOVER CANCEL are not associated to the same UE-associated signaling connection used by the previous handover preparation procedure, the source eNB shall ignore the message.”

However, the counterpart description of abnormal condition is still missing for “Handover Cancel” case in section 8.2.4.4 (so far all abnormal condition is missing!), so should be specified to be aligned to TS38.423.

Note: it makes big difference between “ignore the message” and “ignore the xxx IE”.

The new IE “Candidate Cells To Be Cancelled List” is optional, so may not be included in the “CONDITIONAL HANDOVER CANCEL” message. For  “HANDOVER CANCEL”, in case of above IE absence, it can still be used to cancel immediate HO, hence there is no issue there.

For  “CONDITIONAL HANDOVER CANCEL”, since it can only be used for conditional HO, hence in case of above IE absence, it means that all candidate cells associated with the UE signaling connection are supposed to be canceled.  However, per following baseline statement, it seems that regardless of whether invalid Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message or not, the source eNB shall both consider that the target eNB is about to remove all candidate cells. This is conflicting with the abnormal condition  in section 8.2.Y.4 where it specifies the source eNB shall ignore the message.

“At the reception of the CONDITIONAL HANDOVER CANCEL message, the source eNB shall consider that the target eNB is about to remove any reference to, and release any resources previously reserved to the concerned UE context.”

# 2	Text proposal

This TP is based on the BL CR endorsed after RAN3 #106 [R3-197805].

*** First change, skipped text not changed ***

### 8.2.4	Handover Cancel

#### 8.2.4.1	General

The Handover Cancel procedure is used to enable a source eNB to cancel an ongoing handover preparation or an already prepared handover.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Handover Cancel, successful operation

The source eNB initiates the procedure by sending the HANDOVER CANCEL message to the target eNB. The source eNB shall indicate the reason for cancelling the handover by means of an appropriate cause value.

At the reception of the HANDOVER CANCEL message, the target eNB shall remove any reference to, and release any resources previously reserved to the concerned UE context.

The New eNB UE X2AP ID IE and, if available, the New eNB UE X2AP ID Extension IE shall be included if it has been obtained from the target eNB.

If the Candidate Cells To Be Cancelled List IE is included in the HANDOVER CANCEL message, the target eNB shall consider that the source eNB is cancelling only the handover associated to the candidate cells identified by the included ECGI and

associated to the UE-associated signaling connection identified by the Old eNB UE X2AP ID IE (or the Old eNB UE X2AP ID Extension IE if included) and, if included, also by the New eNB UE X2AP ID IE (or the New eNB UE X2AP ID Extension IE if included).

#### 8.2.4.3	Unsuccessful Operation

Not applicable.

#### 8.2.4.4	Abnormal Conditions

If the HANDOVER CANCEL message refers to a context that does not exist, the target eNB shall ignore the message.

If the Candidate Cells To Be Cancelled List IE is included in the HANDOVER CANCEL message and the handover is not associated to a conditional handover, the target eNB shall ignore the Candidate Cells To Be Cancelled List IE.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the HANDOVER CANCEL message were not prepared using the same UE-associated signaling connection, the target eNB shall ignore those non-associated candidate cells.

*** Next change, skipped text not changed ***

### 8.2.Y	Conditional Handover Cancel

#### 8.2.Y.1	General

The Conditional Handover Cancel procedure is used to enable a target eNB to cancel an already prepared conditional handover.

The procedure uses UE-associated signalling.

#### 8.2.Y.2	Successful Operation

Figure 8.2.X.2-1: Conditional Handover Cancel, successful operation

The target eNB initiates the procedure by sending the CONDITIONAL HANDOVER CANCEL message to the source eNB. The target eNB shall indicate the reason for cancelling the conditional handover by means of an appropriate cause value.

The New eNB UE X2AP ID IE and, if available, the New eNB UE X2AP ID Extension IE shall be included if they are available.

At the reception of the CONDITIONAL HANDOVER CANCEL message, the source eNB shall consider that the target eNB is about to remove any reference to, and release any resources previously reserved for candidate cells associated to the UE-associated signalling identified by the Old eNB UE X2AP ID IE (or the Old eNB UE X2AP ID Extension IE if included) and the New eNB UE X2AP ID IE (or the New eNB UE X2AP ID Extension IE if included).

The New eNB UE X2AP ID IE and, if available, the New eNB UE X2AP ID Extension IE shall be included if they are available.

If the Candidate Cells To Be Cancelled List IE is also included, the source eNB shall consider that only the resources reserved for the cells identified by the included ECGI are about to be released.

#### 8.2.Y.3	Unsuccessful Operation

Not applicable.

#### 8.2.Y.4	Abnormal Conditions

Should the CONDITIONAL HANDOVER CANCEL message refer to a context that does not exist, the source eNB shall ignore the message.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the CONDITIONAL HANDOVER CANCEL message were not prepared using the same UE-associated signaling connection, the source eNB shall ignore those non-associated candidate cells.

*** Remaining text not changed ***