3GPP TSG-RAN3 Meeting #107-e	R3-201306

24 February - 6 March 2020 E-Meeting

Agenda item:	15.4.1.2

Source:	ZTE

Title:	TP for LTE_Mob_enh BL CR for TS 36.423 Correction of CHO Per Target Cell TEID

Document for:	Agreement

1	Introduction

In the agreed R3-196699 at RAN3#106, some confusions regarding HO success procedure were still left.

2	Text proposal

This TP is based on the BL CR endorsed after RAN3 #106 [R3-197805].

*** First change, skipped text not changed ***

### 8.2.X	Handover Success

#### 8.2.X.1	General

The Handover Success procedure is used during a conditional handover or a DAPS handover, to enable a target eNB to inform the source eNB that the UE has successfully accessed the target eNB.

The procedure uses UE-associated signalling.

#### 8.2.X.2	Successful Operation

Figure 8.2.X.2-1: Handover Success, successful operation

The target eNB initiates the procedure by sending the HANDOVER SUCCESS message to the source eNB.

If late data forwarding was configured for this UE, the source eNB shall start data forwarding using the tunnel information related to the global target cell ID provided in the HANDOVER SUCCESS message.

When the source eNB receives the HANDOVER SUCCESS message, it shall consider all other CHO preparations accepted for this UE in the target eNB as cancelled and may initiate Handover Cancel procedure towards other candidate target eNBs for this UE, if any.

#### 8.2.X.3	Unsuccessful Operation

Not applicable.

#### 8.2.X.4	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source eNB shall ignore the message.

*** Remaining text not changed ***