3GPP TSG-RAN3 Meeting #107-e	R3-201307

24 February - 6 March 2020 E-Meeting

Agenda item:	15.4.1.2

Source:	ZTE

Title:	TP for NR_Mob_enh BL CR for TS 38.423 Correction of CHO Per Target Cell TEID

Document for:	Agreement

1	Introduction

In the agreed R3-196698 at RAN3#106, some confusions regarding HO success procedure were still left.

2	Text proposal

This TP is based on the BL CR endorsed after RAN3 #106 [R3-197814].

*** First change, skipped text not changed ***

8.2.A	Handover Success

8.2.A.1	General

The Handover Success procedure is used during a conditional handover or a DAPS handover, to enable a target NG-RAN node to inform the source NG-RAN node that the UE has successfully accessed the target NG-RAN node.

The procedure uses UE-associated signalling.

8.2.A.2	Successful Operation

Figure 8.2.A.2-1: Handover Success, successful operation

The target NG-RAN node initiates the procedure by sending the HANDOVER SUCCESS message to the source NG-RAN node.

If late data forwarding was configured for this UE, the source NG-RAN node shall start data forwarding using the tunnel information related to the global target cell ID provided in the HANDOVER SUCCESS message.

When the source NG-RAN node receives the HANDOVER SUCCESS message, it shall consider all other CHO preparations accepted for this UE in the target NG-RAN node as cancelled and may initiate Handover Cancel procedure towards other candidate target NG-RAN nodes for this UE, if any.

Interactions with other procedures

If a CONDITIONAL HANDOVER CANCEL message was received for this UE prior the reception of the HANDOVER SUCCESS message, the source NG-RAN node shall consider that the UE successfully executed the handover.

Editor's note: The impact on the NG-RAN node's behaviour should be verified if the source NG-RAN node receives the HANDOVER SUCCESS message for a UE from the target NG-RAN node after receiving the CONDITIONAL HANDOVER CANCEL message for the same UE from the same target NG-RAN node.

8.2.A.3	Unsuccessful Operation

Not applicable.

8.2.A.4	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

*** Remaining text not changed ***