3GPP TSG-RAN WG3 #107-e				                        	     R3-201332

24 February – 6 March 2020, E-Meeting

Source:	CATT

Title:	(TP for NR_IAB BL CR for TS 38.401): Clean-up for Intra-CU topology adaptation procedure

Agenda Item:	13.3.2.2

Document for:	Other

Introduction

Base on the summary of the email discussion R3-201148 on intra donor IAB migration, this contribution provides the TP for TS 38.401 to capture the agreements accordingly.

Proposal 1: For stop indication, existing Transmission Action Indicator IE in the UE CONTEXT MODIFICATION REQUEST message could be reused, no additional info is needed, FFS could be removed.

Proposal 2: Configuration of TNL address(es) for the migrating IAB node will be addressed in CB#44.

Proposal 3: Remove the FFS for Configuration of TNL address(es), and add corresponding sentence to reflect our agreement.

Proposal 4: When to trigger the steps 11, 12 and 15 for the migrating IAB-node’s descendant nodes could be left to implementation.

Proposal 5: Add a Note to capture the above description on handling of descendant nodes during IAB node migration.

Proposal 6: The IAB-donor-CU can determine the unsuccessfully transmitted / delivery of downlink data over the backhaul link based on legacy F1-U DDDS from the IAB node, FFS could be removed.

Proposal 7: Whether need to and how to determine the delivery of all upstream packets is left to R17, we just remove the Editor’s note, nothing else needs to be specified in the stage 2 now.

TP for IAB BL CR for TS 38.401

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of the TP >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.2.x	Intra-CU topology adaptation procedure

During the intra-CU topology adaptation, both the source and the target parent node are served by the same IAB-donor-CU. The target parent node may use a different IAB-donor-DU than the source parent node. The source path may further have common nodes with the target path. Figure 8.2.x-1 shows an example of the topology adaptation procedure, where the target parent node uses a different IAB-donor-DU than the source parent node.

Figure 8.2.x-1: IAB intra-CU topology adaptation procedure

1.	The migrating IAB-node MT sends a Measurement Report message to the source parent node DU. This report is based on a Measurement Configuration the migrating IAB-node MT received from the IAB-donor-CU before.

2.	The source parent node DU sends an UL RRC MESSAGE TRANSFER message to the IAB-donor-CU to convey the received Measurement Report.

3.	The IAB-donor-CU sends a UE CONTEXT SETUP REQUEST message to the target parent node DU to create the UE context for the migrating IAB-node MT and setup one or more bearers. These bearers are used by the migrating IAB-node MT for its own data and signalling traffic.

4.	The target parent node DU responds to the IAB-donor-CU with a UE CONTEXT SETUP RESPONSE message.

5.	The IAB-donor-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source parent node DU, which includes a generated RRCReconfiguration message. The Transmission Action Indicator in the UE CONTEXT MODIFICATION REQUEST message indicates to stop the data transmission to the migrating IAB-node.

6.	The source parent node DU forwards the received RRCReconfiguration message to the migrating IAB-node MT.

7.	The source parent node DU responds to the IAB-donor-CU with the UE CONTEXT MODIFICATION RESPONSE message.

8.	A Random Access procedure is performed at the target parent node DU.

9.	The migrating IAB-node MT responds to the target parent node DU with an RRCReconfigurationComplete message.

10.	The target parent node DU sends an UL RRC MESSAGE TRANSFER message to the IAB-donor-CU to convey the received RRCReconfigurationComplete message. Also, uplink packets can be sent from the migrating IAB-node MT, which are forwarded to the IAB-donor-CU through the target parent node DU. These DL and UL packets belong to the MT’s own signalling and data traffic.

11.	The IAB-donor-CU configures backhaul RLC channels and BAP-layer route entries on the target path between migrating IAB-node and target IAB-donor-DU. This step also includes allocation of TNL address(es) that is (are) routable via the target IAB-donor-DU. These configurations may be performed at an earlier stage, e.g. right after step 3. The new TNL address(es) is (are) included in the RRCReconfiguration message at step 5.

12.	All F1-U tunnels and F1-C are switched to use the migrating IAB-node’s new TNL address(es).

13.	The IAB-donor-CU sends a UE CONTEXT RELEASE COMMAND message to the source parent node DU.

14.	The source parent node DU releases the migrating IAB-node MT’s context and responds the IAB-donor-CU with a UE CONTEXT RELEASE COMPLETE message.

15.	The IAB-donor-CU releases BH RLC channels and BAP routing entries on the source path. The migrating IAB-node may further release the TNL address(es) it used on the source path.

NOTE: In case that the source route and target route have common nodes, the BH RLC CHs and BAP routing entries of those nodes may not need to be released in Step 15.

NOTE: Steps 11, 12 and 15 also have to be performed for the migrating IAB-node’s descendant nodes, as follows:

- The descendant nodes must also switch to new TNL addresses that are anchored in the target IAB-donor-DU. The IAB-donor-CU may send these addresses to the descendant nodes and release the old addresses via corresponding RRC signaling

- If needed, the IAB-donor-CU configures backhaul RLC channels, BAP-layer route entries on the target path for the descendant nodes and the BH RLC Channel mappings on the descendant nodes in the same manner as described for the migrating IAB-node in step 11.

- The descendant nodes switch their F1-U and F1-C tunnels to new TNL addresses that are anchored at the new IAB-donor-DU, in the same manner as described for the migrating IAB-node in step 12.

- Based on implementation, these steps can be performed after or in parallel with the handover of the migrating IAB-node. In Rel-16, in-flight packets in UL direction that were dropped during the migration procedure may not be recoverable.

NOTE: In upstream direction, in-flight packets between the source parent node and the IAB-donor-CU can be delivered even after the target path is established.

NOTE:	On-going downlink data in the source path may be discarded up to implementation.

NOTE: IAB-donor-CU can know the unsuccessfully transmitted downlink data over the backhaul link by implementation.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of the TP >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>