3GPP TSG-RAN WG3 Meeting #107-e                                                          R3-201363

Electronic Meeting, February 24th – March 6th, 2020

Agenda item:       13.3.2.1

Source: 		Qualcomm Incorporated

Title:	 (TP to NR_IAB BL CR to 38401) BH RLF recovery procedure

Document for:	Other

# 1	Introduction

This document includes (TP to NR_IAB BL CR to 38401) BH RLF recovery procedure.

# 2 	TP for IAB BL CR to TS 38.401: RLF Recovery

START OF CHANGE

### 8.2.z	Intra-CU Backhaul RLF recovery for IAB-nodes in SA mode

The intra-CU backhaul RLF recovery procedure SA mode enables migration of an IAB-node to another parent node underneath the same IAB-donor-CU, when the IAB-node’s MT declares a backhaul RLF. The declaration of backhaul RLF is described in TS 38.331 [yy].

NOTE: Declaration if recovery occurs at the same or a different IAB-donor CU is up to implementation.

Figure 8.2.z-1 shows an example of the BH RLF recovery procedure for an IAB-node in SA mode. In this example, the IAB-node changes from its initial parent node to a new parent node, where the new parent node is served by an IAB-donor-DU different than the one serving its initial parent node.

Figure 8.2.z: IAB intra-CU backhaul RLF recovery procedure for an IAB-node in SA mode

- The IAB-node MT declares BH RLF as described in TS 38.331 [yy], clause 9.2.7.

- The MT of an IAB-node undergoing recovery from RLF conducts the RRC re-establishment procedure at the new parent node, as defined in clause 8.7. In this procedure, the IAB-donor-CU may provide new TNL address(es), which are anchored at the new-path IAB-donor-DU, to the IAB-node’s MT via RRC signalling.

3.	The remaining part of the procedure follows steps 11-15 of the intra-CU topology adaptation procedure defined in clause 8.2.x.

Descendant node(s) of the IAB-node recovering from RLF may also need to switch to new TNL address(es) anchored in the target-path IAB-donor-DU following the same mechanism as described for IAB intra-CU topology adaptation procedure in clause 8.2.x.

END OF CHANGE