3GPP TSG-RAN WG3 Meeting #107-e	R3-201374

Online, 24th February - 6th March 2020

Agenda Item:	10.2.2.1

Source: 	CMCC

Title: 	TP to SON BLCR 38.300 on support of MLB

Document for:	Approval

Introduction

This contribution provides a TP for 38.300 as per the agreement in summary offline of discussion from CB # 27_Email027-MLB_Xn_X2_F1_E1.

Text proposal for TR 38.300

- 15.X.1	Support for Mobility Load Balancing

Editor’s note: This section captures the stage 2 descriptions for Mobility Load Balancing

- 15.X.1.1	General

The objective of mobility load balancing is to distribute load evenly among cells and among areas of cells, or to transfer part of the traffic from congested cell or from congested areas of cells, or to offload users from one cell, cell area, carrier or RAT to achieve network energy saving. This can be done by means of optimization of cell reselection/handover parameters and handover actions. The automation of such optimisation can provide high quality user experience, while simultaneously improving the system capacity and also to minimize human intervention in the network management and optimization tasks.

Intra-RAT and intra-system inter-RAT load balancing scenarios are supported.

In general, support for mobility load balancing consists of one or more of following functions:

-	 Load reporting;

- Load balancing action based on handovers;

-	 Adapting handover and/or reselection configuration.

- 15.X.1.2 	Load reporting

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces.

The following load related information should be supported which consists of,

-	Radio resource usage (per-cell and per SSB area PRB usage: DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, DL/UL total PRB usage and DL/UL scheduling PDCCH CCE usage);

-	TNL capacity indicator (UL/DL TNL offered capacity and available capacity);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-Capacity value (per cell, per SSB area and per slice: UL/DL available capacity);

-	HW capacity indicator (offered throughput and available throughput); [FFS for F1/Xn]

- RRC connections (number of RRC connections, and available RRC Connection Capacity).

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.