3GPP TSG-RAN3 Meeting #107-e	R3-200101

Electronic meeting, 24th February - 6th March 2020

3GPP TSG-RAN WG2 Meeting #108	R2-1916622

Reno, USA, 18th – 22nd November 2019

Title:	LS on handover without SN configuration query

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN3

CC:

Contact Person:

#### Name:	David Lecompte

Email Address:	david.lecompte@huawei.com

Attachments:

1. Overall Description:

RAN2 agreed to support the scenario in which, in order to reduce intra-RAT handover delay, the source MN does not perform SN configuration query before inter-MN handover in EN-DC, NGEN-DC, NR-DC and NE-DC. According to this, the following agreement has been reached during the online discussion:

RAN2 confirms that for handover, there is a need to allow the source MN to indicate to the target MN that up-to-date information is not available for a list of DRBs, which are in use or potentially in use by the SN.

The target MN needs to be informed by the source MN that SN terminated bearers should be released and, in doing so, a list of used or potentially used DRB IDs to be released is provided by the source MN to the target MN.

2. Actions:

To RAN3:

ACTION: 	RAN2 respectfully ask RAN3 to define the necessary signalling for the case of inter-MN handover without SN configuration query in EN-DC, NGEN-DC, NR-DC and NE-DC, where the source MN informs the target MN that SN terminated bearers should be released and provides to the target MN the list of used or potentially used DRB IDs to be released.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#109		24-28 February 2020				Athens, Greece

RAN2#109bis		20-24 April 2020					Japan